3GPP TSG-RAN WG4 Meeting #89	R4-1815247

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	7.11.8.1.2

Source:	Ericsson

Title:	Measurement BW for CSI-RS based candidate beam detection

Document for:	Discussion

# 1	Introduction

RAN4#88bis agreed with the way forward on beam management [1], and one of the open issues is D=1 case for BFD, CBD, and beam reporting, where D corresponds to frequency domain density (ρ) and D=1 means every PRB transmission and D=3 means every 3rd PRB transmission according to TS38.211 7.4.1.5.3 [2]. So far RAN4 discussed the measurement accuracy and measurement period based on the assumption D=3. However RAN4#88bis discussed another density option D=1 and it is FFS as follows:

Way forward on BFD - FFS if requirements for D=1 with larger BW are defined for CSI-RS based BFD Way forward on CBD - FFS if requirements for D=1 with larger BW are defined for CSI-RS based CBD

In this contribution, we show our simulation results of D=1 with larger BW to discuss the requirement with D=1.

# 2	Discussion

## 2.1	CSI-RS based measurements

Table 1 shows the simulation parameters for CSI-RS based L1-RSRP measurement. In our understanding, it is higher frequency band (i.e. FR2) wider bandwidth can be configured for CSI-RS for measurement. Therefore we perform simulation only with SCS=120kHz.

Table 1	Simulation parameters for CSI-RS based L1-RSRP measurement accuracy

| Parameters | Value |
| --- | --- |
| SCS | 120kHz |
| Side condition (SNR) | -3dB |
| Density (ρ) | 1, 3 |
| Number of samples | 5 (CBD), 10 (BFD) |
| CSI-RS measurement BW (Number of PRBs) | 24, 48 |

Figure 1 shows the simulation results with different fading channel condition. From the simulation results, it is observed the relative RSRP measurement accuracy with D=1 and 48PRB is improved compared with D=3 and 24PRB especially for TDLA30-83 and TDLC30-83. However the improved accuracy is not significant (~0.3dB) considering the resolution of reported value (1dB).

Figure 1	CSI-RS based L1-RSRP measurement accuracy with FR2 SCS=120kHz.

# 3	Summary

Observation: CSI-RS based L1-RSRP measurement accuracy is improved with D=1 and 48PRB compared with D=3 and 48PRB, although the improvement is not so significant.

# References

- R4-1813736, “Way forward on NR beam management”, Huawei, HiSilicon, Nokia.

- TS38.211.