3GPP TSG-RAN WG4 Meeting #89	R4-1815248

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	7.12.3.4

Source:	Ericsson

Title:	TDD configuration for RRM performance requirements

Document for:	Discussion

# 1	Introduction

RAN4#88bis agreed that both LTE and NR carrier should be synchronized for FR1 EN-DC RRM test as follows:

Agreement: The TDD uplink-downlink configuration for LTE carrier should be aligned with NR uplink-downlink configuration in FR1 for EN-DC.

This contribution proposes the NR TDD configuration used for EN-DC RRM tests.

# 2	Discussion

As we discussed in [1], the FR1 EN-DC RRM test uses LTE TDD UL-DL configuration 1 (‘DSUUD DSUUD’) and special subframe configuration to 4 (9DL:3GP:2UL).

Figure 1 and Figure 2 illustrate the NR TDD UL-DL configuration and special slot configuration aligned with LTE TDD configuration.

Figure 1	NR TDD configuration used for FR1 EN-DC RRM test.

Figure 2	NR TDD special slot configuration used for FR1 EN-DC RRM test.

The TDD configurations in Figure 1 and Figure 2 can be set by configuring two TDD UL-DL patterns (pattern1 and pattern2).

TDD-UL-DL-ConfigCommon ::= SEQUENCE { referenceSubcarrierSpacing SubcarrierSpacing, pattern1 TDD-UL-DL-Pattern, pattern2 TDD-UL-DL-Pattern OPTIONAL, -- Need R ... } TDD-UL-DL-Pattern ::= SEQUENCE { dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}, nrofDownlinkSlots INTEGER (0..maxNrofSlots), nrofDownlinkSymbols INTEGER (0..maxNrofSymbols-1), nrofUplinkSlots INTEGER (0..maxNrofSlots), nrofUplinkSymbols INTEGER (0..maxNrofSymbols-1), ..., [[ dl-UL-TransmissionPeriodicity-v1530 ENUMERATED {ms3, ms4} OPTIONAL -- Need R ]] }

For FR1 SCS=15kHz, the pattern 1 is set to ‘DSUU’ with periodicity of 4ms and the pattern 2 is set to ‘D’ with periodicity of 1ms. For special slot, we can set 9 downlink symbols and 2 uplink symbols.

For FR1 SCS=30kHz, the pattern 1 is set to ‘DDDSUUUU’ with periodicity of 4ms and the pattern 2 is set to ‘DD’ with periodicity of 1ms. For special slot, we can set 4 downlink symbols and 4 uplink symbols.

Note for EN-DC tests in FR2, LTE signal comes from the conduced cable and NR signal comes from OTA. This means LTE and NR can configure different TDD configuration and therefore we propose to keep the current TDD configuration for NR RMC in FR2, and is pattern1=DDDSU with periodicity of 0.625ms but no pattern2.

# 3	Conclusion

Proposal: RAN4 specify the following NR TDD slot pattern for RRM tests.

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 | TDDConf.2.1 | TDDConf.3.1 |
| referenceSubcarrierSpacing | kHz | 15 | 30 | 120 |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=’9DL:3GP:2UL’ | ‘3D1S4U’ S=’4DL:6GP:4UL’ | ‘DDDSU’ S=’9DL:3GP:2UL’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 4 | 0.625 |
| nrofDownlinkSlots |  | 1 | 3 | 3 |
| nrofDownlinkSymbols |  | 9 | 4 | 9 |
| nrofUplinkSlot |  | 2 | 4 | 1 |
| nrofUplinkSymbols |  | 2 | 4 | 2 |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ | ‘DD’ | Not configured |
| dl-UL-TransmissionPeriodicity | ms | 1 | 1 | Not configured |
| nrofDownlinkSlots |  | 1 | 2 | Not configured |
| nrofDownlinkSymbols |  | 0 | 0 | Not configured |
| nrofUplinkSlot |  | 0 | 0 | Not configured |
| nrofUplinkSymbols |  | 0 | 0 | Not configured |

# References

- R4-1812846, “TDD configuration for RRM performance requirements”, Ericsson.