3GPP TSG-RAN WG4 Meeting #89	R4-1815249

# Spokane, WA, USA,  12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | <Rev#> | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of TDD configuration for EN-DC RRM tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR TDD configuration used for EN-DC RRM tests is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add NR TDD UL/DL configuration tables - Clarified the PDSCH scheduling location in time domain and frequency domain. - Clarified the PRB location for RMC and CORESET for RMC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN-DC TDD test may not be performed correcty due to the different UL/DL configuration between LTE cell and NR cell. It is not clear where PDCCH/PDSCH is scheduled in time/frequecny domain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.1, A.3.1.3, A.3.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

# A.3	RRM test configurations

### A.3.1	Reference measurement channels

### A.3.1.1	PDSCH

#### A.3.1.1.1	FDD

Table A.3.1.1.1-1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 FDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | [10] |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | [1864] |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | Bits | [6048] |  |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. |  |  |  |  |  |  |  |  |

#### A.3.1.1.2	TDD

Table A.3.1.1.2-1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | [4] |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | [1864] |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | Bits | [6048] |  |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. |  |  |  |  |  |  |  |  |

Table A.3.1.1.1-2: PDSCH Reference Measurement Channels for SCS=30kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | [10] |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | [1864] |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | Bits | [6048] |  |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. |  |  |  |  |  |  |  |  |

Table A.3.1.1.1-3: PDSCH Reference Measurement Channels for SCS=120kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | [48] |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | [1864] |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | Bits | [6048] |  |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

### A.3.1.3	CORESET for RMC scheduling

#### A.3.1.3.1	FDD

Table A.3.1.3.1-1: Control Channel RMC for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | [CCR.1.1] FDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |  |  |
| Subcarrier spacing | KHz | 15 |  |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of CORESET | symbols | 2 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size |  |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved |  |  |  |  |  |  |
| Interleave n_shift |  | 0 |  |  |  |  |  |  |
| Interleave size |  | 2 |  |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DCI formats |  | Note 1 |  |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 |  |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

#### A.3.1.3.2	TDD

Table A.3.1.3.2-1: Control Channel RMC for TDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | [CCR.1.1] TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |  |  |
| Subcarrier spacing | KHz | 15 |  |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of CORESET | symbols | 2 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size |  |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved |  |  |  |  |  |  |
| Interleave n_shift |  | 0 |  |  |  |  |  |  |
| Interleave size |  | 2 |  |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DCI formats |  | Note 1 |  |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 |  |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-2: Control Channel RMC for TDD with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | [CCR.2.1] TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |  |
| Subcarrier spacing | KHz | 30 |  |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of CORESET | symbols | 2 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size |  |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved |  |  |  |  |  |  |
| Interleave n_shift |  | 0 |  |  |  |  |  |  |
| Interleave size |  | 2 |  |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DCI formats |  | Note 1 |  |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 |  |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-3: Control Channel RMC for TDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | [CCR.3.1] TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |  |  |
| Subcarrier spacing | KHz | 120 |  |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of CORESET | symbols | 2 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size |  |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved |  |  |  |  |  |  |
| Interleave n_shift |  | 0 |  |  |  |  |  |  |
| Interleave size |  | 2 |  |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DCI formats |  | Note 1 |  |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 |  |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

### A.3.1.y	TDD UL/DL configuration

Table A.3.1.y-1 TDD UL/DL configuration for SCS=15kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 |  |  |
| referenceSubcarrierSpacing | kHz | 15 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=’9DL:3GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 9 |  |  |
| nrofUplinkSlot |  | 2 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| Note 1: As specified in TS38.213 and TS 38.331. Note 2: For information |  |  |  |  |

Table A.3.y-2 TDD UL/DL configuration for SCS=30kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 |  |  |
| referenceSubcarrierSpacing | kHz | 30 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=’4DL:6GP:4UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 4 |  |  |
| nrofUplinkSlot |  | 4 |  |  |
| nrofUplinkSymbols |  | 4 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 2 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| Note 1: As specified in TS38.213 and TS 38.331. Note 2: For information |  |  |  |  |

Table A.3.y-3 TDD UL/DL configuration for SCS=120kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 |  |  |
| referenceSubcarrierSpacing | kHz | 120 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=’9DL:3GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 9 |  |  |
| nrofUplinkSlot |  | 1 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |  |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured |  |  |
| nrofDownlinkSlots |  | Not configured |  |  |
| nrofDownlinkSymbols |  | Not configured |  |  |
| nrofUplinkSlot |  | Not configured |  |  |
| nrofUplinkSymbols |  | Not configured |  |  |
| Note 1: As specified in TS38.213 and TS 38.331. Note 2: For information |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------