3GPP TSG-RAN WG4 Meeting #89	R4-1815267

Spokane, USA, 12th – 16th Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Receiver requirements for interband EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Receiver requirements apart from REFSENS are missing for interband EN-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Receiver requirements apart from REFSENS are added for interband EN-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Receiver requirements apart from REFSENS are missing for interband EN-DC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4B, 7.5B, 7.6B, 7.7B, 7.8B, 7.9B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********************************* Start of changes *************************************

## 7.4	Maximum input level

## 7.4B	Maximum input level for EN-DC in FR1

### 7.4B.1	Intra-band contiguous EN-DC in FR1

TBD.

### 7.4B.2	Intra-band non-contiguous EN-DC in FR1

TBD.

### 7.4B.3	Inter-band EN-DC within FR1

Maximum input level requirements specified in sub-clauses 7.4 of [2] and [4] apply for each component carrier.

### 7.4B.4	Inter-band EN-DC including FR2

Maximum input level requirements specified in sub-clauses 7.4 of [3] and [4] apply for each component carrier.

### 7.4B.5	Inter-band EN-DC including both FR1 and FR2

Maximum input level requirements specified in sub-clauses 7.4 of [2], [3] and [4] apply for each component carrier.

## 7.5	Adjacent channel selectivity

## 7.5B	Adjacent channel selectivity for EN-DC in FR1

### 7.5B.1	Intra-band contiguous EN-DC in FR1

TBD.

### 7.5B.2	Intra-band non-contiguous EN-DC in FR1

TBD.

### 7.5B.3	Inter-band EN-DC within FR1

Adjacent channel selectivity requirements specified in sub-clauses 7.5 of [2] and [4] apply for each component carrier.

### 7.5B.4	Inter-band EN-DC including FR2

Adjacent channel selectivity requirements specified in sub-clauses 7.5 of [3] and [4] apply for each component carrier.

### 7.5B.5	Inter-band EN-DC including both FR1 and FR2

Adjacent channel selectivity requirements specified in sub-clauses 7.5 of [2], [3] and [4] apply for each component carrier.

## 7.6	Blocking characteristics

## 7.6B	Blocking characteristics for EN-DC in FR1

### 7.6B.1	General

### 7.6B.2	Inband blocking for EN-DC in FR1

#### 7.6B.2.1	Intra-band contiguous EN-DC in FR1

TBD.

#### 7.6B.2.2	Intra-band non-contiguous EN-DC in FR1

TBD.

#### 7.6B.2.3	Inter-band EN-DC within FR1

Inband blocking requirements specified in sub-clauses 7.6.2 of [2] and 7.6.1 of [4] apply for each component carrier..

#### 7.6B.2.4	Inter-band EN-DC including FR2

Inband blocking requirements specified in sub-clauses 7.6.2 of [3] a and 7.6.1 of [4] apply for each component carrier.

#### 7.6B.2.5	Inter-band EN-DC including both FR1 and FR2

Inband blocking requirements specified in sub-clauses 7.6.2 of [2], [3] and 7.6.1 of [4] apply for each component carrier.

### 7.6B.3	Out-of-band blocking for EN-DC in FR1

#### 7.6B.3.1	Intra-band contiguous EN-DC in FR1

TBD.

#### 7.6B.3.2	Intra-band non-contiguous EN-DC in FR1

TBD.

#### 7.6B.3.3	Inter-band EN-DC within FR1

Out-of band blocking requirements specified in sub-clauses 7.6.3 of [2] and 7.6.2 of [4] apply for lowest level EN-DC fallbacks (two bands) in section 5.2B.4.1 with only E-UTRA UL with output power as in TS 36.101 (4dB Below PCmax_l).

#### 7.6B.3.4	Inter-band EN-DC including FR2

Out-of band blocking requirements specified in sub-clauses 7.6.3 of [3] and 7.6.2 of [4] apply for lowest level EN-DC fallbacks (two bands) in section 5.2B.5.1 with only E-UTRA UL with output power as in TS 36.101 (4dB Below PCmax_l).

#### 7.6B.3.5	Inter-band EN-DC including both FR1 and FR2

Out-of band blocking requirements specified in sub-clauses 7.6.3 of [2], [3] and 7.6.2 of [4] apply for lowest level EN-DC fallbacks (three bands) in section 5.2B.6.2 with only E-UTRA UL with output power as in TS 36.101 (4dB Below PCmax_l).

### 7.6B.4	Narrow band blocking for EN-DC in FR1

#### 7.6B.4.1	Intra-band contiguous EN-DC in FR1

TBD.

#### 7.6B.4.2	Intra-band non-contiguous EN-DC in FR1

TBD.

#### 7.6B.4.3	Inter-band EN-DC within FR1

Narrow band blocking requirements specified in sub-clauses 7.6.4 of [2] and 7.6.3 of [4] apply for each component carrier.

#### 7.6B.4.4	Inter-band EN-DC including FR2

Narrow band blocking requirements specified in sub-clauses 7.6.4 of [3] a and 7.6.3 of [4] apply for each component carrier.

#### 7.6B.4.5	Inter-band EN-DC including both FR1 and FR2

Narrow band blocking requirements specified in sub-clauses 7.6.4 of [2], [3] and 7.6.3 of [4] apply for each component carrier.

## 7.7	Spurious response

## 7.7B	Spurious response for EN-DC in FR1

### 7.7B.1	Intra-band contiguous EN-DC in FR1

### TBD.7.7B.2	Intra-band non-contiguous EN-DC in FR1

TBD.

### 7.7B.3	Inter-band EN-DC within FR1

Spurious response requirements specified in sub-clauses 7.7 of [2] and [4] apply for each component carrier.

### 7.7B.4	Inter-band EN-DC including FR2

Spurious response requirements specified in sub-clauses 7.7 of [3] and [4] apply for each component carrier.

### 7.7B.5	Inter-band EN-DC including both FR1 and FR2

Spurious response requirements specified in sub-clauses 7.7 of [2], [3] and [4] apply for each component carrier.

## 7.8	Intermodulation characteristics

## 7.8B	Intermodulation characteristics for EN-DC in FR1

### 7.8B.1	General

### 7.8B.2	Wide band Intermodulation

#### 7.8B.2.1	Intra-band contiguous EN-DC in FR1

TBD.

#### 7.8B.2.2	Intra-band non-contiguous EN-DC in FR1

TBD.

#### 7.8B.2.3	Inter-band EN-DC within FR1

Wide band Intermodulation requirements specified in sub-clauses 7.8.2 of [2] and 7.8.1 of [4] apply for each component carrier.

#### 7.8B.2.4	Inter-band EN-DC including FR2

Wide band Intermodulation requirements specified in sub-clauses 7.8.2 of [3] a and 7.8.1 of [4] apply for each component carrier.

#### 7.8B.2.5	Inter-band EN-DC including both FR1 and FR2

Wide band Intermodulation requirements specified in sub-clauses 7.8.2 of [2], [3] and 7.8.1 of [4] apply for each component carrier.

## 7.9	Spurious emissions

## 7.9B	Spurious emissions for EN-DC in FR1

### 7.9B.1	Intra-band contiguous EN-DC in FR1

TBD.

### 7.9B.2	Intra-band non-contiguous EN-DC in FR1

TBD.

### 7.9B.3	Inter-band EN-DC within FR1

Spurious emissions requirements specified in sub-clauses 7.9 of [2] and [4] apply for each component carrier.

### 7.9B.4	Inter-band EN-DC including FR2

Spurious emissions requirements specified in sub-clauses 7.9 of [3] and [4] apply for each component carrier.

### 7.9B.5	Inter-band EN-DC including both FR1 and FR2

Spurious emissions requirements specified in sub-clauses 7.9 of [2], [3] and [4] apply for each component carrier.

********************************* Start of changes *************************************