3GPP TSG-RAN WG4 Meeting #89                     	R4-1815273

Spokane, US, Nov 12th –16th, 2018

Agenda item:            7.13.1.3

Source:	MediaTek Inc.

Title:	NR PBCH simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation result of PBCH.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of PBCH performance. Table A.1 (in Appendix) is the test case list. Table A.2 contains other detailed parameter setting in the simulation.

Table 2.1 is the 1% BLER SNR values of Test 1 and Test 2. Figure A.2.1~Figure A.2.4 (in Appendix) are the corresponding performance curves (FR1)

Table 2.1 1% BLER SNR

| Case Number | 2RX | 4RX |
| --- | --- | --- |
| 1 | -8.93 | -11.31 |
| 2-1 | -8.96 | -11.16 |
| 2-2 | -8.97 | -11.29 |
| 3 | -8.74 | N/A |
| 4 | -8.00 | N/A |

Observation 1: The operating SNR for 4RX test is very low and it is not suitable to be included in the requirement considering the stability at such low SNR

# 3	Conclusions

In the contribution, we provide our simulation on PBCH. We have the following observations:

Observation 1: The operating SNR for 4RX test is very low and it is not suitable to be included in the requirement considering the stability at such low SNR

# 4	Reference

[1] R4-1814240, “Way forward on NR PBCH demodulation requirements”, CMCC, Huawei, HiSilicon, China Telecom, ZTE, CATT, NTT DOCOMO, Vodafone, Orange, Sprint, KDDI, SoftBank.

Appendix

Table A.1 PBCH test cases

| Parameters | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Frequency range | FR1 | FR1 | FR2 | FR2 |
| Channel BW | 10MHz | 40MHz | 100MHz | 100MHz |
| SS/PBCH SCS | 15kHz | 30kHz | 120kHz | 240kHz |
| SS Block pattern | Case A | Case B (2-1) Case C (2-2) | Case D | Case E |
| Antenna configuration | 1x2 1x4 | 1x2 1x4 | 1x2 | 1x2 |
| Propagation channel | TDL-C RMS DS=300ns Doppler=100Hz | TDL-A RMS DS=30ns, Doppler=10Hz | TDL-A RMS DS=30ns, Doppler=300Hz | TDL-A RMS DS=30ns, Doppler=75Hz |

Table A.2 PBCH simulation detailed parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| SS Block pattern | - Case A - 15 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes of {2, 8} + 14*n. For carrier frequencies smaller than or equal to 3 GHz, n=0, 1. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1, 2, 3. - Case B - 30 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8, 16, 20} + 28*n. For carrier frequencies smaller than or equal to 3 GHz, n=0. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1. - Case C - 30 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {2, 8} + 14*n. For carrier frequencies smaller than or equal to 3 GHz, n=0, 1. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1, 2, 3. - Case D - 120 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8, 16, 20} + 28*n. For carrier frequencies larger than 6 GHz, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18. - Case E - 240 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {8, 12, 16, 20, 32, 36, 40, 44} + 56*n. For carrier frequencies larger than 6 GHz, n=0, 1, 2, 3, 5, 6, 7, 8. | N=0 for all simulation |
| Cell ID | 0 |  |
| Number of SS/PBCH blocks within an SS burst set periodicity | 1 |  |
| SS/PBCH block index | 0 |  |
| SS burst periodicity | 20ms |  |
| MIB size | 24 bits |  |
| PBCH payload size (including CRC=24bits and 8 bits of SS/PBCH location information) | 56 bits |  |

Figure A.2.1 PBCH BLER – Test 1 2RX

SNR

BLER

Figure A.2.2 PBCH BLER – Test 1 4RX

SNR

BLER

Figure A.2.3 PBCH BLER – Test 2 2RX

SNR

BLER

Figure A.2.4 PBCH BLER – Test 2 4RX

SNR

BLER