3GPP TSG-RAN WG4 Meeting #89                     	R4-1815274

Spokane, US, Nov 12th –16th, 2018

Agenda item:            7.13.1.4

Source:	  MediaTek Inc.

Title:	  NR CSI simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation result of CSI.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of CSI performance. Table A.1~Table A.6 (in Appendix) are the simulation assumption for CQI, PMI and RI. Table A.7~Table A.10 contains other detailed parameter setting in the simulation.

2.1 CQI

In this paper, we provide the simulation result of static CQI test.

Table 2.1.1~Table 2.1.4 are the simulation result (CQI report distribution and BLER performance of median CQI, median CQI+1 and median CQI-1) for Static CQI test 1 and test 2 with 2RX and 4RX (FR1). The SNR range is from 0 to 20dB.

Table 2.1.1-1 Static CQI (Distribution) – Test 1 2TX x 2RX Rank2

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 2 | 1 | 100% |
| 2 | 3 | 3 | 100% |
| 4 | 3 | 3 | 100% |
| 6 | 4 | 5 | 100% |
| 8 | 5 | 7 | 100% |
| 10 | 6 | 9 | 100% |
| 12 | 7 | 11 | 100% |
| 14 | 8 | 13 | 100% |
| 16 | 9 | 15 | 100% |
| 18 | 10 | 17 | 100% |
| 20 | 11 | 19 | 100% |

Table 2.1.1-2 Static CQI (BLER) – Test 1 2TX x 2RX Rank2

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 1 | 0 | 0 | 1 |
| 2 | 3 | 3 | 0 | 0 | 1 |
| 4 | 3 | 3 | 0 | 0 | 1 |
| 6 | 4 | 5 | 0 | 0 | 1 |
| 8 | 5 | 7 | 0 | 0 | 0.9998 |
| 10 | 6 | 9 | 0 | 0 | 0.9965 |
| 12 | 7 | 11 | 0 | 0 | 0.9986 |
| 14 | 8 | 13 | 0 | 0 | 0.6329 |
| 16 | 9 | 15 | 0 | 0 | 0.9896 |
| 18 | 10 | 17 | 0 | 0 | 0.7159 |
| 20 | 11 | 19 | 0 | 0 | 0.1341 |

Figure 2.1.2-1 Static CQI (Distribution) – Test 1 2TX x 4RX Rank2 0dB

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 3 | 3 | 100% |
| 2 | 4 | 5 | 100% |
| 4 | 5 | 7 | 100% |
| 6 | 6 | 9 | 100% |
| 8 | 7 | 11 | 100% |
| 10 | 8 | 13 | 100% |
| 12 | 9 | 15 | 100% |
| 14 | 10 | 17 | 100% |
| 16 | 11 | 19 | 100% |
| 18 | 12 | 21 | 100% |
| 20 | 13 | 23 | 100% |

Figure 2.1.2-2 Static CQI (BLER) – Test 1 2TX x 4RX Rank2 0dB

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 3 | 0 | 0 | 1 |
| 2 | 4 | 5 | 0 | 0.2011 | 1 |
| 4 | 5 | 7 | 0 | 0.03756 | 1 |
| 6 | 6 | 9 | 0 | 0.0102 | 1 |
| 8 | 7 | 11 | 0 | 0.002245 | 1 |
| 10 | 8 | 13 | 0 | 0.01612 | 1 |
| 12 | 9 | 15 | 0 | 0.0004082 | 1 |
| 14 | 10 | 17 | 0 | 0.05143 | 1 |
| 16 | 11 | 19 | 0 | 0.05755 | 1 |
| 18 | 12 | 21 | 0 | 0.004082 | 1 |
| 20 | 13 | 23 | 0 | 0.05061 | 1 |

Figure 2.1.3-1 Static CQI (Distribution) – Test 2 2TX x 2RX Rank2 6dB

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 2 | 1 | 100% |
| 2 | 3 | 3 | 100% |
| 4 | 3 | 3 | 100% |
| 6 | 4 | 5 | 100% |
| 8 | 5 | 7 | 100% |
| 10 | 6 | 9 | 100% |
| 12 | 7 | 11 | 100% |
| 14 | 8 | 13 | 100% |
| 16 | 9 | 15 | 100% |
| 18 | 10 | 17 | 100% |
| 20 | 11 | 19 | 100% |

Figure 2.1.3-2 Static CQI (BLER) – Test 2 2TX x 2RX Rank2 6dB

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 1 | 0 | 0 | 1 |
| 2 | 3 | 3 | 0 | 0.005579 | 1 |
| 4 | 3 | 3 | 0 | 0 | 1 |
| 6 | 4 | 5 | 0 | 0 | 1 |
| 8 | 5 | 7 | 0 | 0 | 1 |
| 10 | 6 | 9 | 0 | 0 | 0.97 |
| 12 | 7 | 11 | 0 | 0 | 0.9568 |
| 14 | 8 | 13 | 0 | 0 | 0.6905 |
| 16 | 9 | 15 | 0 | 0 | 1 |
| 18 | 10 | 17 | 0 | 0 | 0.989 |
| 20 | 11 | 19 | 0 | 0 | 0.2808 |

Figure 2.1.4-1 Static CQI (Distribution) – Test 2 2TX x 4RX Rank2 6dB

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 3 | 3 | 100% |
| 2 | 4 | 5 | 100% |
| 4 | 5 | 7 | 100% |
| 6 | 6 | 9 | 100% |
| 8 | 7 | 11 | 100% |
| 10 | 8 | 13 | 100% |
| 12 | 9 | 15 | 100% |
| 14 | 10 | 17 | 100% |
| 16 | 11 | 19 | 100% |
| 18 | 12 | 21 | 100% |
| 20 | 13 | 23 | 100% |

Figure 2.1.4-2 Static CQI (BLER) – Test 2 2TX x 4RX Rank2 6dB

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 3 | 0 | 0.0008264 | 1 |
| 2 | 4 | 5 | 0 | 0.0686 | 1 |
| 4 | 5 | 7 | 0 | 0.009711 | 1 |
| 6 | 6 | 9 | 0 | 0.03326 | 1 |
| 8 | 7 | 11 | 0 | 0 | 1 |
| 10 | 8 | 13 | 0 | 0 | 1 |
| 12 | 9 | 15 | 0 | 0.0002066 | 1 |
| 14 | 10 | 17 | 0 | 0.03099 | 1 |
| 16 | 11 | 19 | 0 | 0.06198 | 1 |
| 18 | 12 | 21 | 0 | 0.01632 | 1 |
| 20 | 13 | 23 | 0 | 0.03864 | 1 |

2.2 PMI

Figure 2.2.1~Figure 2.2.4 are the simulation result for wideband PMI test 1 and test 2 with 8TX (FR1).

Figure 2.2.1 Wideband PMI – Test 1 8TX x 2RX

Figure 2.2.2 Wideband PMI – Test 1 8TX x 4RX

Figure 2.2.3 Wideband PMI – Test 2 8TX x 2RX

Figure 2.2.4 Wideband PMI – Test 2 8TX x 4RX

2.3 RI

TBD

# 3	Conclusions

In the contribution, we provide our simulation on Static CQI and wideband PMI.

# 4	Reference

[1] R4-1811693, “Way forward on NR UE CSI requirements”, Samsung

Appendix

Table A.1 Static Wideband CQI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | 7D1S2U | DDDSU |
| Propagation channel | AWGN | AWGN | AWGN |
| Correlation and antenna configuration | 2*2, 2*4 with static channel in TS36.101 Annex B | 2*2, 2*4 with static channel in TS36.101 Annex B | 2Rx 2*2 with static channel in TS36.101 Annex B |
| resourceConfigType | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset | 5slot 1 slot offset | 10slot, 1 slot offset | 16slot, 1 slot offset |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| CSI-IM configuration | FFS | FFS | FFS |
| reportConfigType | periodic | periodic | periodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 | Opt1: 16/1 Opt2: 8/1 |
| Codebook Restriction | 010000 (fixed rank2) | 010000 (fixed rank2) | 010000 (fixed rank2) |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Table A.2 Fading Wideband COI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | 7D1S2U | DDDSU |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | TDL_A 30ns, 75Hz |
| Correlation and antenna configuration | 2Rx 2*2 4Rx: 2*4 [ULA High] for 2Rx [XP High] for 4Rx | 2Rx 2*2 4Rx: 2*4 [ULA High] for 2Rx [XP High ]for 4Rx | 2Rx 2*2 [XP High] |
| resourceConfigType | Periodic | Periodic | Opt1: Periodic Opt2: Aperiodic |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| reportConfigType | periodic | periodic | Opt1: Periodic Opt2: Aperiodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 | FFS |
| Codebook Restriction | 000001 | 000001 | 000001 |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Table A.3 Fading Sub-band COI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) |
| --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz |
| Transmission Scheme | 1 | 1 |
| Duplex Mode | FDD | TDD |
| DLUL configuration | N.A | 7D1S2U |
| Propagation channel | Clause B.2.4 in TS 36.101, with a =1,fd=5Hz and td =0.45us | Clause B.2.4 in TS 36.101, with a =1,fd=10Hz and td=0.1125us |
| Correlation and antenna configuration | Option1: 2*2 and 2*4 (baseline) Option 2: 1*2 and 1*4 As Static channel | Option1: 2*2 and 2*4 (baseline) Option 2: 1*2 and 1*4 As Static channel |
| resourceConfigType | Periodic | Periodic |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} |
| reportConfigType | periodic | periodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 |
| Codebook Restriction | 000001 | 000001 |
| Reporting granularity | Sub-band | Sub-band |
| Physical channel for reporting | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM |

Table A.4 Wideband PMI test cases (FR1 8TX and FR2 2TX)

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | 7D1S2U | DDDSU |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | TDL_A 30ns, 75Hz |
| Correlation and antenna configuration | 8*2 8*4 [XP spatially High] | 8*2 8*4 [XP spatially High] | 2*2 Option 1: ULA Low Option 2: ULA Medium Option 3: XP Medium A |
| resourceConfigType | aperiodic | aperiodic | aperiodic |
| NZP CSI-RS ports | 8 port {3000…3007} | 8 port {3000…30007} | 2 port {3000,3001} |
| reportConfigType | aperiodic | aperiodic | aperiodic |
| Reporting periodicity and slot offset | 3/1 | 5/1 | 12/1 |
| (N1,N2) | (4,1) | (4,1) | N.A |
| (O1,O2) | (4,1) | (4,1) | N.A |
| CodebookMode | 1 | 1 | N.A |
| Codebook Restriction | 0xFFFF | 0xFFFF | 001111 |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| MCS Table | 64QAM | 64QAM | 64QAM |
| Rank | 2 | 2 | 1 |
| FRC | MCS13 | MCS13 | Option 1:MCS13 (baseline) Option 2: MCS 4 |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| CQI table | Table 1 with 256QAM | Table 1 with 256QAM | Table 1 without 256QAM |

Table A.5 Wideband PMI test cases (FR1 4TX)

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) |
| --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz |
| Transmission Scheme | 1 | 1 |
| Duplex Mode | FDD | TDD |
| DLUL configuration | N.A | 7D1S2U |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz |
| Correlation and antenna configuration | 4*2 4*4 [XP spatially High] | 4*2 4*4 [XP spatially High] |
| resourceConfigType | aperiodic | aperiodic |
| NZP CSI-RS ports | 4 port {3000…3003} | 4 port {3000…30003} |
| reportConfigType | aperiodic | aperiodic |
| Reporting periodicity and slot offset | 3/1 | 5/1 |
| (N1,N2) | (2,1) | (2,1) |
| (O1,O2) | (4,-) | (4,-) |
| CodebookMode | 1 | 1 |
| Codebook Restriction | 11111111 | 11111111 |
| Reporting granularity | Wide-band | Wide-band |
| MCS Table | 64QAM | 64QAM |
| Rank | 1 | 1 |
| FRC | MCS13 | MCS13 |
| Physical channel for reporting | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS |
| CQI table | Table 1 with 256QAM | Table 1 with 256QAM |

Table A.6 RI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 FDD) | Test 3 FR2 |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | 7D1S2U | DDDSU |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | TDL_A 30ns, 75Hz |
| Correlation and antenna configuration | 2RX: 2*2 ULA Low, ULA High 4Rx: 2*4 ULA Low, ULA High. 4*4 ULA Low | 2Rx: 2*2 ULA Low, ULA High 4Rx: 2*4 ULA Low, ULA High 4*4 ULA Low | 2Rx 2*2 [ULA Low, XP High] |
| resourceConfigType | Periodic | Periodic | Opt1: Periodic Opt2: Aperiodic |
| NZP CSI-RS ports | 2/4 | 2/4 | 2 |
| reportConfigType | periodic | periodic | Opt1: Periodic Opt2: Aperiodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 | FFS |
| Codebook Restriction | FFS | FFS | FFS |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Table A.7 CSI simulation detailed parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| TDD UL/DL configurations | - FR1 TDD: 7D1S2U, S = 6D:4G:4U - FR2 TDD: DDSU, S=11D+3G |  |
| HARQ | - For PMI, RV sequence ={0,2,3,1} - For RI and CQI test cases ,no retransmission |  |
| CSI-RS resource/CSI reporting periodicity slot offset for periodic CSI reporting | - FR1 FDD 15 kHz: 5/1 - FR1 TDD 30 kHz: Option 1: 10/1 - FR2 TDD 120 kHz: - Option 1: 16/1 - Option 2: 8/1 |  |
| Minimum CSI delay for periodic CSI | 8ms |  |
| CSI-RS resource/CSI reporting interval and slot offset for aperiodic CSI reporting | - FR1 FDD 15 kHz: 3/1 with total CSI delay [8]slots - FR1 TDD 30 kHz: 5/1 with total CSI delay [12] slots - FR2 TDD 120 kHz: 12/1 with total CSI delay [27] slots |  |
| CSI reporting and CSI-RS resources Type for fading CQI test cases and RI test cases of FR2 | - Option 1: Periodic (previous meeting agreements) - Option 2: aperiodic (baseline) - Companies are encouraged to bring analysis for above options considering different CSI delay assumptions for periodic and aperiodic CSI reporting types |  |
| Number of HARQ process number | - FR1 FDD: 4 - FR1 TDD: 8 - FR2 TDD: 8 |  |

Table A.8 NZP CSI-RS configurations

| First subcarrier index in the PRB used for CSI-RS (k0, k1) | Row 3, (6,-) | Row 4,(0,-) | Row 8,(4,6) |
| --- | --- | --- | --- |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 13 | 13 | 5 |
| Number of CSI-RS ports (X) | 2 | 4 | 8 |
| CDM Type | FD-CDM2 | FD-CDM2 | CDM4 (FD2,TD2) |
| Density (ρ) | 1 | 1 | 1 |

Table A.9 ZP CSI-RS configurations

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| First subcarrier index in the PRB used for CSI-RS (k0) | Row 5, 4 | Row 4, 8 |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 9 | 13 |
| Number of CSI-RS ports (X) | 4 | 4 |
| CDM Type | FD-CDM2 | FD-CDM2 |
| Density (ρ) | 1 | 1 |

Table A.10 CSI-IM configurations

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| CSI-IM-RE-pattern | Pattern 0 | Pattern 1 |
| CSI-IM-ResourceMapping (kCSI-IM,lCSI-IM) | (4,9) | (8,13) |