3GPP TSG-RAN WG4 Meeting #89				  												R4-1815286

Spokane, US, 12 - 16 November 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.141-1: 4.9.2.3 Data content of Physical channels and Signals for NR-FR1-TM

Agenda item:	7.9.2.3

Document for:	Approval

# Introduction

Test models for BS conformance tests were further discussed during last RAN4#88bis meeting held in Chengdu. Good progress was achieved, with following agreements for FR1 captured in text proposals for test models and data content of physical channels and signals for FR1 test models [1, 2].

This contribution provides some corrections to section 4.9.2.3 Data content of Physical channels and Signals for NR-FR1-TM in TS 38.141-1.  Description of proposed change is provided in separate discussion paper [3]. Text proposal with updates for data content for FR1 is submitted in [4].

# References

- R4-1813882 TP for test model for FR1, Huawei

- R4-1813880 TP for data content for FR1, Nokia, Nokia Shanghai Bell

- R4-1815285 Corrections for test models and data content for FR1, Nokia, Nokia Shanghai Bell

- R4-1815288 TP to TS 38.141-1: 4.9.2 Test models, Nokia, Nokia Shanghai Bell

< Start of TP >

### 4.9.2.3	Data content of Physical channels and Signals for NR-FR1-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS38.211, Clause 5.2 [xx] which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of ‘0’ bits shall be generated prior to the scrambling.

Initialization of the scrambler and RE-mappers as defined in TS38.211 [xx] use the following additional parameters:

-	[ = 1 for the lowest configured carrier,  = 2 for the 2nd lowest configured carrier,…,  = n for the nth configured carrier]

-	Antenna ports starting with 1000 for PDSCH

-	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1 (single layer)

#### 4.9.2.3.1	PDCCH

-	PDCCH modulation to be QPSK as described in TS38.211, clause 5.1.3

-	For each slot the required amount of bits for all PDCCHs is as follows: 1 (# of PDCCH)* 1(# of CCE per PDCCH)* 6(REG per CCE)*9(data RE per REG)*2(bits per RE) with these parameters according to the NR -FR1-TM definitions in subclause 4.9.2.2

-	Generate this amount of bits according to ‘all 0’ data

-

-	1 CCE shall be according to TS38.211, clause 7.3.2 using non-interleaved CCE-to-REG mapping. PDCCH should occupy first symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH multiplexing and scrambling according to TS38.211, clause 7.3.2.3

-	NID = 0 in DM-RS sequence generation in TS38.211, clause 7.4.1.3

-	nRNTI = 0 in scrambling sequence generation in TS38.211, clause 7.3.2.3

-	Perform mapping to REs according to TS38.211, clause 7.3.2.5

#### 4.9.2.3.2		PDSCH

-	For each slot generate the required amount of bits for all PRBs according to ‘all 0’ data

-	NR-FR1-TMs utilize 1 user PDSCH transmissions distinguished by .

Table 4.9.2.3.2-1: Mapping of PRBs to  for NR-FR1-TM

| NR-FR1-TM1.1 | 0 for all PRBs |
| --- | --- |
| NR-FR1-TM1.2 | 0 for boosted PRBs 1 for de-boosted PRBs |
| NR-FR1-TM2 | 0 for all PRBs |
| NR-FR1-TM2a | 0 for all PRBs |
| NR-FR1-TM3.1 | 0 for all PRBs |
| NR-FR1-TM3.1a | 0 for all PRBs |
| NR-FR1-TM3.2 | 0 for QPSKPRBs 1 for 16QAM PRBs |
| NR-FR1-TM3.3 | 0 for 16QAM PRBs 1 for QPSK PRBs |

- 	The required amount of PDSCH ‘0’ bits within slot and allocated PRBs shall be generated

-	[Perform user specific scrambling according to TS38.211, clause 7.3.1.1.]

-	[Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS38.211, clause 6.3.2]

-	Perform mapping of the complex-valued symbols to layer according to TS38.211, clause 7.3.1.3.   Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is equal to 1.

-	Perform PDSCH mapping type A according to TS38.211

-	DM-RS sequence generation according to TS38.211, clause 7.4.1.1.1 where l is the OFDM symbol number within the slot, using symbol 3 and symbol 11 for every other subcarrier

-	DM-RS mapping according to TS38.211, clause 7.4.1.1.2 where type 1 configuration is considered.

< End of TP >