3GPP TSG RAN WG4 Meeting #89	R4-1815297

Spokane, US, November 12-16, 2018

Agenda Item:	7.9.2.3

Source:	Huawei, HiSilicon

Title:	TP for TS38.141-1 base conformation test models

Document for:	Approval

# Introduction

In the last meeting, many details for section 4.9 in TS38.141-1 were completed. During the process of merging test proposals, some details need clarification and editing. This text proposal provides suggested modification.

# Revisions

In addition to minor edits, the following sections are modified.

## Section 4.9.2.2

- Replace “subcarrier space” with “subcarrier spacing”

- Remove brackets regarding measurements of a special slot when the number of downlink symbols of that special slot is less than a threshold. The reason is that there are insufficient number of reference symbols for measurements.

- Add a sentence that measurements are performed starting on the third symbol of a downlink slot

- For Table 4.9.2.2-2, the number of available REGs is 12 = 6 RBs/symbol  1 REG / RB  2 symbols. Fix the values accordingly.

- The filler PDSCH in the first two symbols is not described. Added a table describing the filler PDSCH and for which tests it is used.

  - QPSK modulated and the power per RB is the same as the PDCCH

  - No reference symbols needed.

## Minor modifications

The following table summarizes the type of change in the listed sections.

Table 1. listing of sections with minor changes

| Section | Modification |
| --- | --- |
| 4.9.2.2.2 | Update boosting location formula according to [1] |
| 4.9.2.2.3 | Correct the location for the top RB Remove description of filler PSDCH |
| 4.9.2.2.7 | Update boosting location formula according to [1] |
| 4.9.2.2.8 | Update boosting location formula according to [1] |

## Section 4.9.2.3

Incorporate the changes in [2]

# Conclusion

Adopt the TP in the appendix.

# References

R4-1815299 “Boosting patterns in test models in TS38.141-1”, Huawei, HiSilicon, RAN4#89, Nov. 12-16, 2018.

R4-1815302, “PSDCH/PDCCH data content in test models in TS38.141-1”, Huawei, RAN4#89, Nov. 12-16, 2018.

# Appendix: TP

***************************** BEGIN ***********************

#### 4.9.2.2	NR FR1 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR FR1 test models (NR-FR1-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in TS 38.104 [2], table 5.3.2-2.

-	Normal CP

-	Virtual resource blocks of localized type

[Measurements start on the third symbol of a slot when the first symbol of a slot is a downlink symbol.]

For NR FR1-TDD, test models are derived based on the uplink/downlink configuration as showing in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon.

If the number of downlink symbols in a special slot is less than 7 symbols, no measurements will be performed on that special slot.

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Common physical channel parameters for all NR FR1 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 for PDSCH, and table 4.9.2.2-4. Specific physical channel parameters for NR FR1 test models are described in sections 4.9.2.2.1 to 4.9.2.2.8. Table 4.9.2.2-4 is not used for NR- FR1-TM2 and NR-FR1-TM2a.

Table 4.9.2.2-2: Common physical channel parameters for PDCCH

| Parameter |  |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number of PDCCH | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 12 |
| Aggregation level(s) | 1 |
| # of RBs not allocated for PDCCH in the each symbol | NRB – 6 |
| DM-RS configuration and density | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |
| ratio of PDCCH EPRE to DM-RS EPRS | 0 dB |
| Boosting level of control region | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH

| Parameter |  |
| --- | --- |
| DM-RS configuration and density | Type 1, Comb2 (every other subcarrier) in symbols 3 and 11 |
| ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

Table 4.9.2.2-4: Common physical channel parameters for Filler PDSCH

| Parameter |  |
| --- | --- |
| DM-RS | Not present |
| ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Modulation | QPSK |
| # of symbols used | 2 |
| Starting symbol number | 0 |
| # of RBs allocated for filler PSDCH in the each symbol | NRB – 6 |

#### 4.9.2.2.1	NR FR1 test model 1.1 (NR-FR1-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Transmitter intermodulation

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter |  |
| --- | --- |
| # of QPSK PDSCH PRBs | NRB |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |

#### 4.9.2.2.2	NR FR1 test model 1.2 (NR-FR1-TM1.2)

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

Table 4.9.2.2.2-1: Physical channel parameters of NR-FR1-TM-1.2

| Parameter | SCS: x kHz, BW: y MHz |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | 3 |
| Location of PDSCH RBGs which are boosted | NRBP-1, 2n, n=10,…,xNRBP-3xNRBP≥12NRBP+3xNRBP-2otherwisexNRBP-2 1 if xNRBP≥12NRBP+3 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-PxNRBP |
| Level of deboosting (dB) | 10log10NRB-PxNRBPNRB-103/10PxNRBP |

#### 4.9.2.2.3	NR FR1 test model 2 (NR-FR1-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Table 4.9.2.2.3-1: Specific physical channel parameters of NR-FR1-TM2

| Parameter |  |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1-1; n=0,…,10×2μ-23-1. |
| # of PDSCH PRBs which are not allocated | NRB-1 |
|  | NRB-6 |

#### 4.9.2.2.4	NR FR1 test model 2a (NR-FR1-TM2a)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 256QAM PRB allocation (at min power)

-	Frequency error (at min power)

Physical channel parameters and numbers of the allocated PRB are defined in table 4.9.2.2.3-1 with all 64QAM PDSCH PRBs replaced by 256QAM PDSCH PRBs.

#### 4.9.2.2.5	NR FR1 test model 3.1 (NR-FR1-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

Physical channel parameters are defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 64QAM PDSCH PRBs.

#### 4.9.2.2.6	NR FR1 test model 3.1a (NR- FR1-TM3.1a)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 256QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 256QAM modulation (at max power)

Physical channel parameters are defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 256QAM PDSCH PRBs.

#### 4.9.2.2.7	NR FR1 test model 3.2 (NR-FR1-TM3.2)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

Table 4.9.2.2.7-1: Specific physical channel parameters of NR-FR1-TM3.2

| Parameter |  |
| --- | --- |
| Percent of 16QAM PDSCH PRBs boosted | x=60% |
| # of 16QAM PDSCH PRBs within a slot for which EVM is measured | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | -3 |
| Location of 16QAM PRBs which are boosted | NRBP-1, 2n, n=10,…,xNRBP-3xNRBP≥12NRBP+3xNRBP-2otherwisexNRBP-2 1 if xNRBP≥12NRBP+3 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-PxNRBP |
| Level of boosting (dB) | 10log10NRB-PxNRBPNRB-10-3/10PxNRBP |

#### 4.9.2.2.8	NR FR1 test model 3.3 (NR-FR1-TM3.3)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for QPSK modulation

Table 4.9.2.2.8-1: Specific physical channel parameters of NR-FR1-TM3.3

| Parameter |  |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted | x=50% |
| # of QPSK PDSCH RBGs within a slot for which EVM is measured | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | -6 |
| Location of QPSK RBGs which are boosted | NRBP-1, 2n, n=10,…,xNRBP-3xNRBP≥12NRBP+3xNRBP-2otherwisexNRBP-2 1 if xNRBP≥12NRBP+3 |
| # of 16QAM PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-PxNRBP |
| Level of boosting (dB) | 10log10NRB-PxNRBPNRB-10-6/10PxNRBP |

### 4.9.2.3	Data content of Physical channels and Signals for NR-FR1-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS38.211, Clause 5.2 [xx] which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of ‘0’ bits shall be generated prior to the scrambling.

For the test models, a single bandwidth part spans NRB RBs.

It is assumed that the PDCCH DM-RS and PDSCH DM-RS to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters.

Initialization of the scrambler and RE-mappers as defined in TS38.211 [xx] use the following additional parameters:

-	NIDcell=1 = 1 for the lowest configured carrier, NIDcell=2 for the 2nd lowest configured carrier, …, NIDcell=n for the nth configured carrier

-	Antenna ports starting with 1000 for PDSCH

-	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1 (single layer)

#### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS38.211, clause 5.1.3

-	For each slot the required amount of bits for all PDCCHs is as follows: (# of PDCCH)*(# of CCE per PDCCH)* 6(REG per CCE)*12(RE per REG)*2(bits per RE) with these parameters according to the NR-FR1-TM definitions in subclause 4.9.2.2

-	Generate this amount of bits according to ‘all 0’ data

NsymbCORESET= 1

-	1 CCE shall be according to TS38.211 [xx], clause 7.3.2 using non-interleaved CCE-to-REG mapping. PDCCH should occupy first symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS38.211 [xx], clause 7.3.2.3

-	nID=NIDcell and nRNTI=0 for scrambling

-	PDCCH modulation to be QPSK as described in TS38.211 [xx], clause 7.3.2.4

-	Perform mapping to REs according to TS38.211 [xx], clause 7.3.2.5

-	nID=NIDcell in DM-RS sequence generation in TS38.211 [xx], clause 7.4.1.3

#### 4.9.2.3.2		PDSCH

-	For each slot generate the required amount of bits for all PRBs according to ‘all 0’ data

-	NR-FR1-TMs utilize 1 user PDSCH transmissions distinguished by . For the filler PDSCH, nRNTI=2.

Table 4.9.2.3.2-1: Mapping of PRBs to  for NR-FR1-TM

| NR-FR1-TM1.1 | 0 for all PRBs |
| --- | --- |
| NR-FR1-TM1.2 | 0 for boosted PRBs 1 for de-boosted PRBs |
| NR-FR1-TM2 | 0 for all PRBs |
| NR-FR1-TM2a | 0 for all PRBs |
| NR-FR1-TM3.1 | 0 for all PRBs |
| NR-FR1-TM3.1a | 0 for all PRBs |
| NR-FR1-TM3.2 | 0 for QPSK PRBs 1 for 16QAM PRBs |
| NR-FR1-TM3.3 | 0 for 16QAM PRBs 1 for QPSK PRBs |

-	Perform user specific scrambling according to TS38.211 [xx], clause 7.3.1.1.

-	nID=NIDcell

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS38.211 [xx], clause 7.3.1.2

-	Perform mapping of the complex-valued symbols to layer according to TS38.211 [20], clause 7.3.1.3.   Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is equal to 1.

-	Perform PDSCH mapping type A according to TS38.211 [xx]

-	DM-RS sequence generation according to TS38.211 [xx], clause 7.4.1.1.1 where l is the OFDM symbol number within the slot, using symbol l=3 and symbol l=11 for every other subcarrier

-	NIDnSCID=NIDcell and nSCID=0

-	DM-RS mapping according to TS38.211 [xx], clause 7.4.1.1.2 where type 1 configuration is considered.

***************************** END ***********************