3GPP TSG-RAN4 Meeting #89	R4-1815319

# Spokane, US, 12th - 16th November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0018 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Adding technical background FBW declaration of EIRP in sub-clause 9.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The reason for change is to add missing technical background information related to manufacturer declarations part of radiated transmit power requirement in TS 38.104, sub-clause 9.2. This is a revision of previous version (R4-1812900) submitted to RAN4#88bis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In R4-1806907 a concept to allow for separate declaration for EIRP for wide supported NR bands or sub-bands was introduced in TS 38.104, sub-clause 9.2. Due to the physical size of the antenna aperture the directivty will change over large frequency bands, therefore a concept for declaring EIRP for lowest supported frequency and highest supported freqeuncy was introduced in NR. Currently, the technical background and justifiaction is not documented in TR 38.817-02. This CR adds relevant background information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, TR 38.817-02 will not capture all technical background information used to develop RF core requirements specified in TS 38.104. Information related to manufacturer declarations relevant for EIRP and radiated transmit power for wide NR operating bands could be missused and/or missunderstood. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 9.2	Radiated transmit power

### 9.2.1	General

The minimum requirements for radiated transmit power, are placed on one or more manufacturer declared beam(s) over a declared OTA peak direction set. OTA requirements for NR BS output power are defined for directional EIRP requirements as radiated transmit power requirements (sub-clauses 9.2.2 and 9.2.3) and for TRP requirements as OTA base station output power (sub-clauses 9.3.2 and 9.3.3).

Since NR will support legacy band and new wider bands the impact on EIRP due to large operating band need some consideration. In Table 9.2.1-1, the operating band width is studied in terms of absolute bandwidth, relative bandwidth and fractional bandwidth for some wide E-UTRA and NR bands.

Table 9.2.1-1: Operating band comparsion

| Band | Band Definition fl to fh (MHz) | Absolute Bandwidth fh – fl (MHz) | Relative Bandwidth fh/fl (dB) | Fractional Bandwidth 100.(fh-fl)/fc (%) |
| --- | --- | --- | --- | --- |
| 44 | 703 to 803 | 100 | 0.6 | 13.3 |
| 39 | 1880 to 1920 | 40 | 0.1 | 2.1 |
| 40 | 2300 to 2400 | 100 | 0.2 | 4.3 |
| 38 | 2570 to 2620 | 50 | 0.08 | 1.9 |
| 41, n41 | 2496 to 2690 | 194 | 0.3 | 7.5 |
| 42 | 3400 to 3600 | 200 | 0.2 | 5.7 |
| 43 | 3600 to 3800 | 200 | 0.2 | 5.4 |
| n77 | 3300 to 4200 | 900 | 1.0 | 24.0 |
| n78 | 3300 to 3800 | 500 | 0.6 | 14.1 |
| 48 | 3550 to 3700 | 150 | 0.2 | 4.1 |
| n79 | 4400 to 5000 | 600 | 0.6 | 12.8 |
| 46 | 5150 to 5925 | 775 | 0.6 | 14.0 |
| n257 | 26500 to 29500 | 3000 | 0.5 | 10.7 |
| n258 | 24250 to 27500 | 3250 | 0.5 | 12.6 |
| n260 | 37000 to 40000 | 2500 | 0.3 | 6.5 |
| n261 | 27500 to 28350 | 850 | 0.13 | 3.0 |

Based on the analysis of bandwidth, its clear that for antenna design point of view it is reasonable to consider fractional bandwidth as key parameter for wide band antennas. The fractional bandwidth of an antenna is a measure of how wideband the antenna is. If the antenna operates at centre frequency fc between lower frequency fl and upper frequency fh, then the fractional bandwidth FBW is given in percent as:

FBW=100fh-flfc=200fh-flfh+fl   (%)

The fractional bandwidth varies between 0% and 200%. For NR, where new bands have been introduced with large bandwidths the directivity impact on radiated EIRP have been studied. The radiated transmit power in a specific direction in terms of EIRP can be expressed as:

EIRP = TRP + D in dBm, where TRP is the total radiated power in dBm and D is the directivity in dBi. For a Uniform Rectangular Array (URA) antenna, where elements are arranged in a uniform rectangular lattice the directivity per polarization is determined by:

D = DE + 10log(N), where N is the number of elements (relating to the array factor) and DE is the element directivity in dBi (relating to the element factor).

From TR 38.803, Annex C [2], the element directivity can be expressed as:

DE=10log4πAeffλ2

, where Aeff is the antenna aperture in m2 and  is the wave length in m. For a URA antenna, where elements as separated d m along both y-axis and the z-axis, the maximum area for the antenna aperture is limited to an area of (d)2 m2. Typically, the d is in the range of 0.5 to 0.7 and  is derived from the highest supported frequency.

Since the directivity is depending on frequency, it is interesting to analyze the directivity characteristics as function of very wide operation bands. For wide operating bands the antenna array needs to be designed to support a large bandwidth, B = fh – fl, where fh is the highest supported frequency in MHz and fl is the lowest supported frequency in MHz.

The directivity difference, , in dB due to large supported bandwidth can be expressed as the difference between the directivity at fl (Dl) and the directivity at fh (Dh) as:

δ=Dl-Dh=10log4πd2λ2λl24πd2λ2λh2=20logflfh

In Table 9.2.1-2, some wide NR bands have been analyzed with respect to directivity variations due to fixed antenna aperture.

Table 9.2.1-2: Directivity variation over supported frequency range

| Band |  (dB) |
| --- | --- |
| n41 | -0.6 |
| n77 | -2.1 |
| n78 | -1.2 |
| n79 | -1.1 |
| n257 | -0.9 |
| n258 | -1.1 |
| n260 | -0.7 |

It’s clear that the directivity is lower at lowest supported frequency compared with the highest supported frequency as expected. For a NR base station supporting wide bands the directivity charactersitcs can be handled by having multiple declarations of EIRP.

### 9.2.2	Minimum requirement for BS type 1-O

For requirement set 1-H and 1-O, the radiated transmit power requirements will be the same as those for E-UTRA, background information for the minimum requirement is captured in 3GPP TR 37.842 [8], sub-clause 7.2.4. The radiated transmit power requirements for FR1 are defined as the directional requirements on the output power accuracy for EIRP.

As the radiated transmit power for FR1 is based on the manufacturer’s declarations, the basic limit concept does not apply to this requirement.