3GPP TSG-RAN4 Meeting #89	R4-1815324

# Spokane, US, 12th - 16th November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | 0003 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.843: Procedure for RC and framework for MU and MU table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of RC for TRP measurements in TR 37.843 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of description, error sources and MU table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The pallete of TRP methods lack of test method to conserve test time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.2.3 and Annex E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of Text Proposal]

#### 10.5.2.3	Reverberation chamber

##### 10.5.2.3.1	General

A reverberation chamber (RC) is a shielded metal enclosure with rotating or moving stirrers, and optionally turntables. When properly designed, this facility can be used for non-directional antenna measurements, such as TRP. When measuring TRP of radiations with high antenna directivity, special care must be taken to characterize the working volume of the chamber, see xxx. The working volume of a characterized RC is the volume wherein the DUT must be placed.

The symbols ⟨x ⟩ and σ= stdx denotes the average value and standard deviation. The auto-correlation function is used to calculate the offset between statistically un-correlated samples and is calculated as

rk=1σ2N-1n=0N-1xn- x xmodn+k,N- x

where the modulus operator mod(x,y) is the remainder of x/y, here performing  a circular shift of the measurement samples. The threshold value for uncorrelated samples is

rlim≤0.371-7.22/N0.64

The distance klimbetween un-correlated samples is calculated as the minimum k-value satisfying rk≤rlim. The number of independent samples is calculated as

Nind=Nklim

##### 10.5.2.3.2	Chamber characterization

The purpose of the chamber characterization is to ensure that the effect of a non-uniform propagation conditions in the chamber has a small effect on the measurement result when the DUT is placed in an imaginary volume denoted the working volume. In the characterization procedure the actual mode of operation, including stirrer movement, DUT movement, diversity antenna usage, etc., shall be used. A reference Tx antenna (REF TX) is mounted as the DUT would be mounted in an actual measurement. The directivity of the reference antenna shall be at least as high as the directivity of the DUTs to be measured.

For measurement of spurious emissions, a directivity of [10 dBi] can be assumed. The REF TX and the measurement receive antenna (RX) are connected to a Network Analyzer (NA).

Figure X.1 Setup for characterization of a reverberation chamber

The measurements in the characterization procedure are performed with the REF TX placed in different positions and rotations throughout the working volume. The working volume shall be at least half a wavelength from the chamber walls and other electromagnetic reflective objects according to [3], and according to [1] this distance may be restricted to 0.75m below 100 MHz. The number of positions and orientations to use depends on the chamber size and the directivity of the REF TX. The volume spanned by the positions and orientations used, constitutes the characterized working volume and is associated with a certain contribution to the measurement uncertainty.

In cases where the DUT is placed on a moving turntable during the measurement, different orientations in orthogonal orientations to the turntable movement shall be tested. For example: if an azimuth rotation turntable is used, different vertical rotations shall be tested, whereas additional fixed azimuthal orientations are not needed.

For each position/orientation n of the REF TX, RC sample, and desired frequency f, the transfer function

Pn=S212

is measured. The explicit dependence on RC sample and frequency is not written out. At least 250 independent samples shall be used per position/orientation. Using a lower number is not compatible with the underlying analysis on measurement uncertainty, see [1, 2].

The following tests are performed and shall be verified for each frequency:

- TRP uniformity: For each position/orientation evaluate ⟨Pn⟩. The standard deviation of these average values shall be below [1.5 dB].

- Dynamic range: The dynamic range of each Pn shall be at least 20 dB.

- Line-of-sight component: The line-of-sight component quantified as the K-factor

K=S212S21-S212

shall be below [-15 dB].

- Independent samples: At least 250 independent samples shall be used.

##### 10.5.2.3.3	Calibration

In the calibration, the radiation efficiency η of the reference antenna (REF TX) is obtained. The calibration shall be repeated for each frequency being tested and for each test antenna. This calibration can be done in any suitable chamber, e.g. an anechoic chamber calibrated for gain and directivity measurement. In this case the efficiency is obtained as η= G/D.

##### 10.5.2.3.4	Procedure

| (a) | (b) | (c) |
| --- | --- | --- |
| Figure 1 The procedure for TRP measurements uses three consecutive setups: (a) Reference measurement, (b) ambient noise measurement (b), and (c) TRP measurement. |  |  |

- Place the DUT and the REF TX in the working volume of the RC. Turn off DUT power and DUT controls. See Figure 1 (a).

- Set the stirrers and turntables in the mode of operation used in the chamber characterization.

- Set the sampling rate as in the chamber characterization.

- Connect the REF TX and the RX antenna with a calibrated Network analyser (NA) and measure the scattering parameters S11r and S22r. Calculate the transfer function Pr=S21r2 and the mis-match efficiency 	M1r=1-S11r2.

- Disconnect the NA and connect the RX antenna to a calibrated Spectrum analyser (SA). Turn on the DUT control. Keep the DUT power off. Terminate the REF TX. See Figure 1 (b).

- Measure the voltage data Uamb and calculate the ambient power level as Pamb=Uamb250

- Calculate the ambient TRP level as TRPamb=ηM1rTrPamb

- Turn on the DUT power and measure the received voltage UEUT, see Figure 1c.

- Calculate the DUT power as PEUT=UEUT250

- Calculate the TRP from the DUT as TRPDUT=η M1rPEUTPr

- The following tests shall be performed on the measurement data:

  - The dynamic range TRPDUT/ TRPamb must be at least 20 dB

  - The line-of-sight component, quantified as the K-factor K=S212S21-S212 using reference measurement data, shall be below -15 dB.

  - The number of independent samples, calculated via the auto-correlation function, see xxx, shall be at least 250.

##### 10.5.2.3.5	MU assessment

###### 10.5.2.3.5.1	MU Budget

Table 10.5.2.2.4.1-1: Reverberation  Chamber uncertainty contributions Tx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Uncertainty of the measurement equipment | E5-1 |
| 2 | Impedance mismatch in the receiving chain | E5-2 |
| 3 | Random uncertainty | E5-3 |
| Stage 1: Calibration measurement |  |  |
| 4 | Reference antenna radiation efficiency | E5-4 |
| 5 | Mean value estimation of reference antenna mis-match efficiency | E5-5 |
| 6 | Line of sight component | E5-6 |
| 7 | Uncertainty of the Network Analyzer | E5-7 |
| 8 | Influence of the reference antenna feed cable | E5-8 |
| 9 | Mean value estimation of transfer function | E5-9 |
| 10 | Uniformity of transfer function | E5-10 |

10.5.2.3.5.2	MU Value

Table 10.5.2.2.4.2-1: Reverberation  Chamber uncertainty assessment Tx spurious emissions

| UID | Uncertainty source | Uncertainty value 380 MHz < f ≦ 3 GHz | Uncertainty value 3 GHz < f ≦ 3.8 GHz | Uncertainty value 3.8 GHz < f ≦ 12.75 GHz | Uncertainty value 12.75 GHz < f ≦ 19 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 380 MHz < f ≦ 3 GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 3.8 GHz | Standard uncertainty ui [dB] 3.8 GHz < f ≦ 12.75 GHz | Standard uncertainty ui [dB] 12.75 GHz < f ≦ 19 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.14 | 0.26 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.2 | 0.2 | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0 | 0 | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 | 0 | 0 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 6 | Line of sight component | 0.14 | 0.14 | 0.14 | 0.14 | Rectangular | √3 | 1 | 0.08 | 0.08 | 0.08 | 0.08 |
| 7 | Uncertainty of the Network Analyzer | 0.13 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 | 0.20 | 0.20 |
| 8 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 | 0.20 |
| 9 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 10 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 | 1.5 | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.66 | 1.68 | 1.70 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.25 | 3.29 | 3.34 | 3.38 |

Note: This MU budget is applicable if the data tests in procedure step 11) of Clause 10.5.2.3.4 are fulfilled.

#### 10.5.3.4	Test Tolerance

The conduced test tolerance for the mandatory spurious emissions requirements is zero. As the requirements are set by regulatory limits the same test tolerance is used for OTA.

TT=0.

[End of Text Proposal]

[Start of Text Proposal]

# E.4	Reverberation Chamber

###### E5-1 Uncertainty of the measurement equipment

###### This contribution originates from limited absolute level accuracy and non-linearity of the measurement equipment. The measurement equipment such as a BS simulator, spectrum analyzer, or power meter measures the received signal level in EIRP tests either as an absolute level or as a relative level. The uncertainty value will be indicated in the manufacturer's data sheet in logs.

###### E5-2 Impedance mismatch in the receiving chain

###### This contribution originates from multiple reflections between the receiving antenna and the power measurement equipment. The multiple reflections can produce an overall reflection that depends not only upon the individual reflections of each part but their reflective interactions as well. The combination loss by the overall reflection can be higher or lower than individual loss by multiple reflections. The combination loss is called the mismatch error and leads to the measurement uncertainty.

###### E5-3 Random uncertainty

###### The random uncertainty characterizes the undefined and miscellaneous effects which cannot be forecasted. One can estimate this type of uncertainty with a repeatability test by making a series of repeated measurement with a reference AAS BS without changing anything in the measurement set-up.

###### E5-4 Reference antenna radiation efficiency

###### Residue of uncertainty of reference antenna radiation efficiency after calibration.

###### E5-5 Mean value estimation of reference antenna mismatch efficiency

###### Error of the estimated mean related to the use of a finite number N of samples in the measurement of the reference antenna mismatch efficiency. The mean value estimation is calculated as σ/N where σ is the standard deviation of the series of measured values.

###### E5-6 Line of sight component

The error of the transfer function S212 caused by a non-zero mean value S21, and quantified by the K-factor. The contribution in the MU-budget enters as a relative error.

###### E5-7 Uncertainty of the network analyzer

###### This contribution originates from all uncertainties involved in the S21 measurement (including drift and frequency flatness) with a network analyzer. The uncertainty value will be indicated in the manufacturer's data sheet in logs. This uncertainty value can be found in Annex E and was a result of compromised value in order to align all test methods having this uncertainty contribution.

###### E5-8 Influence of the reference antenna feed cable

###### Before performing the calibration, the reference antenna feed cable loss has to be measured. The measurement can be done with a network analyzer to measure its S21 and uncertainty is introduced.

###### E5-9 Mean value estimation of transfer function

###### Error of the estimated mean related to the use of a finite number N of samples in the measurement of the transfer function. The mean value estimation is calculated as σ/N where σ is the standard deviation of the series of measured values.

###### E5-10 Uniformity of transfer function

Standard deviation over DUT positions and rotations of the transfer function S212. This uncertainty expresses the variations of measured TRP values with respect to translations and rotations of the DUT. Ideally, the TRP does neither depend on translations nor rotations of the DUT.

[End of Text Proposal]