3GPP TSG-RAN4 Meeting #89	R4-1815327

# Spokane, US, 12th - 16th November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0020 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.817-02: Addition of RC MU tables for unwanted emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding measurement uncertainty table for TRP measurement in a reverberation chamber. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This text proposal introduces reverberation chamber as an option to measure TRP both for FR1 and FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the TR will lack technical background for TRP measurements in RC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7.1.1.1, 12.7.1.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of Text Proposal]

#### 12.7.1.1.1	MU for TRP

###### 12.7.1.1.1.1	Reverberation Chamber MU Assessment

Descriptions of uncertainty contributors are given in [X]

Table 12.7.1.1.1.1-1: Reverberation Chamber uncertainty assessment Tx spurious emissions

| UID | Uncertainty source | Uncertainty value 380 MHz < f ≦ 3 GHz | Uncertainty value 3 GHz < f ≦ 3.8 GHz | Uncertainty value 3.8 GHz < f ≦ 12.75 GHz | Uncertainty value 12.75 GHz < f ≦ 25 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 380 MHz < f ≦ 3 GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 3.8 GHz | Standard uncertainty ui [dB] 3.8 GHz < f ≦ 12.75 GHz | Standard uncertainty ui [dB] 12.75 GHz < f ≦ 25 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.14 | 0.26 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.2 | 0.2 | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0 | 0 | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 | 0 | 0 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 6 | Line of sight component | 0.14 | 0.14 | 0.14 | 0.14 | Rectangular | √3 | 1 | 0.08 | 0.08 | 0.08 | 0.08 |
| 7 | Uncertainty of the Network Analyzer | 0.13 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 | 0.20 | 0.20 |
| 8 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 | 0.20 |
| 9 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 10 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 | 1.5 | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.66 | 1.68 | 1.70 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.25 | 3.29 | 3.34 | 3.38 |

[End of Text Proposal]

[Start of Text Proposal]

###### 12.7.1.2.2.3	Reverberation Chamber MU Assessment

Descriptions of uncertainty contributors are given in [X]

Table 12.7.1.2.2.3-1: Reverberation Chamber uncertainty assessment Tx spurious emissions 380 MHz-18 GHz

| UID | Uncertainty source | Uncertainty value 380 MHz < f ≦ 3 GHz | Uncertainty value 3 GHz < f ≦ 3.8 GHz | Uncertainty value 3.8 GHz < f ≦ 12.75 GHz | Uncertainty value 12.75 GHz < f ≦ 18 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 380 MHz < f ≦ 3 GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 3.8 GHz | Standard uncertainty ui [dB] 3.8 GHz < f ≦ 12.75 GHz | Standard uncertainty ui [dB] 12.75 GHz < f ≦ 18 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.14 | 0.26 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.2 | 0.2 | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0 | 0 | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 | 0 | 0 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 6 | Line of sight component | 0.14 | 0.14 | 0.14 | 0.14 | Rectangular | √3 | 1 | 0.08 | 0.08 | 0.08 | 0.08 |
| 7 | Uncertainty of the Network Analyzer | 0.13 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 | 0.20 | 0.20 |
| 8 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 | 0.20 |
| 9 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 10 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 | 1.5 | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.66 | 1.68 | 1.70 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.25 | 3.29 | 3.34 | 3.38 |

Table 12.7.1.2.2.3-2: Reverberation Chamber uncertainty assessment Tx spurious emissions 18 GHz – 60 GHz

| UID | Uncertainty source | Uncertainty value 18 GHz < f ≦ 26.5 GHz | Uncertainty value 26.5 GHz < f ≦ 40 GHz |  | Uncertainty value 40 GHz < f ≦ 60 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Uncertainty value 18 GHz < f ≦ 26.5 GHz | Uncertainty value 26.5 GHz < f ≦ 40 GHz | Uncertainty value 40 GHz < f ≦ 60 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.52 | 0.84 |  | 1.15 | Gaussian | 1 | 1 | 0.52 | 0.84 | 1.15 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 |  | 0.45 | U-shaped | √2 | 1 | 0.32 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0 | 0 |  | 0 | Gaussian | 1 | 1 | 0 | 0 | 0 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 |  | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | 0.27 |  | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 |
| 6 | Line of sight component | 0.14 | 0.14 | 0.14 |  | Rectangular | √3 | 1 | 0.08 | 0.08 | 0.08 |
| 7 | Uncertainty of the Network Analyzer | 0.20 | 0.30 | 0.30 |  | Gaussian | 1 | 1 | 0.20 | 0.30 | 0.30 |
| 8 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 |  | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 |
| 9 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 |  | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 |
| 10 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 |  | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.76 | 1.89 | 2.05 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.45 | 3.71 | 4.02 |

[End of Text Proposal]