3GPP TSG-RAN WG4 Meeting #89                 	R4-1815350

Spokane, US, 12 - 16 November 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUCCH simulation results

Agenda item:	7.13.2.3.1

Document for:	Discussion

Introduction

In the RAN4#88-bis meeting, a way forward was agreed for NR PUCCH demodulation in [1]. In this contribution we provide simulation results with the simulation assumptions agreed in the WF.

Simulation Results (ideal)

Simulation assumptions

The simulation results in the following section are based on the simulation assumptions that were agreed in the WF [1]:

Table 1: Simulation assumptions for simulation alignment

| Parameters |  | PUCCH Format 0 | PUCCH Format 1 | PUCCH Format 2 | PUCCH Format 3 | PUCCH Format 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | FR1: 15kHz: 5MHz, 10MHz, 20MHz; 30kHz: 10MHz, 20MHz, 40MHz, 100MHz FR2: 60kHz: 50MHz, 100MHz; 120kHz: 50MHz, 100MHz, 200MHz |  |  |  |  |
| Antenna |  | 1T2R, 1T4R and 1T8R for FR1; 1T2R for FR2 |  |  |  |  |
| Propagation condition |  | FR1: TDLC300-100; FR2: TDLA30-300 |  |  |  |  |
| Carrier frequency (GHz) |  | FR1: 4GHz; FR2: 30GHz |  |  |  |  |
| (#Bits, #Symbols, #PRBs) |  | (1, 1, 1) (1, 2, 1) | (2, 14,1) | (4, 1, 4) (22, 2, 9) | (16, 14, 1) (16, 4, 3) | (22, 14, 1) |
| DMRS pattern |  |  |  |  | FR1: Without additional DMRS for all cases With additional DMRS for cases with the number of OFDM symbols more than 9 FR2: Without additional DMRS |  |
| Modulation |  |  |  | QPSK | QPSK | QPSK |
| initialCyclicShift |  | 0 | 0 |  |  |  |
| StartingSymbolIndex |  | 13 for 1 symbol; 12 for 2 symbols; | 0 | 13 for 1 symbol; 12 for 2 symbols; | 0 | 0 |
| Index of orthogonal sequence (time-domain-OCC) |  |  | 0 |  |  |  |
| occ-Length |  |  |  |  |  | n2 |
| occ-Index |  |  |  |  |  | n0 |
| Test metric | DTX to Ack probability <1% | √ | √ | √ for bits <=11 | × | × |
|  | Missed Ack probability < 1% | √ | √ | √ | × | × |
|  | NACK2ACK < 0.1% | × | √ | FFS for bits <=11 | × | × |
|  | BLER < 1% | × | × | √ for bits >11 | √ | √ |

Simulation results format 0

Simulation results for PUCCH format 0 and with the parameters defined are shown in Table 2 to Table 4.

Table 2: Simulation results for Format 0, 1T2R, FR1

| PUCCH FR1 TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | SNR[dB]@1% Missed detection |
| --- | --- | --- | --- | --- | --- |
| Format 0 | 15kHz, 10MHz | 1 | 1 | 1 | 8.2 |
|  |  | 1 | 2 | 1 | 1.3 |
|  | 30kHz, 40MHz | 1 | 1 | 1 | 8.2 |
|  |  | 1 | 2 | 1 | 1.3 |

Table 3: Simulation results for Format 0, 1T2R, FR2

| PUCCH FR2 TDLA30-300 |  | Number of bits | Number of OFDM symbols | Number of PRBs | SNR[dB]@1% Missed detection |
| --- | --- | --- | --- | --- | --- |
| Format 0 | 60kHz, 100MHz | 1 | 1 | 1 | 7.4 |
|  |  | 1 | 2 | 1 | 2.6 |
|  | 120kHz, 100MHz | 1 | 1 | 1 | 7.4 |
|  |  | 1 | 2 | 1 | 1.6 |

Table 4: Simulation results for Format 0, 1T4R, FR1

| PUCCH FR1 TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | SNR[dB]@1% Missed detection |
| --- | --- | --- | --- | --- | --- |
| Format 0 | 15kHz, 10MHz | 1 | 1 | 1 | 1.0 |
|  |  | 1 | 2 | 1 | -3.0 |
|  | 30kHz, 40MHz | 1 | 1 | 1 | 1.4 |
|  |  | 1 | 2 | 1 | -2.7 |

Simulation results format 2

Simulation results for PUCCH format 2 and with the parameters defined are shown in Table 5 to Table 8.

Table 5: Simulation results for Format 2 1T2R, FR1

| PUCCH FR1 1T2R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR[dB]@1% Missed detection |
| --- | --- | --- | --- | --- | --- | --- |
| Format 2 | 15kHz, 10MHz | 4 | 1 | 4 | ACK miss | 1.6 |
|  |  |  |  |  | NACK to ACK | -3.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -1.6 |
|  | 30kHz, 40MHz | 4 | 1 | 4 | ACK miss | 0.6 |
|  |  |  |  |  | NACK to ACK | -4.2 |
|  |  | 22 | 2 | 9 | BLER@1% | -2.6 |

Table 6: Simulation results for Format 2 1T2R, FR2

| PUCCH FR2 1T2R, TDLA30-300 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR[dB]@1% Missed detection |
| --- | --- | --- | --- | --- | --- | --- |
| Format 2 | 60kHz, 100MHz | 4 | 1 | 4 | ACK miss | 2.8 |
|  |  |  |  |  | NACK to ACK | -3 |
|  |  | 22 | 2 | 9 | BLER@1% | -1 |
|  | 120kHz, 100MHz | 4 | 1 | 4 | ACK miss | 1.9 |
|  |  |  |  |  | NACK to ACK | -3.1 |
|  |  | 22 | 2 | 9 | BLER@1% | -1.7 |

Table 7: Simulation results for Format 2 1T4R, FR1

| PUCCH FR1 1T4R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR[dB]@1% Missed detection |
| --- | --- | --- | --- | --- | --- | --- |
| Format 2 | 15kHz, 10MHz | 4 | 1 | 4 | ACK miss | -3.4 |
|  |  |  |  |  | NACK to ACK | -7.8 |
|  |  | 22 | 2 | 9 | BLER@1% | -5.8 |
|  | 30kHz, 40MHz | 4 | 1 | 4 | ACK miss | -3.8 |
|  |  |  |  |  | NACK to ACK | -7.9 |
|  |  | 22 | 2 | 9 | BLER@1% | -6.3 |

Table 8: Simulation results for Format 2 1T8R, FR1

| PUCCH FR1 1T8R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR[dB]@1% Missed detection |
| --- | --- | --- | --- | --- | --- | --- |
| Format 2 | 15kHz, 10MHz | 4 | 1 | 4 | ACK miss | -7.7 |
|  |  |  |  |  | NACK to ACK | -10.9 |
|  |  | 22 | 2 | 9 | BLER@1% | -9.4 |
|  | 30kHz, 40MHz | 4 | 1 | 4 | ACK miss | -7.8 |
|  |  |  |  |  | NACK to ACK | -11 |
|  |  | 22 | 2 | 9 | BLER@1% | -9.6 |

Simulation results formats 1, 3, and 4

Simulation results for PUCCH formats 1, 3, and 4 with the parameters defined are shown in Table 9 to Table 10.

Table 9: Simulation results for Formats 1, 3, and 4 for FR1

| PUCCH FR1, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | 1T2R, SNR (dB) | 1T4R, SNR (dB) | 1T8R, SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format 1 | 15kHz, 10MHz | 2 | 14 | 1 | ACK miss (1%) | -6.9 | -10.7 | -13.6 |
|  |  |  |  |  | NACK to ACK (0.1%) | -5.5 | -10 | -13.6 |
|  | 30kHz, 40MHz |  |  |  | ACK miss (1%) | -7.4 | -11.1 | -13.8 |
|  |  |  |  |  | NACK to ACK (0.1%) | -6.2 | -10 | -13.5 |
| Format 3 | 15kHz, 10MHz | 16 | 14 | 1 | BLER (1%, without additional DMRS) | -1.8 | -5.8 | -9.1 |
|  |  |  |  |  | BLER (1%, with additional DMRS) | -2.4 | -6.7 | -10.1 |
|  |  | 16 | 4 | 3 | BLER (1%, without additional DMRS) | -0.6 | -5.5 | -9 |
|  | 30kHz, 40MHz | 16 | 14 | 1 | BLER (1%, without additional DMRS) | -2.2 | -6.3 | -9.4 |
|  |  |  |  |  | BLER (1%, with additional DMRS) | -2.9 | -7.2 | -10.5 |
|  |  | 16 | 4 | 3 | BLER (1%, without additional DMRS) | -1.6 | -6 | -9.2 |
| Format 4 | 15kHz, 10MHz | 22 | 14 | 1 | BLER (1%, without additional DMRS) | 0 | -4.6 | -8.1 |
|  |  |  |  |  | BLER (1%, with additional DMRS) | -0.3 | -5.2 | -9 |
|  | 30kHz, 40MHz |  |  |  | BLER (1%, without additional DMRS) | -0.5 | -5.1 | -8.4 |
|  |  |  |  |  | BLER (1%, with additional DMRS) | -0.8 | -5.6 | -9.2 |

Table 10: Simulation results for Formats 1, 3, and 4 for FR2

| PUCCH FR2, TDLA30-300 1T2R, without additional DMRS |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Format 1 | 60kHz, 100MHz | 2 | 14 | 1 | ACK miss (1%) | -7.1 |
|  |  |  |  |  | NACK to ACK (0.1%) | -5.9 |
|  | 120kHz, 100MHz |  |  |  | ACK miss (1%) | -7 |
|  |  |  |  |  | NACK to ACK (0.1%) | -5.9 |
| Format 3 | 60kHz, 100MHz | 16 | 14 | 1 | BLER (1%) | -1.9 |
|  |  | 16 | 4 | 3 | BLER (1%) | -0.9 |
|  | 120kHz, 100MHz | 16 | 14 | 1 | BLER (1%) | -2 |
|  |  | 16 | 4 | 3 | BLER (1%) | -1.3 |
| Format 4 | 60kHz, 100MHz | 22 | 14 | 1 | BLER (1%) | -0.5 |
|  | 120kHz, 100MHz |  |  |  | BLER (1%) | -0.4 |

Conclusion

In this contribution we have provided simulation results for NR PUCCH.

- References

- R4-1813943, Way forward on NR PUCCH demodulation requirements, Huawei, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018