3GPP TSG-RAN WG4 Meeting #89                 	R4-1815351

Spokane, US, 12 - 16 November 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PRACH simulation results

Agenda item:	7.13.2.4.1

Document for:	Discussion

Introduction

In the RAN4#88bis meeting, a way forward was agreed for NR PRACH demodulation in [1]. In this contribution we provide simulation results with the simulation assumptions agreed in the WF.

Simulation Results (ideal)

Simulation Assumptions

In the following we will provide simulation results based on the simulation assumptions as agreed earlier for NR PRACH in [1].

Table 1: Simulation assumptions for NR PRACH

|  |  |  | Ncs | v | Frequency Offset (Hz) | Channel type |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 4 GHz | Format 0 | 1.25kHz |  | 32 | 400 | AWGN and TDL |
|  | Format A1 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format A2 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format A3 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format B4 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format C0 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format C2 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
| FR2 30 GHz | Format A1 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format A2 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format A3 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format B4 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format C0 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format C2 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |

Further parameters and details:

Propagation condition details:

FR1 PRACH: AWGN, TDL-C 300ns, 100 Hz

FR2 PRACH: AWGN, TDL-A 30ns, 300Hz

System Bandwidth [2]:

FR1: 10MHz/15kHz; 40MHz/30kHz; 40MHz/1,25kHz

FR2: 100MHz/60kHz; 100MHz/120kHz

Target false alarm probability = 0.05%

Simulation results format 0

Simulation results for long PRACH format 0, based on parameters in Table 1 are shown below in Table 2 to Table 4.

Table 2: Simulation results for PRACH format 0, 1T2R

| PRACH FR1 1T2R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -16,35 |
|  |  | TDLC300-100 | 400 | -9,03 |

Table 3: Simulation results for PRACH format 0, 1T4R

| PRACH FR1 1T4R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -18,77 |
|  |  | TDLC300-100 | 400 | -14,56 |

Table 4: Simulation results for PRACH format 0, 1T8R

| PRACH FR1 1T8R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -21,09 |
|  |  | TDLC300-100 | 400 | -18,69 |

Simulation results short PRACH

Simulation results for short PRACH, based on parameters in Table 1 are shown below in Table 5 to Table 6.

Table 5: Simulation results for long PRACH, 1T2R, FR1

| PRACH FR1 1T2R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- | --- | --- |
| Format A1 | 15kHz | AWGN | 0 | -11,77 |
|  |  | TDLC300-100 | 400 |  |
|  | 30kHz | AWGN | 0 | -11,39 |
|  |  | TDLC300-100 | 400 |  |
| Format A2 | 15kHz | AWGN | 0 | -14,72 |
|  |  | TDLC300-100 | 400 |  |
|  | 30kHz | AWGN | 0 | -14,44 |
|  |  | TDLC300-100 | 400 |  |
| Format A3 | 15kHz | AWGN | 0 | -16,14 |
|  |  | TDLC300-100 | 400 |  |
|  | 30kHz | AWGN | 0 | -15,73 |
|  |  | TDLC300-100 | 400 |  |
| Format B4 | 15kHz | AWGN | 0 | -18,74 |
|  |  | TDLC300-100 | 400 |  |
|  | 30kHz | AWGN | 0 | -18,50 |
|  |  | TDLC300-100 | 400 |  |
| Format C0 | 15kHz | AWGN | 0 | -8,70 |
|  |  | TDLC300-100 | 400 |  |
|  | 30kHz | AWGN | 0 | -8,44 |
|  |  | TDLC300-100 | 400 |  |
| Format C2 | 15kHz | AWGN | 0 | -14,80 |
|  |  | TDLC300-100 | 400 |  |
|  | 30kHz | AWGN | 0 | -14,38 |
|  |  | TDLC300-100 | 400 |  |

Table 6: Simulation results for long PRACH, 1T2R, FR2

| PRACH FR2 1T2R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- | --- | --- |
| Format A1 | 60kHz | AWGN | 0 | -10,76 |
|  |  | TDLA30-300 | 4000 |  |
|  | 120kHz | AWGN | 0 | -10,78 |
|  |  | TDLA30-300 | 4000 |  |
| Format A2 | 60kHz | AWGN | 0 | -14,23 |
|  |  | TDLA30-300 | 4000 |  |
|  | 120kHz | AWGN | 0 | -14,19 |
|  |  | TDLA30-300 | 4000 |  |
| Format A3 | 60kHz | AWGN | 0 | -15,68 |
|  |  | TDLA30-300 | 4000 |  |
|  | 120kHz | AWGN | 0 | -15,63 |
|  |  | TDLA30-300 | 4000 |  |
| Format B4 | 60kHz | AWGN | 0 | -18,22 |
|  |  | TDLA30-300 | 4000 |  |
|  | 120kHz | AWGN | 0 | -18,29 |
|  |  | TDLA30-300 | 4000 |  |
| Format C0 | 60kHz | AWGN | 0 | -8,16 |
|  |  | TDLA30-300 | 4000 |  |
|  | 120kHz | AWGN | 0 | -8,23 |
|  |  | TDLA30-300 | 4000 |  |
| Format C2 | 60kHz | AWGN | 0 | -14,23 |
|  |  | TDLA30-300 | 4000 |  |
|  | 120kHz | AWGN | 0 | -14,26 |
|  |  | TDLA30-300 | 4000 |  |

Conclusion

In this contribution we have provided simulation results for NR PRACH.

- References

- R4-1813757, Way forward on NR PRACH demodulation requirements, Ericsson, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018

- R4-1813758, Work plan for NR BS demodulation performance requirements, China Telecom, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018