TSG-RAN Working Group 4 (Radio) meeting #89	R4-1815361

Spokane, US, 12 - 16 November 2018

Source:	Ericsson

Title:	Simulation results for PRACH requirements

Agenda item:	7.13.2.4.1

Document for:	Discussion

# Introduction

In this contribution, simulation results for initial simulation alignments based on agreed assumptions in [1] are presented herein.

# Simulation

# Simulation setups

The initial simulation results to be presented in section 2.2 are obtained using the agreed simulation parameters [1] as shown below:

Table 1. Configuration for aligned simulation scenarios

| Parameters | Short sequence (Format B4, A3, A2, A1) |  |  |  | Long sequence (Format 0) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 |  | FR2 |  | FR1 |
| Number of Tx | 1 |  |  |  | 1 |
| Carrier frequency (GHz) | 4 |  | 30 |  | 2 |
| Sub-Carrier Spacing (kHz) | 15 | 30 | 60 | 120 | 1.25 |
| Cyclic shift Ncs | 23 | 46 | 69 | 69 | 13 |
| Logical sequence index | 0 |  |  |  | 22 |
| Number of Rx | 2, 4, 8 | 2, 4, 8 | 2 | 2 | 2, 4, 8 |
| Propagation Channel | AWGN, TDL-C simplified | AWGN, TDL-C simplified | AWGN, TDL-A simplified | AWGN, TDL-A simplified | AWGN, TDL-C simplified |
| v value | 0 |  |  |  | 32 |

To be specific, the parameters of the AWGN and TDL simplified channels are provided below.

Table 2. Channel specification

|  | TDL-C simplified | TDL-A simplified | AWGN |
| --- | --- | --- | --- |
| Reference | [2] | [2] |  |
| Delay spread (r.m.s.) | 300 ns | 30 ns |  |
| Maximum Doppler frequency | 100 Hz | 300 Hz |  |
| Frequency offset | 400 Hz | 4000 Hz | 0 Hz |

## Simulation results

In this sub-section, the simulation results are presented.

Table 3. PRACH performance for FR1 with 1Tx and 2Rx - ideal

| Format | Subcarrier Spacing | Propagation Channel | SNR demanded (dB) |
| --- | --- | --- | --- |
| Format 0 | 1.25 kHz | AWGN | -16.9 |
|  |  | TDL-C simplified | -9.5 |
| Format A1 | 15 kHz | AWGN | -11.9 |
|  |  | TDL-C simplified | -4.5 |
|  | 30 kHz | AWGN | -11.8 |
|  |  | TDL-C simplified | -5.2 |
| Format A2 | 15 kHz | AWGN | -14.9 |
|  |  | TDL-C simplified | -7.5 |
|  | 30 kHz | AWGN | -14.8 |
|  |  | TDL-C simplified | -8.2 |
| Format A3 | 15 kHz | AWGN | -16.7 |
|  |  | TDL-C simplified | -8.9 |
|  | 30 kHz | AWGN | -16.6 |
|  |  | TDL-C simplified | -9.9 |
| Format B4 | 15 kHz | AWGN | -19.4 |
|  |  | TDL-C simplified | -10.5 |
|  | 30 kHz | AWGN | -19.3 |
|  |  | TDL-C simplified | -12.3 |

Table 3. PRACH performance for FR1 with 1Tx and 2Rx – with impairments

| Format | Subcarrier Spacing | Propagation Channel | SNR demanded (dB) |
| --- | --- | --- | --- |
| Format 0 | 1.25 kHz | AWGN | -14.5 |
|  |  | TDL-C simplified | -7.4 |
| Format A1 | 15 kHz | AWGN | -9.7 |
|  |  | TDL-C simplified | -2.2 |
|  | 30 kHz | AWGN | -9.3 |
|  |  | TDL-C simplified | -2.7 |
| Format A2 | 15 kHz | AWGN | -12.8 |
|  |  | TDL-C simplified | -5.0 |
|  | 30 kHz | AWGN | -12.3 |
|  |  | TDL-C simplified | -5.9 |
| Format A3 | 15 kHz | AWGN | -14.2 |
|  |  | TDL-C simplified | -6.8 |
|  | 30 kHz | AWGN | -14.3 |
|  |  | TDL-C simplified | -7.4 |
| Format B4 | 15 kHz | AWGN | -17.0 |
|  |  | TDL-C simplified | -8.0 |
|  | 30 kHz | AWGN | -16.9 |
|  |  | TDL-C simplified | -10.2 |

Table 4. PRACH performance for FR1 with 1Tx and 4Rx - ideal

| Format | Subcarrier Spacing | Propagation Channel | SNR demanded (dB) |
| --- | --- | --- | --- |
| Format 0 | 1.25 kHz | AWGN | -19.4 |
|  |  | TDL-C simplified | -15.1 |
| Format A1 | 15 kHz | AWGN | -14.3 |
|  |  | TDL-C simplified | -9.6 |
|  | 30 kHz | AWGN | -14.2 |
|  |  | TDL-C simplified | -9.5 |
| Format A2 | 15 kHz | AWGN | -17.3 |
|  |  | TDL-C simplified | -12.5 |
|  | 30 kHz | AWGN | -17.2 |
|  |  | TDL-C simplified | -12.5 |
| Format A3 | 15 kHz | AWGN | -19.1 |
|  |  | TDL-C simplified | -14.0 |
|  | 30 kHz | AWGN | -18.9 |
|  |  | TDL-C simplified | -14.2 |
| Format B4 | 15 kHz | AWGN | -21.8 |
|  |  | TDL-C simplified | -15.5 |
|  | 30 kHz | AWGN | -21.7 |
|  |  | TDL-C simplified | -16.5 |

Table 4A. PRACH performance for FR1 with 1Tx and 4Rx – with impairments

| Format | Subcarrier Spacing | Propagation Channel | SNR demanded (dB) |
| --- | --- | --- | --- |
| Format 0 | 1.25 kHz | AWGN | -16.9 |
|  |  | TDL-C simplified | -13.0 |
| Format A1 | 15 kHz | AWGN | -12.0 |
|  |  | TDL-C simplified | -7.4 |
|  | 30 kHz | AWGN | -11.8 |
|  |  | TDL-C simplified | -7.1 |
| Format A2 | 15 kHz | AWGN | -15.2 |
|  |  | TDL-C simplified | -10.2 |
|  | 30 kHz | AWGN | -14.9 |
|  |  | TDL-C simplified | -10.1 |
| Format A3 | 15 kHz | AWGN | -16.7 |
|  |  | TDL-C simplified | -11.6 |
|  | 30 kHz | AWGN | -16.7 |
|  |  | TDL-C simplified | -11.8 |
| Format B4 | 15 kHz | AWGN | -19.4 |
|  |  | TDL-C simplified | -13.4 |
|  | 30 kHz | AWGN | -19.5 |
|  |  | TDL-C simplified | -14.2 |

Table 5.  PRACH performance for FR1 with 1Tx and 8Rx - ideal

| Format | Subcarrier Spacing | Propagation Channel | SNR demanded (dB) |
| --- | --- | --- | --- |
| Format 0 | 1.25 kHz | AWGN | -21.6 |
|  |  | TDL-C simplified | -19.0 |
| Format A1 | 15 kHz | AWGN | -16.5 |
|  |  | TDL-C simplified | -13.4 |
|  | 30 kHz | AWGN | -16.4 |
|  |  | TDL-C simplified | -12.8 |
| Format A2 | 15 kHz | AWGN | -19.5 |
|  |  | TDL-C simplified | -16.3 |
|  | 30 kHz | AWGN | -19.4 |
|  |  | TDL-C simplified | -15.8 |
| Format A3 | 15 kHz | AWGN | -21.3 |
|  |  | TDL-C simplified | -17.7 |
|  | 30 kHz | AWGN | -21.1 |
|  |  | TDL-C simplified | -17.5 |
| Format B4 | 15 kHz | AWGN | -24.0 |
|  |  | TDL-C simplified | -19.2 |
|  | 30 kHz | AWGN | -23.8 |
|  |  | TDL-C simplified | -20.0 |

Table 5A.  PRACH performance for FR1 with 1Tx and 8Rx – with impairments

| Format | Subcarrier Spacing | Propagation Channel | SNR demanded (dB) |
| --- | --- | --- | --- |
| Format 0 | 1.25 kHz | AWGN | -19.1 |
|  |  | TDL-C simplified | -16.8 |
| Format A1 | 15 kHz | AWGN | -14.2 |
|  |  | TDL-C simplified | -11.2 |
|  | 30 kHz | AWGN | -14.0 |
|  |  | TDL-C simplified | -10.5 |
| Format A2 | 15 kHz | AWGN | -17.2 |
|  |  | TDL-C simplified | -13.9 |
|  | 30 kHz | AWGN | -16.9 |
|  |  | TDL-C simplified | -13.3 |
| Format A3 | 15 kHz | AWGN | -19.0 |
|  |  | TDL-C simplified | -15.6 |
|  | 30 kHz | AWGN | -19.0 |
|  |  | TDL-C simplified | -15.3 |
| Format B4 | 15 kHz | AWGN | -21.5 |
|  |  | TDL-C simplified | -17.0 |
|  | 30 kHz | AWGN | -21.3 |
|  |  | TDL-C simplified | -17.8 |

Table 6.  PRACH performance for FR2 with 1Tx and 2Rx - ideal

| Format | Subcarrier Spacing | Propagation Channel | SNR demanded (dB) |
| --- | --- | --- | --- |
| Format A1 | 60 kHz | AWGN | -11.6 |
|  |  | TDL-A simplified | -3.8 |
|  | 120 kHz | AWGN | -11.6 |
|  |  | TDL-A simplified | -4.3 |
| Format A2 | 60 kHz | AWGN | -14.6 |
|  |  | TDL-A simplified | -6.0 |
|  | 120 kHz | AWGN | -14.6 |
|  |  | TDL-A simplified | -7.2 |
| Format A3 | 60 kHz | AWGN | -16.4 |
|  |  | TDL-A simplified | -6.1 |
|  | 120 kHz | AWGN | -16.4 |
|  |  | TDL-A simplified | -8.6 |
| Format B4 | 60 kHz | AWGN | -19.2 |
|  |  | TDL-A simplified | 2.2 |
|  | 120 kHz | AWGN | -19.2 |
|  |  | TDL-A simplified | -9.8 |

Table 6A.  PRACH performance for FR2 with 1Tx and 2Rx – with impariments

| Format | Subcarrier Spacing | Propagation Channel | SNR demanded (dB) |
| --- | --- | --- | --- |
| Format A1 | 60 kHz | AWGN | [-9.1] |
|  |  | TDL-A simplified | [-1.3] |
|  | 120 kHz | AWGN | [-9.2] |
|  |  | TDL-A simplified | [-2.1] |
| Format A2 | 60 kHz | AWGN | [-12.2] |
|  |  | TDL-A simplified | [-3.8] |
|  | 120 kHz | AWGN | [-12.2] |
|  |  | TDL-A simplified | [-5.1] |
| Format A3 | 60 kHz | AWGN | [-14.0] |
|  |  | TDL-A simplified | [-4.0] |
|  | 120 kHz | AWGN | [-14.2] |
|  |  | TDL-A simplified | [-6.5] |
| Format B4 | 60 kHz | AWGN | [-17.1] |
|  |  | TDL-A simplified | [4.6] |
|  | 120 kHz | AWGN | [-16.8] |
|  |  | TDL-A simplified | [-7.6] |

# Conclusion

Simulation results for initial simulation alignments are presented in Section 2.

# References

R4-1813757, “WF on NR PRACH demodulation requirements”, Ericsson

R4-1814066, “TP on channel models for TS38.101-4”, Huawei, HiSilicon, Intel, Spirent, Ericsson