3GPP TSG-RAN WG4 Meeting # 89	R4-1815589

# Spokane, USA, 12 November, 2018 - 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of side condition in FR2 handover |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  |  |  | Date: |  | 2018/02/11 |
| Category: | F |  |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Side condition of “provided the signal quality is sufficient for successful cell detection on the first attempt” is ill-defined. This becomes more important for handover to known (already measured) cell. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace with target cell Es/Iot≥-3dB |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not perform handover successfully even in good conditions, because the condition for cell detection on first attempt is not specifeid. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |

#### 6.1.1.4	NR FR2- NR FR2 Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR2 cell to NR FR2 cell.

##### 6.1.1.4.1	Handover delay

Procedure delays for all procedures that can command a handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.4.2.

##### 6.1.1.4.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing+ T∆ ms

Where:

Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is an intra-frequency cell and the target cell Es/Iot≥-3dB, then Tsearch = [8* Trs + 2] ms. If the target cell is an inter-frequency cell and the target cell Es/Iot≥-3dB, then Tsearch = [8*3* Trs + 2] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = [1]* Trs.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of [6].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell prior to or in the handover command, otherwise the requirement in this section is applied with Trs=5ms unless the SSB transmission periodicity is not 5ms. If UE is provided with both SMTC configuration and SSB transmission periodicity the requirement shall be based on SMTC periodicity.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

NOTE 2:	Void

#### 6.1.1.5	NR FR1- NR FR2 Handover

The requirements in this clause are applicable to inter-frequency handovers from NR FR1 cell to NR FR2 cell.

##### 6.1.1.5.1	Handover delay

Procedure delays for all procedures that can command a handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.5.2.

##### 6.1.1.5.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing+ T∆ ms

Where:

Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is an intra-frequency cell and  the target cell Es/Iot≥-3dB, then Tsearch = [8* Trs + 2] ms. If the target cell is an inter-frequency cell and the target cell Es/Iot≥-3dB,, then Tsearch = [8*3* Trs + 2] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

Tprocessing is time for UE processing. Tprocessing can be up 40ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = [1]* Trs.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of [6].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell prior to or in the handover command, otherwise the requirement in this section is applied with Trs=5ms unless the SSB transmission periodicity is not 5ms. If UE is provided with both SMTC configuration and SSB transmission periodicity the requirement shall be based on SMTC periodicity.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

NOTE 2:	Void