3GPP TSG-RAN WG4 #89	R4-1815713

Spokane, USA, 12th – 16th Nov 2018

Agenda Item:	6.11.3

Source:	Ericsson

Title:		Simulation results and proposals for network-based CRS interference mitigation UE demodulation tests

Document for:	Approval

# Introduction

The WF on the UE demod test for network-based CRS interference mitigation WI [1] was agreed in the document [2] as following. In this contribution we provide simulation results and proposals for the tests, to conclude the WI in time.

- Define performance requirements for the UE supporting nw-BasedCRS-InterferenceMitigation-r15.

  - The baseline receiver for such requirements is LMMSE-IRC

  - CRS muting pattern

    - Option 1: The interference cell CRS is configured with CRS muting pattern where every [two] radio frames consist of [11] subframes with full system BW CRS followed by [9] subframes with CRS only on the center 6 PRBs.

    - Option 2: The interference cell CRS muting is applied in all subframes.

    - One of the options above will be decided in next meeting in order to close the WI.

- Define one TM3 test with 2Rx only

- Simulation assumptions are listed in next page and companies are encouraged to bring alignment and impairment results for both options in next meeting.

Simulation assumptions for TM3 tests

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (NOTE 1) | -3 (NOTE 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| PDSCH transmission mode |  |  | 3 | NA |
| CRS transmission |  |  | On | . See slide 2 |
| nw-BasedCRS-InterferenceMitigation-r15 |  |  | Disabled | Enabled |
| Cell ID |  |  | 0 | 1 |
| Time offset |  |  | NA | 0 |
| Frequency offset |  |  | NA | 0 |
| E_s/N_oc |  |  | NA | For FRC Option 1: 10 For FRC Option 2: 5 |
| Correlation matrix and antenna config. |  |  | 2x2 Low | 2x2 Low |
| Bandwidth |  |  | 10MHz | 10MHz |
| Channel model |  |  | [EVA5] | [EVA5] |
| FRC |  |  | Option 1: R.11 (FDD/TDD) Option 2: R.10 (FDD/TDD) Modify FRC such that only 41PRB are allocated and 9 center PRBs are excluded | NA |
| NOTE 1: . NOTE 2: Interested companies can bring results with other FRC and interference level with the purpose to identify any technical issues if there is any. |  |  |  |  |

# Simulation results and proposals

For the CRS muting pattern between Option 1 and Option 2 there is no direction comparison when it comes to performance. The only basis should be based on the practical usage. We take Option 1 as more practical setup to reflect the actual network condition, as in reality it’s too optimistic to always assume all the interfering cells are CRS muted all the time. So, we propose to use Option 1 as the CRS muting pattern.

Proposal 1: To reflect the practical network we should take Option 1 with the interference cell CRS configured with CRS muting pattern where every two radio frames consist of 11 subframes with full system BW CRS followed by 9 subframes with CRS only on the center 6 PRBs.

Among the FRC options with different power level for the interfering cells we have run more results as shown in Figure 1 for FRC Option 1 and Figure 2 for FRC Option 2 with different Es/Noc levels.

Figure 1 FRC Option 1 with different Es/Noc levels

Figure 2 FRC Option 2 with different Es/Noc levels

Based on the results above we think it’s good enough to choose 16QAM on the serving cell with Noc=10dB, to represent a practical condition.

Proposal 2: Choose 16QAM with R.11 with only 41PRB allocated and Es/Noc=10dB on the interfering cell for defining the PDSCH demodulation performance requirement for nw-based CRS-IM.

Furthermore, we collect the alignment/impairment results in [3] as attached with our results filled in and a CR to introduce the tests in [4].

# Conclusion

In this paper, we discussed how to define performance requirements for LTE_NW_CRS_IM, based on the discussion, we have the following proposals:

- To reflect the practical network, we should take Option 1 with the interference cell CRS configured with CRS muting pattern where every two radio frames consist of 11 subframes with full system BW CRS followed by 9 subframes with CRS only on the center 6 PRBs.

- Choose 16QAM with R.11 with only 41PRB allocated and Es/Noc=10dB on the interfering cell for defining the PDSCH demodulation performance requirement for nw-based CRS-IM.

# References

RP-181315, New LTE WI on UE requirements for network-based CRS mitigation, Ericsson, Jun, 2018

R4-1813715, Way forward on simulation assumption for demodulation performance requirements for network-based CRS interference mitigation

R4-1815715, Summary of alignment and impairment results of for network-based CRS interference mitigation

R4-1815714, CR for demodulation performance requirements for network-based CRS interference mitigation, Ericsson