3GPP TSG-RAN WG4 Meeting #89 	R4-1815760

# Spokane, Washington, USA, 12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Phase 2-21B FR2-SA interruptions at UL carrier RRC reconfiguration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No test case FR2-SA interruptions at UL carrier RRC reconfiguration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case FR2-SA interruptions at UL carrier RRC reconfiguration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case FR2-SA interruptions at UL carrier RRC reconfiguration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.5.2 New sections: A.7.5.2.1, A.7.5.2.1.1, A.7.5.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### A.7.5.2	Interruption

#### A.7.5.2.1	Interruptions at UL carrier RRC reconfiguration

##### A.7.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the interruption, which occurs immediately after the RRC reconfiguration procedure [2], on PCell and all activated SCells within the same FR as the reconfigured UL carrier is within the specified limits, when the UL carrier or supplementary UL carrier is configured or deconfigured at the UE operating in standalone (SA) operation mode with PCell in FR2. The test will verify the interruption requirements specified in Section 8.2.2.2.4.

There are two cells: Cell 1 and Cell 2. Cell 1 is FR2 PCell on PCC, which is continuously scheduled in DL and UL during the whole test duration. Cell 2 is an activated FR2 SCell on SCC. The test consists of two tests, each with three time periods T1, T2, and T3. The resources for UL transmission are assigned to the UE prior to the start of time period T2.

In Test 1, during time period T1 there is an UL carrier in Cell 2, during time period T2 a supplementary UL carrier is configured for Cell 2 through RRCReconfiguration, and during time period T3 the supplementary UL carrier is released through RRCReconfiguration.

In Test 2, during time period T1 there is a supplementray UL carrier in Cell 2, during time period T2 an UL carrier is configured for Cell 2 through RRCReconfiguration, and during time duration T3 the UL carrier is released through RRCReconfiguration.

In both Test 1 and Test 2, the UE is configured to report periodic PHR for all the configured UL carriers.

Editor’s note: PHR reporting configuration is FFS.

The test parameters are given in Tables A.7.5.2.1.1-1, A.7.5.2.1.1-2, A.7.5.2.1.1-3, and A.7.5.2.1.1-4.

Table A.7.5.2.1.1-1: Test configurations

| Config | Description |  |
| --- | --- | --- |
|  | PCell (Cell 1) | SCell (Cell 2) |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.7.5.2.1.1-2: General test parameters

| Parameter | Unit | Test Configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Frequency range |  | Conf 1, 2, 3, 4 | FR2 |  |
| RF Channel Numbers |  | Conf 1, 2, 3, 4 | 1 and 2 | Two radio channels in FR2 are used in each of Test 1 and Test 2. RF Channel 1 is for Cell 1, and RF Channel 2 is for Cell 2 |
| Active cells |  | Conf 1, 2, 3, 4 | Cell 1 and Cell 2 | Cell 1 is the PCell on RF channel number 1. Cell 2 is configured and activated secondary cell on RF channel number 2. |
| CP length |  | Conf 1, 2, 3, 4 | Normal |  |
| DRX |  | Conf 1, 2, 3, 4 | OFF | Continuous monitoring of PCell |
| Measurement gap pattern Id |  | Conf 1, 2, 3, 4 | OFF | No measurement gaps are configured |
| Filter coefficient |  | Conf 1, 2, 3, 4 | 0 | L3 filtering is not used |
| T1 | s | Conf 1, 2, 3, 4 | 5 |  |
| T2 | s | Conf 1, 2, 3, 4 | 5 |  |
| T3 | s | Conf 1, 2, 3, 4 | 5 |  |

Table A.7.5.2.1.1-3: Cell specific test parameters for Cell 1

| Parameter | Unit | Test Configuration | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4 | 1 |  |  |  | 1 |  |  |
| PUSCH parameters |  | Conf 1, 2, 3, 4 | TBD | TBD | TBD |  | TBD | TBD | TBD |
| PUCCH parameters |  | Conf 1, 2, 3, 4 | TBD | TBD | TBD |  | TBD | TBD | TBD |
| PDSCH reference measurement channel |  | Conf 1, 2 | SR.3.1 TDD |  |  |  | SR.3.1 TDD |  |  |
|  |  | Conf 3, 4 | TBD |  |  |  | TBD |  |  |
| RMSI CORESET reference channel |  | Conf 1, 2 | CR.3.1 TDD |  |  |  | CR.3.1 TDD |  |  |
|  |  | Conf 3, 4 | TBD |  |  |  | TBD |  |  |
| RMC CORESET reference channel |  | Conf 1, 2 | CCR.3.1 TDD |  |  |  | CCR.3.1 TDD |  |  |
|  |  | Conf 3, 4 | TBD |  |  |  | TBD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3, 4 | OP.1 |  |  |  | OP.1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4 | SMTC.1 |  |  |  | SMTC.1 |  |  |
| SSB configuration |  | Conf 1, 2 | SSB.3 FR2 |  |  |  | SSB.3 FR2 |  |  |
|  |  | Conf 3, 4 | SSB.4 FR2 |  |  |  | SSB.4 FR2 |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4 | DLBWP.0 |  |  |  | DLBWP.0 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4 | DLBWP.1 |  |  |  | DLBWP.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4 | ULBWP.1 |  |  |  | ULBWP.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4 | -102 |  |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1, 2 | -102 |  |  |  | -102 |  |  |
|  |  | Conf 3, 4 | -99 |  |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1, 2 | -86 | -86 |  | -86 | -86 | -86 | -86 |
|  |  | Conf 3, 4 | -83 | -83 |  | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ Ch BW | Conf 1, 2, 3, 4 | TBD | TBD |  | TBD | TBD | TBD | TBD |
| Propagation Condition |  | Conf 1, 2, 3, 4 | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4 | 1 x 2 |  |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table A.7.5.2.1.1-4: Cell specific test parameters for Cell 2

| Parameter | Unit | Test Configuration | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4 | 2 |  |  |  | 2 |  |  |
| PUSCH parameters |  | Conf 1, 2, 3, 4 | TBD | TBD | TBD |  | N/A | TBD | TBD |
| PUCCH parameters |  | Conf 1, 2, 3, 4 | TBD | TBD | TBD |  | N/A | TBD | TBD |
| PUSCH parameters for supplementary UL |  | Conf 1, 2, 3, 4 | N/A | TBD | TBD |  | TBD | TBD | TBD |
| PUCCH parameters for supplementary UL |  | Conf 1, 2, 3, 4 | N/A | TBD | TBD |  | TBD | TBD | TBD |
| PDSCH reference measurement channel |  | Conf 1, 3 | SR.3.1 TDD |  |  |  | SR.3.1 TDD |  |  |
|  |  | Conf 2, 4 | TBD |  |  |  | TBD |  |  |
| RMSI CORESET reference channel |  | Conf 1, 3 | CR.3.1 TDD |  |  |  | CR.3.1 TDD |  |  |
|  |  | Conf 2, 4 | TBD |  |  |  | TBD |  |  |
| RMC CORESET reference channel |  | Conf 1, 3 | CCR.3.1 TDD |  |  |  | CCR.3.1 TDD |  |  |
|  |  | Conf 2, 4 | TBD |  |  |  | TBD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3, 4 | OP.1 |  |  |  | OP.1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4 | SMTC.1 |  |  |  | SMTC.1 |  |  |
| SSB configuration |  | Conf 1, 3 | SSB.3 FR2 |  |  |  | SSB.3 FR2 |  |  |
|  |  | Conf 2, 4 | SSB.4 FR2 |  |  |  | SSB.4 FR2 |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4 | DLBWP.0 |  |  |  | DLBWP.0 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4 | DLBWP.1 |  |  |  | DLBWP.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4 | ULBWP.1 |  |  |  | ULBWP.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4 | -102 |  |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1, 3 | -102 |  |  |  | -102 |  |  |
|  |  | Conf 2, 4 | -99 |  |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1, 3 | -86 | -86 |  | -86 | -86 | -86 | -86 |
|  |  | Conf 2, 4 | -83 | -83 |  | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ CH BW | Conf 1, 2, 3, 4 | TBD | TBD |  | TBD | TBD | TBD | TBD |
| Propagation Condition |  | Conf 1, 2, 3, 4 | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4 | 1 x 2 |  |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.7.5.2.1.2	Test Requirements

The UE shall be continuously scheduled in PCell during the entire test. During the enture length of time period T2, at least 99% of all expected ACK/NACKs shall be transmitted on PCell by the UE. During the enture length of time period T3, at least 99% of all expected ACK/NACKs shall be transmitted on PCell by the UE.

Each interruption shall not exceed [1] slot immediately after the RRC reconfiguration procedure [2]. The interruption is for both uplink and downlink of PCell and all the activated SCells within the same FR as the configured or de-configured UL.

The rate of correct events observed during repeated tests shall be at least 90%.