3GPP TSG-RAN WG4 Meeting #89 	R4-1815767

# Spokane, Washington, USA, 12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates in bands grouping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SDL bands need to be accounted for in bands grouping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Groups for SDL bands are added in bands grouping |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SDL bands missing in bands grouping and all requirements in 38.133 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## --- start of change 1 ---

## 3.5	Frequency bands grouping

### 3.5.1	Introduction

The intention with the frequency band grouping below is to increase the readability of the specification.

The frequency bands grouping is derived based on UE REFSENS requirements specified in [18, 19, 20] and assuming 0.5 dB step between the neighbour groups. The groups are defined in the order of increasing REFSENS, i.e., the group A has the smallest REFSENS among the groups. For the same SCS and a given bandwidth, the bands within the same group have the same Io conditions in a corresponding requirement in this specification, provided the bands support this SCS. For different SCSs supported by a frequency band and the same bandwidth, different Io conditions may apply for the frequency band in the requirements, while the band group is the same, based on the lowest REFSENS requirement normalized by the number of subcarriers among its supported SCSs for this bandwidth. For the same SCS but different supported bandwidths, the group for a band is determined based on the lowest REFSENS requirement normalized by the number of subcarriers among its supported bandwidths.

### 3.5.2	NR operating bands in FR1

NR frequency bands grouping for FR1 is specified in Table 3.5.2-1.

Table 3.5.2-1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation |  | Operating bands |
| A | NR_FDD_FR1_A | n1, n70, n744 | NR_TDD_FR1_A | n34, n38, n39, n40, n50, n51 | NR_SDL_FR1_A |  | n75, n76 |
| B | NR_FDD_FR1_B | n66, n743 | NR_TDD_FR1_B | - | NR_SDL_FR1_B | - |  |
| C | NR_FDD_FR1_C | - | NR_TDD_FR1_C | n771, n78, n79 | NR_SDL_FR1_C | - |  |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - |  |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41 | NR_SDL_FR1_E | - |  |
| F | NR_FDD_FR1_F | - | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - |  |
| G | NR_FDD_FR1_G | n3, n8, n12, n20, n71 | NR_TDD_FR1_G | - | NR_SDL_FR1_G | - |  |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - |  |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. |  |  |  |  |  |  |  |

### 3.5.3	NR operating bands in FR2

NR frequency bands grouping for FR2 is specified in Table 3.5.3-1.

Table 3.5.3-1: NR frequency band groups for FR2

| Group | Band group notation | Operating bands |
| --- | --- | --- |
| A | NR_TDD_FR2_A | n2571, n2581, n2611 |
| B | NR_TDD_FR2_B | n2574, n2584, n2614 |
| C | NR_TDD_FR2_C |  |
| D | NR_TDD_FR2_D |  |
| E | NR_TDD_FR2_E |  |
| F | NR_TDD_FR2_F | n2604 |
| G | NR_TDD_FR2_G | n2572, n2582, n2601, n2612 |
| H | NR_TDD_FR2_H |  |
| I | NR_TDD_FR2_I |  |
| J | NR_TDD_FR2_J |  |
| K | NR_TDD_FR2_K |  |
| L | NR_TDD_FR2_L |  |
| M | NR_TDD_FR2_M |  |
| N | NR_TDD_FR2_N |  |
| O | NR_TDD_FR2_O |  |
| P | NR_TDD_FR2_P |  |
| Q | NR_TDD_FR2_Q |  |
| R | NR_TDD_FR2_R |  |
| S | NR_TDD_FR2_S |  |
| T | NR_TDD_FR2_T | n2573, n2583, n2613 |
| U | NR_TDD_FR2_U |  |
| V | NR_TDD_FR2_V |  |
| W | NR_TDD_FR2_W |  |
| X | NR_TDD_FR2_X |  |
| Y | NR_TDD_FR2_Y | n2603 |
| NOTE 1: UE power class 1. NOTE 2: UE power class 2. NOTE 3: UE power class 3. NOTE 4: UE power class 4. |  |  |

--- end of change 1 ---

## --- start of change 2 ---

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

BWP	Bandwidth Part

CA	Carrier Aggregation

CC	Component Carrier

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

DC	Dual Connectivity

DL	Downlink

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FR	Frequency Range

HARQ	Hybrid Automatic Repeat Request

HO	Handover

MAC	Medium Access Control

MCG	Master Cell Group

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

NR	New Radio

NSA	Non-Standalone operation mode

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

PBCH	Physical Broadcast Channel

PCell	Primary Cell

PLMN	Public Land Mobile Network

PRACH	Physical RACH

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RRC	Radio Resource Control

RRM	Radio Resource Management

RSSI	Received Signal Strength Indicator

SA	Standalone operation mode

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SFN	System Frame Number

SI	System Information

SIB	System Information Block

SMTC	SSB-based Measurement Timing configuration

SRS	Sounding Reference Signal

SS	Synchronization Signal

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

TA	Timing Advance

TAG	Timing Advance Group

TDD	Time Division Duplex

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

## --- end of change 2 ---