3GPP TSG-RAN WG4 Meeting #89	R4-1815782

Spokane, US, 12 - 16 November 2018

Agenda item:	7.13.1.4

Title: 	Discussion on NR performance of CQI test under static and fading

Source: 	Huawei, Hisilicon

Document for:	Discussion

Introduction

Base on the previous agreements and the way forward of R4-1811693, companies are suggested to provide simulation results under the assumptions below.

In this contribution, we provide our simulation results of CQI test under static and fading.

Initial Simulation assumption

For CQI (Static)

Table 1 lists the common parameters for the CQI (static) test cases.

Table 1 CQI static Test parameters

| Parameter | Unit | Static CQI test (FR1 FDD) | Static CQI test (FR1 TDD) | Static CQI test (FR2) |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | 7D1S2U | DDDSU |
| Propagation channel |  | AWGN | AWGN | AWGN |
| Correlation and antenna configuration |  | 2*2, 2*4 with static channel in TS36.101 Annex B | 2*2, 2*4 with static channel in TS36.101 Annex B | 2Rx 2*2 with static channel in TS36.101 Annex B |
| CSI-RS resource setting (periodic) |  |  |  |  |
| resourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | 5slot 1 slot offset | 5slot, 1 slot offset | 5slot, 1 slot offset |
| NZP CSI-RS ports |  | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (periodic) |  |  |  |  |
| reportConfigType |  | periodic | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| Codebook Restriction |  | 010000 (fixed rank2) | 010000 (fixed rank2) | 010000 (fixed rank2) |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

For CQI (Fading)

Table 2 CQI fading Test parameters

| Parameter | Unit | Fading CQI test (FR1 FDD) | Fading CQI test (FR1 TDD) | Fading CQI test (FR2) |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | 7D1S2U | DDDSU |
| Propagation channel |  | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | TDL_A 30ns, 75Hz |
| Correlation and antenna configuration |  | 2Rx 2*2 4Rx: 2*4 [ULA High] for 2Rx [XP High] for 4Rx | 2Rx 2*2 4Rx: 2*4 [ULA High] for 2Rx [XP High ]for 4Rx | 2Rx 2*2 [ULA High] |
| CSI-RS resource setting (periodic) |  |  |  |  |
| resourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | 5slot 1 slot offset | 5slot, 1 slot offset | 5slot, 1 slot offset |
| NZP CSI-RS ports |  | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (periodic) |  |  |  |  |
| reportConfigType |  | periodic | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| Codebook Restriction |  | 000001 | 000001 | 000001 |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Simulation results

TBD

Conclusion

In this contribution, we provide our simulation results of CQI test under static and fading.

Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018