3GPP TSG-RAN WG4 Meeting #89	R4-1815804

Spokane, USA, 12 – 16 November 2018

Source:	Ericsson, Verizon

Title:	TP for TR 38.716-01-01: n261 UL

Agenda item:			9.1.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.716-01-01 to add fallbacks needed for n261 UL configurations CA_n261I_UL_n261H and CA_n261M_UL_n261H defined in the TP [1] for NR Intra-band.

# 2. Text Proposal

# ---Start of changes---

Table 5.5A.1-2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Aggregated BW (MHz) | BCS | Fallback group |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |  |
| CA_n261G | CA _n261G | 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n261H | CA _n261H | 100 | 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n261I | CA _n261H | 100 | 100 | 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| CA_n261J | CA _n261H | 100 | 100 | 100 | 100 | 50, 100 |  |  |  | 500 | 0 |  |
| CA_n261K | CA _n261H | 100 | 100 | 100 | 100 | 100 | 50, 100 |  |  | 600 | 0 |  |
| CA_n261L | CA _n261H | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 |  | 700 | 0 |  |
| CA_n261M | CA _n261H | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 | 800 | 0 |  |
| NOTE 1: The maximum bandwidth of band n261 is 850MHz |  |  |  |  |  |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1]		RP-1812351, “TP for TR 38.716-01-01 NR Intra-band n260 CA”, Verizon