3GPP TSG-RAN WG4 Meeting #89	R4-1815807

# Spokane, USA, 12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.863-01-01 | CR | 0006 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to correct n260 BW class. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correcting BW classes in the n260 part of the following EN-DC combinations: 2A_n260 5A_n260 12A_n260 30A_n260 66A_n260 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting BW classes n260 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete BW class n260 in EN-DC combinations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.85, 6.89, 6.98, 6.106, 6.111 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... 38.101-3 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

## 6.85	DC_66A_n260

### 6.85.1	Operating bands for DC

Table 6.85.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n260 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.85.2	Channel bandwidths per operating band for DC

Table 6.85.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  | Fallback Group |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 (220 for Band n260 with 60kHz SCS) |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |
| DC_66A_n260(2A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 (420 for Band n260 with 60 kHz SCS) |  |  |
|  | n260 | 60 | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 [4] |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n260A(3A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 1220 (620 for Band n260 with 60 kHz SCS) |  |  |
|  | n260 | 60 | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 [4] |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n260A(4A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 1620 (820 for Band n260 with 60 kHz SCS) |  |  |
|  | n260 | 60 | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 [4] |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-n260B | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 1 |
|  | n260 | 60 | See CA_n260B Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 820 |  |  |  |
| DC_66A-n260C | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260C Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 1220 |  |  |  |
| DC_66A-n260D | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  | 2 |
|  | n260 | 60 | See CA_n260D Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 420 |  |  |  |
| DC_66A-n260E | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260E Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 620 |  |  |  |
| DC_66A-n260F | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260F Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 820 |  |  |  |
| DC_66A-n260G | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |  | 3 |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 220 |  |  |  |
| DC_66A-n260H | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 320 |  |  |  |
| DC_66A-n260I | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 420 |  |  |  |
| DC_66A-n260J | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 520 |  |  |  |
| DC_66A-n260K | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 620 |  |  |  |
| DC_66A-n260L | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 720 |  |  |  |
| DC_66A-n260M | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 820 |  |  |  |
| DC_66A-n260O | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |  | 4 |
|  | n260 | 60 | See CA_n260O Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 220 |  |  |  |
| DC_66A-n260P | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |  |
|  | n260 | 60 | See CA_n260P Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 320 |  |  |  |
| DC_66A-n260Q | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |  |
|  | n260 | 60 | See CA_n260Q Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  | 420 |  |  |  |

Table  6.85.2-2: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] |  | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A-n260(2A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 420 |  |  |
|  | n260 | 60 | See CA_n260(2A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(3A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260(3A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 1220 |  |  |
| DC_66A-n260(4A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(4A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 1620 |  |  |
| DC_66A-n260(D-G) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260(D-G) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |
| DC_66A-n260(D-H) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 720 |  |  |
|  | n260 | 60 | See CA_n260(D-H) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 720 |  |  |
| DC_66A-n260(D-I) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(D-I) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(D-O) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260(D-O) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |
| DC_66A-n260(D-P) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 720 |  |  |
|  | n260 | 60 | See CA_n260(D-P) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 720 |  |  |
| DC_66A-n260(D-Q) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(D-Q) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(E-O) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(E-O) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(E-P) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 920 |  |  |
|  | n260 | 60 | See CA_n260(E-P) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 920 |  |  |
| DC_66A-n260(E-Q) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1020 |  |  |
|  | n260 | 60 | See CA_n260(E-Q) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 1020 |  |  |
| DC_66A-n260(A-I) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(G-I) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |

### 6.85.3	Co-existence studies

Table 6.85.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 66+Band n260 2UL DC.

Table 6.85.3-1: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 111000 | 120000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 148000 | 160000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 185000 | 200000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 222000 | 240000 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 259000 | 280000 |
| 2nd order IMD products | ¦f2_low–f1_high¦ |  | ¦f2_high–f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 35220 |  | 38290 | 38710 | 41780 |
| 3rd order IMD products | ¦f2_low–2*f1_high¦ |  | ¦f2_high–2*f1_low¦ | ¦2*f2_low–f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 33440 |  | 36580 | 72220 | 78290 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ |  | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 40420 |  | 43560 | 75710 | 81780 |
| 4rd order IMD products | ¦3*f1_low – f2_high¦ |  | ¦3*f1_high – f2_low¦ | ¦3*f2_low–f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limits (MHz) | 31660 |  | 34870 | 109220 | 118290 |
| 4rd order IMD products | ¦3*f1_low +f2_low¦ |  | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limits (MHz) | 42130 |  | 45340 | 112710 | 121780 |
| 4rd order IMD products | ¦2*f1_low-2*f2_high ¦ |  | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low+2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limits (MHz) | 70440 |  | 76580 | 77420 | 83560 |
| 5rd order IMD products | ¦f1_low –4*f2_high ¦ |  | ¦f1_high –4*f2_low¦ | ¦f2_low–4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limits (MHz) | 146220 |  | 158290 | 29880 | 33160 |
| 5rd order IMD products | ¦f1_low +4*f2_low¦ |  | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limits (MHz) | 149710 |  | 161780 | 43840 | 47120 |
| 5rd order IMD products | ¦2*f1_low-3*f2_high¦ |  | ¦2*f1_high–3*f2_low¦ | ¦2*f2_low-3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limits (MHz) | 107440 |  | 116580 | 68660 | 74870 |
| 5rd order IMD products | ¦2*f1_low +3*f2_low¦ |  | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limits (MHz) | 114420 |  | 123560 | 79130 | 85340 |

Based on Table 6.85.3-1, it can be seen that

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48, n77, n78

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, n1, 2, n2, 3, n3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, n50, 51, n51, 65, 66, n66, 70, n70, 74, n74, 75, n75, 76, n76, n80, n84

Therefore it can be concluded that there is no harmonic interference and IMD product falling into their own Rx bands.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.85.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.85.3-2: Band 66 and Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 3rd harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | Yes | China | 3rd, 4th, 5th IMD |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.85.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.85.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n260A | Bands 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.85.4	∆TIB and ∆RIB values

For DC_66A_n260, the TIB,c and RIB,c values are given in the tables below.

Table 6.85.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A_n260 | 66 | 0 |
|  | n260 | 0 |

Table 6.85.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66A_n260 | 66 | 0 |
|  | n260 | 0 |

### 6.85.5	REFSENs requirements

There is no specific REFSENs requirement.

---Text omitted---

## 6.89	DC_5A-n260

### 6.89.1	Operating bands for DC

Table 6.89.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-n260 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.89.2	Channel bandwidths per operating band for DC

Table 6.89.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  | Fallback Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-n260A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 210 |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 410 |  |  |
| DC_5A-n260B | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | n260 | 60 | See NR CA_n260B Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260C | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See NR CA_n260C Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 1210 |  |  |
| DC_5A-n260D | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  | 2 |
|  | n260 | 60 | See NR CA_n260D Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |
| DC_5A-n260E | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n260 | 60 | See NR CA_n260E Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
| DC_5A-n260F | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n260 | 60 | See NR CA_n260F Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260G | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |  | 3 |
|  | n260 | 60 | See NR CA_n260G Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 210 |  |  |
| DC_5A-n260H | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |  |  |
|  | n260 | 60 | See NR CA_n260H Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 310 |  |  |
| DC_5A-n260I | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  |  |
|  | n260 | 60 | See NR CA_n260I Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |
| DC_5A-n260J | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |  |  |
|  | n260 | 60 | See NR CA_n260J Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 510 |  |  |
| DC_5A-n260K | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n260 | 60 | See NR CA_n260K Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
| DC_5A-n260L | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |  |  |
|  | n260 | 60 | See NR CA_n260L Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 710 |  |  |
| DC_5A-n260M | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n260 | 60 | See NR CA_n260M Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260O | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |  | 4 |
|  | n260 | 60 | See NR CA_n260O Bandwidth Combination Fall-back Group 4 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 210 |  |  |
| DC_5A-n260P | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |  |  |
|  | n260 | 60 | See NR CA_n260P Bandwidth Combination Fall-back Group 4 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 310 |  |  |
| DC_5A-n260Q | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  |  |
|  | n260 | 60 | See NR CA_n260Q Bandwidth Combination Fall-back Group 4 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |

Table 6.89.2-2: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz |  | [40 MHz] | 50 MHz |  | [60 MHz] |  |  | 80 MHz | 100 MHz |  | 200 MHz | 400 MHz |  | Maximum aggregated bandwidth [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-n260(2A) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 410 |  |
|  | n260 | 60 | See NR CA_n260 (2A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
| DC_5A-n260(3A) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
|  | n260 | 60 | See NR CA_n260(3A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1210 |  |
| DC_5A-n260(4A) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  | n260 | 60 | See NR CA_n260(4A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1610 |  |
| DC_5A-n260(D-G) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
|  | n260 | 60 | See CA_n260(D-G) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
| DC_5A-n260(D-H) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |
|  | n260 | 60 | See CA_n260(D-H) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |
| DC_5A-n260(D-I) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  | n260 | 60 | See CA_n260(D-I) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
| DC_5A-n260(D-O) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
|  | n260 | 60 | See NR CA_n260(D-O) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
| DC_5A-n260(D-P) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |
|  | n260 | 60 | See NR CA_ n260(D-P) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |
| DC_5A-n260(D-Q) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  | n260 | 60 | See NR CA_ n260(D-Q) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
| DC_5A-n260(E-O) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See NR CA_n260(E-O) Bandwidth Combination in Table 8.9.2-22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
| DC_5A-n260(E-P) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See NR CA_n260(E-P) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 910 |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 910 |  |
| DC_5A-n260(E-Q) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See NR CA_n260(E-Q) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 910 |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 910 |  |
| DC_5A-n260(A-I) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
| DC_5A-n260(G-I) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |

### 6.89.3	Co-existence studies

Table 6.89.3-1 lists Band 5A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.89.3-1: Band 5 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39176 | 36151 | 37824 | 40849 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38352 | 35302 | 73151 | 79176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38648 | 41698 | 74824 | 80849 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 36990 | 40010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78352 | 72302 | 75648 | 81698 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37528 | 34453 | 110151 | 119176 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39472 | 42547 | 111824 | 120849 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159176 | 147151 | 33604 | 36704 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148824 | 160849 | 40296 | 43396 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118352 | 109302 | 71453 | 77528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112648 | 121698 | 76472 | 82547 |

Based on Table 6.89.3-1

-	3rd order harmonic may fall into frequencies of band 41, n41

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into frequencies of band 5, n5,6, 8, n8, 12, n12, 13, n13, 14, 17, 18, 19, 20, n20, 26, 27, 28, n28, 29, 31, 44, 67, 68, 71, n71, 72, 73, n81, n82 and n83

-	4th order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.89.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.89.3-2: 2UL Band 5 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.89.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.89.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.89.4	∆TIB and ∆RIB values

For DC_5A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-n260A | 5 | 0 |
|  | n260 | 0 |

Table 6.89.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-n260A | 5 | 0 |
|  | n260 | 0 |

### 6.89.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

---Text omitted---

## 6.98	DC_2A_n260A and DC_2A_n260(2A)

### 6.98.1	Operating bands for DC

Table 6.98.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A_n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.98.2	Channel bandwidths per operating band for DC

Table 6.98.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A_n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 (220 for Band n260 with 60kHz SCS) |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_2A_n260(2A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 (420 for Band n260 with 60 kHz SCS) |
|  | n260 | 60 | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 [4] |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260G | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260H | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260I | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260J | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260K | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260L | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260M | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260(A-I) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260(G-I) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.98.3	Co-existence studies

Table 6.98.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 2+Band n260 2UL DC.

Table 6.98.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 |  | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 |  | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 |  | 5730 | 111000 | 120000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 |  | 148000 | 160000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 |  | 9550 | 185000 | 200000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 11100 |  | 11460 | 222000 | 240000 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 12950 |  | 13370 | 259000 | 280000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 |  | 35090 | 38850 | 41910 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 |  | 33180 | 72090 | 78150 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 |  | 43820 | 75850 | 81910 |
| 4rd order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 |  | 70180 | 77700 | 83820 |
| 4rd order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 |  | 31270 | 109090 | 118150 |
| 4rd order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 |  | 45730 | 112850 | 121910 |
| 5rd order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 |  | 146090 | 29360 | 32600 |
| 5rd order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 |  | 161910 | 44400 | 47640 |
| 5rd order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 |  | 107180 | 68270 | 74450 |
| 5rd order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 |  | 123820 | 79550 | 85730 |

Based on Table 6.98.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 43, 48, 49, n77 and n78.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD may fall into own Rx frequencies of band n260 (which can be disregarded, since no self-interference in TDD).

-	4th order IMD may fall inteo RX frequency of band n259.

-	5th order IMD may fall inteo RX frequency of bands n257 or n259.

Therefore, it can be concluded that there is no harmonic interference and IMD product falling into their own Rx bands.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.98.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.98.3-2: Band 2 and Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | 3rd harmonic |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | Yes | China | 3rd, 4th, 5th IMD |
|  | 47200 | - | 48400 | Yes | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.98.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.98.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_2A_n260A | Bands 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 42, 48, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.98.4	∆TIB and ∆RIB values

For DC_2A_n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.98.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A_n260A | 2 | 0 |
|  | n260 | 0 |

Table 6.98.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A_n260A | 2 | 0 |
|  | n260 | 0 |

### 6.98.5	REFSENS requirements

There is no specific REFSENS requirement.

---Text omitted---

## 6.106	DC_12A_n260A

### 6.106.1	Operating bands for DC

Table 6.106.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12A_n260A | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.106.2	Channel bandwidths per operating band for DC

Table 6.106.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n260A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 (210 for Band n260 with 60 kHz SCS) |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_12A_n260G | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260H | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260I | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260J | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260K | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260L | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260M | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260(A-I) | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260(G-I) | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.106.3	Co-existence studies

Table 6.106.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 12+Band n260 2UL DC.

Table 6.106.3-1: Band 12 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 619 | 716 |  | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 |  | 1432 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 |  | 2148 | 111000 | 120000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 |  | 148000 | 160000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3095 |  | 3580 | 185000 | 200000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 3714 |  | 4296 | 222000 | 240000 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4333 |  | 5012 | 259000 | 280000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39301 |  | 36284 | 37699 | 40716 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38602 |  | 35568 | 73284 | 79301 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38398 |  | 41432 | 74699 | 80716 |
| 4rd order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78602 |  | 72568 | 75398 | 81432 |
| 4rd order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37903 |  | 34852 | 110284 | 119301 |
| 4rd order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39097 |  | 42148 | 111699 | 120716 |
| 5rd order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159301 |  | 147284 | 34136 | 37204 |
| 5rd order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148699 |  | 160716 | 39796 | 42864 |
| 5rd order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118602 |  | 109568 | 71852 | 77903 |
| 5rd order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112398 |  | 121432 | 76097 | 82148 |

Based on Table 6.106.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 50, 51, n75 and n76.

-	3rd order harmonic may fall into Rx frequencies of band 1, 4, 10, 65 or 66.

-	2nd 3rd 4th and 5th order IMD may fall into own Rx frequencies of band n260 (which can be disregarded, since no self-interference in TDD).

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.106.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.106.3-2: Band 12 and Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.106.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.106.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_12A_n260A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 41, 48, 71 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | E-UTRA Band 4, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.106.4	∆TIB and ∆RIB values

For DC_12A_n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.106.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12A_n260A | 12 | 0 |
|  | n260 | 0 |

Table 6.106.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12A_n260A | 12 | 0 |
|  | n260 | 0 |

### 6.106.5	REFSENS requirements

No IMD issues are expected for DC configuration between the LTE bands below 6 GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency in either the LTE band or the NR band.

---Text omitted---

## 6.111	DC_30A_n260A

### 6.111.1	Operating bands for DC

Table 6.111.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_30A_n260A | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.111.2	Channel bandwidths per operating band for DC

Table 6.111.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_30A_n260A | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 (210 for Band n260 with 60 kHz SCS) |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_30A_n260G | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260H | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260I | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260J | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260K | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260L | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260M | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260(A-I) | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260(G-I) | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.111.3	Co-existence studies

Table 6.111.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 30+Band n260 2UL DC.

Table 6.111.3-1: Band 30 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 |  | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4610 |  | 4630 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6915 |  | 6945 | 111000 | 120000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9220 | 9260 |  | 148000 | 160000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11525 |  | 11575 | 185000 | 200000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 13830 |  | 13890 | 222000 | 240000 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 16135 |  | 16205 | 259000 | 280000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 37695 |  | 34685 | 39305 | 42315 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 35390 |  | 32370 | 71685 | 77695 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 41610 |  | 44630 | 76305 | 82315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 75390 |  | 69370 | 78610 | 84630 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 33085 |  | 30055 | 108685 | 117695 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 43915 |  | 46945 | 113305 | 122315 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 157695 |  | 145685 | 27740 | 30780 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 150305 |  | 162315 | 46220 | 49260 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 115390 |  | 106370 | 67055 | 73085 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 115610 |  | 124630 | 80915 | 86945 |

Based on Table 6.111.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band n79.

-	2nd order IMD may fall into own Rx frequencies of band n260 (which can be disregarded, since no self-interference in TDD).

-	4th order IMD may fall into band n259.

-	5th order IMD may fall into band n257.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.111.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.111.3-2: Band 30 and Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | Yes | China | 3rd IMD |
|  | 47200 | - | 48400 | Yes | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.111.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.111.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_30A_n260A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.111.4	∆TIB and ∆RIB values

For DC_30A_n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.111.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30A_n260A | 30 | 0 |
|  | n260 | 0 |

Table 6.111.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_30A_n260A | 30 | 0 |
|  | n260 | 0 |

### 6.111.5	REFSENS requirements

No IMD issues are expected for DC configuration between the LTE bands below 6 GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency in either the LTE band or the NR band.

---End of changes---