3GPP TSG-RAN WG4 Meeting #89	R4-1815813

Spokane, USA, 12 – 16 November 2018

Source:	Ericsson, AT&T

Title:	TP to TR 37.716-31-11: Addition of EN-DC configurations with n260

Agenda item:	9.5.3

Document for:	Approval

# Background

This contribution provides text proposal to TR 37.716-31-11 for addition of EN-DC n260 configurations as defined in [1].

# Text Proposal

##### ---Start of changes---

## 5.2.16	DC_2A-12A-30A_n260, DC_2A-2A-12A-30A_n260

### 5.2.16.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-12-30_n260 | CA_2-12-30 | n260 |  |

### 5.2.16.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A-30A_n260A DC_2A-12A-30A_n260G DC_2A-12A-30A_n260H DC_2A-12A-30A_n260I DC_2A-12A-30A_n260J DC_2A-12A-30A_n260K DC_2A-12A-30A_n260L DC_2A-12A-30A_n260M | DC_2A_n260A DC_12A_n260A DC_30A_n260A | CA_2A-12A-30A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-2A-12A-30A_n260A | DC_2A_n260A DC_12A_n260A DC_30A_n260A | CA_2A-2A-12A-30A | n260 |

### 5.2.16.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12-30_n260 | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 30 | 0.3 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12-30_n260 | 2 | 0.4 |
|  | 12 | 0 |
|  | 30 | 0.5 |
|  | n260 | 0 |

## 5.2.17	DC_2A-5A-30A_n260, DC_2A-2A-5A-30A_n260

### 5.2.17.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5-30_n260 | CA_2-5-30 | n260 |  |

### 5.2.17.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-30A_n260 DC_2A-5A-30A_n260G DC_2A-5A-30A_n260H DC_2A-5A-30A_n260I DC_2A-5A-30A_n260J DC_2A-5A-30A_n260K DC_2A-5A-30A_n260L DC_2A-5A-30A_n260M | DC_2A_n260A DC_5A_n260A DC_30A_n260A | CA_2A-5A-30A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-2A-5A-30A_n260 | DC_2A_n260A DC_5A_n260A DC_30A_n260A | CA_2A-2A-5A-30A | n260 |

### 5.2.17.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-30_n260 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-30_n260 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | n260 | 0 |

## 5.2.18	DC_2A-12A-66A_n260, DC_2A-2A-12A-66A_n260, DC_2A-12A-66A-66A_n260

### 5.2.18.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-12-66_n260 | CA_2-12-66 | n260 |  |

### 5.2.18.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A-66A_n260A DC_2A-12A-66A_n260G DC_2A-12A-66A_n260H DC_2A-12A-66A_n260I DC_2A-12A-66A_n260J DC_2A-12A-66A_n260K DC_2A-12A-66A_n260L DC_2A-12A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_66A_n260A | CA_2A-12A-66A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-2A-12A-66A_n260A | DC_2A_n260A DC_12A_n260A DC_66A_n260A | CA_2A-2A-12A-66A | n260 |
| DC_2A-12A-66A-66A_n260A | DC_2A_n260A DC_12A_n260A DC_66A_n260A | CA_2A-12A-66A-66A | n260 |

### 5.2.18.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12-66_n260 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12-66_n260 | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n260 | 0 |

## 5.2.19	DC_2A-5A-66A_n260, DC_2A-2A-5A-66A_n260, DC_2A-5A-66A-66A_n260

### 5.2.19.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5-66_n260 | CA_2-5-66 | n260 |  |

### 5.2.19.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-66A_n260A DC_2A-5A-66A_n260G DC_2A-5A-66A_n260H DC_2A-5A-66A_n260I DC_2A-5A-66A_n260J DC_2A-5A-66A_n260K DC_2A-5A-66A_n260L DC_2A-5A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_66A_n260A | CA_2A-5A-66A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-2A-5A-66A_n260A | DC_2A_n260A DC_5A_n260A DC_66A_n260A | CA_2A-2A-5A-66A | n260 |
| DC_2A-5A-66A-66A_n260A | DC_2A_n260A DC_5A_n260A DC_66A_n260A | CA_2A-5A-66A-66A | n260 |

### 5.2.19.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n260 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n260 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
|  | n260 | 0 |

## 5.2.20	DC_5A-30A-66A_n260, DC_5A-30A-66A-66A_n260

### 5.2.20.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-30-66_n260 | CA_5-30-66 | n260 |  |

### 5.2.20.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-30A-66A _n260A DC_5A-30A-66A _n260G DC_5A-30A-66A _n260H DC_5A-30A-66A _n260I DC_5A-30A-66A _n260J DC_5A-30A-66A _n260K DC_5A-30A-66A _n260L DC_5A-30A-66A _n260M | DC_5A_n260A DC_30A_n260A DC_66A_n260A | CA_5A-30A-66A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_5A-30A-66A-66A _n260A | DC_5A_n260A DC_30A_n260A DC_66A_n260A | CA_5A-30A-66A-66A | n260 |

### 5.2.20.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-30-66_n260 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-30-66_n260 | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n260 | 0 |

## 5.2.21	DC_2A-30A-66A_n260

### 5.2.21.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-30-66_n260 | CA_2-30-66 | n260 |  |

### 5.2.21.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-30A-66A _n260A DC_2A-30A-66A _n260G DC_2A-30A-66A _n260H DC_2A-30A-66A _n260I DC_2A-30A-66A _n260J DC_2A-30A-66A _n260K DC_2A-30A-66A _n260L DC_2A-30A-66A _n260M | DC_2A_n260A DC_30A_n260A DC_66A_n260A | CA_2A-30A-66A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-30A-66A-66A _n260A | DC_2A_n260A DC_30A_n260A DC_66A_n260A | CA_2A-30A-66A-66A | n260 |

### 5.2.21.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-30-66_n260 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-30-66_n260 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n260 | 0 |

## 5.2.22	DC_2A-29A-30A_n260, DC_2A-2A-29A-30A_n260

### 5.2.22.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-29-30_n260 | CA_2-29-30 | n260 |  |

### 5.2.22.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-29A-30A_n260A DC_2A-29A-30A_n260G DC_2A-29A-30A_n260H DC_2A-29A-30A_n260I DC_2A-29A-30A_n260J DC_2A-29A-30A_n260K DC_2A-29A-30A_n260L DC_2A-29A-30A_n260M | DC_2A_n260A DC_30A_n260A | CA_2A-29A-30A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-2A-29A-30A_n260A | DC_2A_n260A DC_30A_n260A | CA_2A-2A-29A-30A | n260 |

### 5.2.22.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-30_n260 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29-30_n260 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n260 | 0 |

## 5.2.23	DC_12A-30A-66A_n260, DC_12A-30A-66A-66A_n260

### 5.2.23.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_12-30-66_n260 | CA_12-30-66 | n260 |  |

### 5.2.23.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-30A-66A_n260A DC_12A-30A-66A_n260G DC_12A-30A-66A_n260H DC_12A-30A-66A_n260I DC_12A-30A-66A_n260J DC_12A-30A-66A_n260K DC_12A-30A-66A_n260L DC_12A-30A-66A_n260M | DC_12A_n260A DC_30A_n260A DC_66A_n260A | CA_12A-30A-66A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_12A-30A-66A-66A _n260A | DC_12A_n260A DC_30A_n260A DC_66A_n260A | CA_12A-30A-66A-66A | n260 |

### 5.2.23.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-30-66_n260 | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-30-66_n260 | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n260 | 0 |

##### ---End of changes---

# Reference

[1]	181550, “Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson