3GPP TSG-RAN WG4 Meeting #89	R4-1815821

Spokane, USA, 12 – 16 November 2018

Source:	Ericsson, Telstra

Title:	TP for TR 38.716-01-01: n258B – n258M

Agenda item:			9.1.3

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.716-01-01 to n258B - n258M defined in the TP [1] for NR Intra-band.

# 2. Text Proposal

---Start of changes---

## 7.x	CA_n258

### 7.x.1	Operating bands for CA

Table 7.x.1-1: Intra-band CA

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n258 | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for CA

Table 7.x.2-1: NR CA configurations, bandwidth combination sets and fallback group defined for intra-band contiguous CA

|  |  | NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated BW (MHz) | BCS | Fallback group |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |  |
| CA_n258B | - | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n258C | - | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  | 1200 | 0 |  |
| CA_n258D | - | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n258E | - | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n258F | - | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  | 800 | 0 |  |
| CA_n258G | - | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n258H | - | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n258I | - | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| CA_n258J | - | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  | 500 | 0 |  |
| CA_n258K | - | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 600 | 0 |  |
| CA_n258L | - | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  | 700 | 0 |  |
| CA_n258M | - | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 800 | 0 |  |

### 7.x.3	Co-existence studies

Table 7.x.3-1: Impact of UL and DL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge |
| n258 | 24250 | 27500 | 48500 | 55000 | 72750 | 82500 | 97000 | 110000 | 121250 | 137500 | 145500 | 165000 | 169750 | 192500 |

---End of changes---

# Reference

[1]		RP-181548, “Revised WID on Rel-16 LTE intra-band Carrier Aggregation for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y)”, Ericsson