3GPP RAN WG4 Meeting #89	R4-1815825

Spokane, WA, USA November 12-16, 2018

Agenda item:	7.6.14.1

Source:	Apple Inc.

Title:	EIS Spherical Coverage for FR2 UEs

Document for:	Approval

# Introduction

The progress of the NR Work Item [1] had been discussed during the 3GPP RAN #80 meeting.  It was the decision of TSG RAN to extend the core part of the WI.  During the 3GPP RAN #81 meeting the WI was further extended, with the remaining issues documented in [7]. This contribution examines the relevant background information and provides proposals to finalize the EIS spherical coverage requirement for PC3 UEs.

# Discussion

## Background

The peak EIS requirement for power class 3 UEs was agreed in RAN4 #86bis and the requirement captured in Section 7.3.2.3 of TS38.101-2 [2]. However, the EIS spherical coverage requirement has not yet been finalized by RAN4. Additionally, over the past few meetings, RAN4 has also discussed a beam correspondence requirement without a final outcome.

In previous RAN4 meetings, companies have addressed the EIS spherical coverage requirement and have argued that if a UE supports beam correspondence, EIS spherical coverage performance can be inferred from EIRP spherical CDF [3-4]. The discussions on EIS spherical coverage were therefore postponed till conclusion of beam correspondence and EIRP spherical coverage.

Discussions on the EIRP spherical coverage requirement for PC3 UEs concluded at RAN4#87 with agreement to define it at 50%-tile of EIRP CDF. The requirement is captured in Section 6.2.1.3 of TS38.101-2 [2]. In the same meeting, some companies proposed that the EIS spherical coverage requirement be defined analogously, i.e., the gap between 50%-tile of EIS CDF and minimum EIS is the same as the drop from the peak EIRP to the 50%-tile EIRP value [5].

In RAN#80, analysis in [6] identified a gap in applicability of the EIRP spherical coverage requirement to UEs supporting multiple FR2 bands as the requirement was defined for single band FR2 UEs. In RAN#81 this gap was recognized for the EIS spherical coverage and peak EIS requirements by including the corresponding objective in the exception sheet [7].

In RAN4#88 and RAN4#88bis, we presented our view on the EIS spherical coverage requirement for single band UEs and also UEs supporting multiple bands [15-16]. We explained that factors such as form-factor de-sense, extreme temperature conditions, and multi-band support have impacts on EIS peak and spherical coverage. We also proposed to define the spherical coverage requirement at 50%-tile of the EIS CDF. Finally, we suggested to apply the combined relaxations including the impacts of form-factor de-sense, extreme temperature conditions and multi-band support at 50%-tile of EIS CDF.

In this paper, we review and follow up the discussions on peak EIS and EIS spherical coverage and provide the technical justification to also extend the EIS spherical coverage requirements’ applicability to UEs supporting multiple FR2 bands.

## Analysis

As discussed in [3], EIS spherical coverage performance may be inferred from the EIRP CDF if the UE supports beam correspondence. Furthermore, direct testing of an EIS spherical performance requirement is time consuming as throughput vs. receive power needs to be measured across multiple test points.

The RF response of transmit and receive chains is inherently different [9]. However, for a UE that supports beam correspondence and the difference in transmit and receive responses is appropriately bounded, the EIRP spherical coverage provides a good approximation of the EIS spherical coverage performance. This eliminates the need for EIS spherical coverage testing. Consequently, we make the following observation and proposals.

Observation 1: EIS spherical coverage performance may be inferred from the EIRP CDF if a UE supports beam correspondence.

Proposal 1: A simplified test method for EIS spherical coverage is desired for UEs supporting beam correspondence and meeting the EIRP spherical coverage requirement.

In the event that RAN4 decides to define a spherical EIS requirement, we propose that it be defined at the 50%-tile CDF value, analogous to the agreements reached for EIRP spherical coverage.

Proposal 2: Define the EIS spherical coverage requirement in FR2 at 50%-tile value of its CDF.

In defining the EIS spherical coverage requirement, we have identified additional parameters, such as form factor de-sense, extreme temperature conditions, and supporting multiple bands, that can impact the EIS as discussed below. These factors should also be considered in deriving the EIS requirement in the beam peak direction. However, since RAN4 has already reached agreement on beam peak EIS we do not suggest revisiting this value.

Observation 2: Parameters such as form factor de-sense, extreme temperature conditions, and supporting multiple bands can have impact on EIS peak and spherical coverage.

Proposal 3: Consider impact of form factor de-sense and extreme temperature conditions in defining the spherical EIS requirement.

### Form Factor De-sense

As mentioned earlier, the requirement on beam peak EIS was agreed in RAN4#86bis. During the discussions, some companies expressed concern about the impact of form-factor de-sense on the beam peak EIS value [10]. The beam peak EIS requirement was however agreed without considering the impact of form factor de-sense [11]. Based on our analysis, we propose a 2dB relaxation to account for form factor de-sense.

Proposal 4: Incorporate a 2 dB relaxation for form factor de-sense in the EIS spherical coverage requirement.

### Extreme Thermal Condition

The UE in FR2 is required to work in extreme temperature conditions per Section E.2.1 in TS 38.101-2 [2].  There were however some discussions in RAN4#88bis on applicability of extreme thermal conditions to RF requirements in FR2 and it was agreed that companies to propose a list of RF requirements which need be tested due to expected different performance in extreme temperature conditions with technical justifications in next meeting [17].

As we mentioned in [16], the current REFSENS requirement is derived at 20C as it is using -174 dBm(kT) [14]. Therefore, the difference in the noise floor between 20C and either of two extreme temperatures i.e. -10C and +55C is about 0.5 dB using the equation below.

∆NF=10log10273.15+55273.15+20≈0.5dB

Observation 3: The REFSENS requirement is derived under normal temperature conditions.

We think it makes sense to test the EIS spherical coverage requirement under similar thermal conditions as REFSENS. We therefore propose to test the EIS spherical coverage under normal temperature conditions.

Proposal 5: Test the EIS spherical coverage under normal temperature conditions.

In case RAN4 decides to test the EIS spherical coverage under extreme thermal conditions, we propose a 0.5 dB noise floor increase to address the extreme temperature conditions.

Proposal 6: In case RAN4 decides to test the EIS spherical coverage under extreme thermal conditions, consider a 0.5 dB relaxation in the EIS spherical coverage requirement to address the noise floor increase at extreme temperature conditions.

### Multi-band Support

Analysis in [6] identified a gap in the applicability of the EIRP spherical coverage requirement to UEs supporting multiple FR2 bands as the requirement is only applicable to UEs which support single FR2 band. In RAN#81 this gap was recognized for EIS spherical coverage and peak EIS requirements by including the corresponding objective in the exception sheet [7].

We provided a comprehensive analysis describing how to extend the peak EIRP and EIRP spherical coverage requirements’ applicability to UEs supporting multiple FR2 bands in [12].

We note that the discussions on the relaxations of peak EIRP and EIRP spherical coverage requirements for UEs supporting multiple bands are still ongoing in RAN4. In RAN4#88bis meeting, it was however agreed that same relaxations for peak EIRP & peak EIS and same relaxations for 50%-tile CDF EIRP and 50%-tile CCDF EIS are defined. We can therefore propose to use the relaxation values we suggested in [12] for EIS spherical coverage requirement.

### Summary of EIS Spherical Coverage Requirement

Based on the technical analysis provided in the previous meetings in [12] and [16], we propose relaxations for the beam peak EIS and EIS spherical coverage requirement, labeled ΔTEMBP and ΔTEMBS, respectively, for PC3 UEs supporting multiple FR2 bands.

Observation 4: Supporting multi-band in FR2 requires relaxation for peak EIS, and EIS spherical coverage requirements for PC3 UEs.

In this paper, we explained the impact of form-factor de-sense, extreme temperature conditions on peak EIS and EIS spherical coverage for single and multi-band UEs. We however suggested to only consider impact of form-factor de-sense on the EIS spherical coverage and suggested to test the EIS spherical coverage requirement under normal temperature conditions. We also proposed to define the spherical coverage requirement at the 50%-tile of the EIS CCDF. Table 1 shows the proposes EIS spherical coverage limit considering relaxations due to form-factor de-sense.

Table 1: EIS Peak and Spherical Coverage for Single Band UEs

| Supported bands | Applicable to band | Form Factor De-sense (dB) | Extreme Temperature Condition Support (dB) | Peak EIS limit at 50 MHz BW (dBm) | EIS Spherical coverage limit at 50 MHz BW (dBm) |
| --- | --- | --- | --- | --- | --- |
| n257 | n257 | 2.0 | 0 | -88.3 | -75.4 |
| n258 | n258 |  |  | -88.3 | -75.4 |
| n260 | n260 |  |  | -85.7 | -71.1 |
| n261 | n261 |  |  | -88.3 | -75.4 |

Proposal 7: The EIS spherical coverage for single band UEs, as summarized in Table 1, is defined in the draft CR accompanying this contribution [13].

We suggested a combined relaxation including the impacts of form-factor de-sense, and multi-band support at the 50%-tile of EIS CCDF. These relaxations are indexed by the bands which the device supports and are applicable in a band-specific manner, as shown in Table 2 below.

Table 2: Summary of the multi-band framework for EIS peak and spherical coverage

| Supported bands | Applicable to band | Multi-band relaxation at 50%-tile (dB) | Multi-band relaxation at peak (dB) | Peak EIS limit at 50 MHz BW (dBm) | EIS Spherical coverage limit at 50 MHz BW (dBm) |
| --- | --- | --- | --- | --- | --- |
| n260, n261 | n260 | 1.6 | 0 | -85.7 | -69.5 |
| n260, n261 | n261 | 0.0 | 0.0 | -88.3 | -75.4 |
| n257, n258, n260, n261 | n257 | 1.9 | 2.1 | -86.2 | -73.5 |
| n257, n258, n260, n261 | n258 | 2.0 | 2.1 | -86.2 | -73.4 |
| n257, n258, n260, n261 | n260 | 1.8 | 1.5 | -84.2 | -69.3 |
| n257, n258, n260, n261 | n261 | 1.2 | 2.0 | -86.3 | -74.2 |

Proposal 8: The multi-band framework for EIS spherical coverage, as summarized in Table 2, is defined in the draft CR accompanying this contribution [13].

# Proposals (for approval)

In this paper, we explained the impact of form-factor de-sense, extreme temperature conditions, and multi-band support on EIS peak and spherical coverage. We also proposed to define the spherical coverage requirement at 50%-tile of the EIS CDF. We suggest to test the EIS spherical coverage requirement under normal temperature conditions and proposed to apply the combined relaxations including the impacts of form-factor de-sense, and multi-band support at 50%-tile of EIS CDF making the gap to the peak EIS larger compared to that of EIRP spherical coverage.

Based on the simulation results, the following observations and proposal are made:

Observation 1: EIS spherical coverage performance may be inferred from the EIRP CDF if a UE supports beam correspondence.

Observation 2: Parameters such as form factor de-sense, extreme temperature conditions, and supporting multiple bands can have impact on EIS peak and spherical coverage.

Observation 3: The REFSENS requirement is derived under normal temperature conditions.

Observation 4: Supporting multi-band in FR2 requires relaxation for peak EIS, and EIS spherical coverage requirements for PC3 UEs.

Proposal 1: A simplified test method for EIS spherical coverage is desired for UEs supporting beam correspondence and meeting the EIRP spherical coverage requirement.

Proposal 2: Define the EIS spherical coverage requirement in FR2 at 50%-tile value of its CDF.

Proposal 3: Consider impact of form factor de-sense and extreme temperature conditions in defining the spherical EIS requirement.

Proposal 4: Incorporate a 2 dB relaxation for form factor de-sense in the EIS spherical coverage requirement.

Proposal 5: Test the EIS spherical coverage under normal temperature conditions.

Proposal 6: In case RAN4 decides to test the EIS spherical coverage under extreme thermal conditions, consider a 0.5 dB relaxation in the EIS spherical coverage requirement to address the noise floor increase at extreme temperature conditions.

Proposal 7: The EIS spherical coverage for single band UEs, as summarized in Table 1, is defined in the draft CR accompanying this contribution [13].

Proposal 8: The multi-band framework for EIS spherical coverage, as summarized in Table 2, is defined in the draft CR accompanying this contribution [13].

# References

- RP-172834, “New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #78, December 2017

- TS38.101-2 v15.2.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” 3GPP, June 2018

- R4-1710897, “EIS versus EIRP spherical coverage”, MediaTek, 3GPP RAN4 #84bis, October 2017

- R4-1710607, “mmWave EIS requirement”, Huawei, 3GPP RAN4 #84bis, October 2017

- R4-1808198,	 “WF for spherical coverage for FR2”, Qualcomm Inc., 3GPP RAN4 #87, May 2018

- RP-180933, “Extending FR2 spherical coverage requirement to multi-band UEs,” Apple Inc., 3GPP RAN #80, June 2018

- RP-182083, “Rel-15 Work Item Exception for New Radio Access Technology – Core,” NTT DOCOMO, 3GPP RAN #81, September 2018

- R4-1707824, “Discussion on mmWave EIRP/EIS test”, Huawei, 3GPP RAN4 #84, August 2017

- R4-1805322,	“On UE Beam Correspondence and EIS spherical coverage”, Sony, 3GPP RAN4 #86bis, April 2018

- R4-1804125, “FR2 peak EIS requirement for handheld UE”, Intel Corporation, Apple Inc., 3GPP RAN4 #86bis, April 2018

- R4-1804589, “ MMW UE EIS“, Qualcomm Inc., 3GPP RAN4 #86bis, April 2018

- R4-1812390, “Further views on extending FR2 power class requirements to multi-band UEs”, Apple Inc., 3GPP RAN4 #88bis, October 2018

- R4-1815826, “Draft CR to TS 38.101-2: Introducing EIS spherical coverage requirements for PC3”, Apple Inc., 3GPP RAN4 #89, November 2018

- R4-1709140, “WF on REFSENS for mm-wave,” LG Electronics, Huawei, Sumitomo, Sony, Ericsson, Skyworks, Intel Corporation, KT, MediaTek, 3GPP RAN4 #84, August 2017

- R4-1810065, “On EIS Spherical Coverage for FR2 UEs”, Apple Inc, 3GPP RAN4 #88, August 2018

- R4-1812388, “Further Views On EIS Spherical Coverage for FR2 UEs”, Apple Inc, 3GPP RAN4 #88bis, October 2018

- R4-1814173, “WF on FR2 Temperature Conditions for UE RF Requirements Verification”, MediaTek, 3GPP RAN4 #88bis, October 2018

- R4-1814401, “RAN4#88-Bis Meeting Report”, ETSI MCC, RAN4#89, November 2018