3GPP TSG-RAN WG4 Meeting #89	R4-1815866

# Spokane, USA, 12th August – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-3 to introduce B41 intra-band EN-DC MPR for non NS04 cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Skyworks Solutions Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MPR for B41 intra-band EN-DC non NS04 cases is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | MPR equations are added in sections 6.2B.2.1 and 6.2B.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unknown UE behavior of release 15 UE in networks outside the US using Band 41 contiguous and non contiguous EN_DC combinations with UL configurations DC_(n)41AA and DC_41A_n41A. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2.1 and 6.2B.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ... 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### <Start of changes>

### 6.2B.2	UE maximum output power reduction and EN-DC

#### 6.2B.2.1	Intra-band contiguous EN-DC

##### 6.2B.2.1.2	MPR for DC_(n)41AA

6.2B.2.1.2.0	General

When the UE is configured for B41/n41 intra-band contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this subclause. First the UE determines which emission limit may be failed with IMD3 products for -13dBm/MHz or -25dBm/MHZ SEM limits and -30dBm/MHz spurious emissions limit and applies the corresponding MPR equations from sections 6.2B.2.1.2.1, 6.2B.2.1.2.2 and 6.2B.2.1.2.3 respectivelly.

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRIM3 )

MPRNR = MAX(MPRsingle,NR, MPRIM3 )

For UEs supporting dynamic power sharing , with backoff applied equally to E-UTRA and NR

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRIM3 )

MPRNR = MAX(MPRsingle,NR, MPRIM3)

where

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in [2]

6.2B.2.1.2.1	MPRIM3 to meet -13 dBm / 1MHz for 26dBm UE power

MPR in this sub-clause is relative to 26 dBm for power class 2. The same MPR is used relative to 23 dBm for power class 3. The allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

MPRIM3 = MA

Where MA is defined as follows

MA = 	14	;	 0 ≤ B < 0.5

9	;	 0.5 ≤ B < 1.0

7	; 	1.0 ≤ B < 2.0

5	; 	2.0 < B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

6.2B.2.1.2.2	MPR to meet -25 dBm / 1MHz

MPR in this sub-clause is relative to 26 dBm. The same MPR is used relative to 23 dBm for power class 3. The allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

MPRIM3 = MA

Where MA is defined as follows

MA = 	14	; 	0 ≤ B < 1.0

13	; 	1.0 ≤ B < 2.0

12	; 	2.0 ≤ B < 5.0

11	; 	5.0 < B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

For NR

B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

6.2B.2.1.2.3	MPR to meet -30 dBm / 1MHz

MPR in this sub-clause is relative to 26 dBm. The same MPR is used relative to 23 dBm for power class 3. The allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 	17	; 	0 ≤ B < 1.0

16	; 	1.0 ≤ B < 2.0

15	; 	2.0 ≤ B < 5.0

14	; 	5.0 < B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

For NR

#### B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

#### 6.2B.2.2	Intra-band non-contiguous EN-DC

##### 6.2B.2.2.1	MPR for DC_41A_n41A

When the UE is configured for B41/n41 intra-band non-contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this subclause. First the UE determines which emission limit may be failed with IMD3 products for -13dBm/MHz or -25dBm/MHZ SEM limits and -30dBm/MHz spurious emissions limit and applies the corresponding MPR equations from sections 6.2B.2.1.2.1, 6.2B.2.1.2.2 and 6.2B.2.1.2.3 respectivelly.

The UE determines the Channel Configuration Case and the value of MPRIM3 as follows:

If AND( FIM3,low_block,high < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

MPRIM3 defined in Subclause 6.2B.2.1.2.1

Else

UE determines which emission limit may be failed with IMD3 products for -13dBm/MHz or -25dBm/MHZ SEM limits and -30dBm/MHz spurious emissions limit and applies the corresponding MPR equations from sections 6.2B.2.1.2.1, 6.2B.2.1.2.2 and 6.2B.2.1.2.3 respectivelly.

where

-	FIM3,low_block,high = (2 * Flow_channel,high_edge ) – Fhigh_channel,low_edge

-	FIM3,high_block,low = (2 * Fhigh_channel,low_edge) – Flow_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Flow_channel,high_edge is the uppermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,low_edge is the lowermost frequency of upper transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz.

The UE determines the value of MPRACLRoverlap as specified in Table 6.2B.2.2.1-1:

Table 6.2B.2.2.1-1: A-MPRACLRoverlap

| Wgap | A-MPRACLRoverlap |
| --- | --- |
| < BWchannel,E-UTRA + BWchannel,NR | 4 dB |
| ≥ BWchannel,E-UTRA + BWchannel,NR | 0 dB |
| NOTE 1: Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge |  |

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

MPR E-UTRA  = MAX(MPRsingle,E-UTRA, MPRIM3, MPRACLRoverlap )

A-MPRNR = MAX(MPRsingle,NR, MPRIM3, MPRACLRoverlap )

For UEs supporting dynamic power sharing , with IM3 backoff applied equally to E-UTRA and NR

MPREN-DC = MAX(A-MPRIM3, A-MPRACLRoverlap )

MPRE-UTRA = MAX(MPRsingle,E-UTRA, A-MPREN-DC )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPREN-DC )

where

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in [4].

-	MPRsingle,NR is the A-MPR defined for the NR transmission in [2].

#### <End of changes>