3GPP TSG-RAN WG4 Meeting #89	R4-1815870

Spokane, WA, USA, 12th – 16th Nov, 2018

Agenda item:	6.11.3

Source: 	Qualcomm Incorporated

Title: 	NW CRS-IM demod simulation results

Document for:	Discussion

# Introduction

In the last RAN4#88bis meeting, the Way Forward for network based CRS interference mitigation demodulation requirement was agreed [1]. Two FRC options and two muting pattern options were agreed for simulations. In this paper, we present simulation results for these cases in FDD along with some discussion points.

# Simulation results

# 2.1 FDD

Figure 1 shows the muting pattern for option 1 with 11 subframes hit by interference from neighbor cell and 9 subframes without interference repeated every 2 radio frames. The shaded subframes represent subframes hit with interference. Since both FRC options do not have PDSCH allocation in subframe 5, UE is receiving 9 subframes with no interference and another 9 with interference (i.e., 50% duty cycle for interference). For option 2, neighbor cell does not transmit CRS except for middle 6 RBs in any subframe resulting in 0% effective duty cycle.

Figure 1 FDD CRS interference pattern for muting pattern option 1

Figures 2 shows FDD performance for FRC1 and FRC2 with interference level at 10 and 5 dB, respectively where option 1 (red) corresponds to 50% interference duty cycle and option 2 (green) corresponds to 0% duty cycle, and FRC1 and FRC2 use the same modulation order and coding rate as FDD R.11 and R.10, respectively, with only 41 RBs configured for PDSCH as agreed in [1].

Figure 2 FDD simulation results for FRC1 and FRC2 with muting options 1 and 2

For FRC1, the SNR point corresponding to 70% of max throughput is achieved at 12.5 and 10.5 dB for muting pattern options 1 and 2, respectively. For FRC2, the SNR point corresponding to 70% of max throughput is achieved at 1.5 and 0.5 dB for muting pattern options 1 and 2, respectively.

Observation 1. In FDD, for FRC1, the SNR point corresponding to 70% of max throughput is achieved at 12.5 and 10.5 dB for muting pattern options 1 and 2, respectively. For FRC2, the SNR point corresponding to 70% of max throughput is achieved at 1.5 and 0.5 dB for muting pattern options 1 and 2, respectively. These SNR values are without implementation margin.

2.2 Discussion

It is our view that option 1 for muting pattern represents a more realistic muting of CRS transmission since it protects SIB1 with full bandwidth CRS transmission as specified in Section 3.6.1.1 of TS36.133. Option 2 for muting pattern does not have significant difference from a single-cell test from demod perspective.

Proposal 1. Adopt FRC1 with option 1 for muting pattern for NW-based CRS-IM demod tests.

It is noted that muting pattern option 1 which requires 11 valid DL subframes does not fit well with TDD UL/DL configuration 1 which is associated with FRC1 and FRC2. This option effectively turns all DL subframes of the 2 radio frame period into subframes hit by neighbour cell interference. It is proposed to change UL/DL configuration from 1 to either 4 or 5 for TDD tests.

Proposal 2. Modify TDD ULDL configuration from 1 to either 4 or 5 for TDD CRS-IM tests to fit muting pattern option 1.

# Conclusions

Observation 1. In FDD, for FRC1, the SNR point corresponding to 70% of max throughput is achieved at 12.5 and 10.5 dB for muting pattern options 1 and 2, respectively. For FRC2, the SNR point corresponding to 70% of max throughput is achieved at 1.5 and 0.5 dB for muting pattern options 1 and 2, respectively. These SNR values are without implementation margin.

Proposal 1. Adopt FRC1 with option 1 for muting pattern for NW-based CRS-IM demod tests.

Proposal 2. Modify TDD ULDL configuration from 1 to either 4 or 5 for TDD CRS-IM tests to fit muting pattern option 1.

# References

[1] R4-1813714