3GPP TSG-RAN WG4 Meeting #89	R4-1815871

Spokane, WA, USA, 12th – 16th Nov, 2018

Agenda item:	6.7.6.1.1

Source: 	Qualcomm Incorporated

Title: 	On CQI2MCS tables for subslot PDSCH fading tests

Document for:	Discussion

# Introduction

In the last RAN4#88 meeting, CQI reporting tests under fading conditions for slot-based and subslot-based PDSCH were introduced [1].  In this paper, we discuss the subslot CQI2MCS tables and propose options for feasible testing under fading channel conditions.

# Discussion on subslot CQI2MCS tables

Table 1 below  tabulates the MCS index for different subslot TTIs in CRS-based, DMRS-based with no CSI-RS and DMRS-based with CSI-RS according to 64QAM Table 7.1.7.1-1 in TS 36.213.

| CQI Index |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation |  |  |  | QPSK |  |  |  |  |  |  | 16QAM |  |  | 64QAM |  |  |  |  |  |  |
| Target Code Rate |  |  |  | OOR | 0.0761 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6016 | 0.4551 | 0.5537 | 0.6504 | 0.7539 | 0.8525 | 0.9258 |  |
| MCS Scheme | PRB | Available RE-s | Subslot number | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.35-1 | 50 | 1408 | 1 | DTX | 0 | 1 | 3 | 6 | 8 | 9 | 14 | 16 | 16 | 22 | 25 | 28 | 28 | 28 | 28 | CRS-based subslot |
| MCS.35-2 | 50 | 1008 | 2 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 28 |  |
| MCS.35-3 | 50 | 872 | 3 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 20 | 22 | 24 | 25 |  |
| MCS.35-4 | 50 | 1008 | 4 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 28 |  |
| MCS.35-5 | 50 | 1472 | 5 | DTX | 0 | 2 | 4 | 6 | 9 | 9 | 14 | 16 | 16 | 22 | 25 | 28 | 28 | 28 | 28 |  |
| MCS.36-1 | 50 | 1180 | 1 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 12 | 15 | 16 | 20 | 22 | 24 | 27 | 28 | 28 | DMRS-based subslot (no CSI-RS) |
| MCS.36-2 | 50 | 680 | 2 | DTX | 0 | 0 | 0 | 2 | 4 | 6 | 10 | 10 | 12 | 17 | 17 | 18 | 19 | 21 | 21 |  |
| MCS.36-3 | 50 | 612 | 3 | DTX | 0 | 0 | 0 | 2 | 3 | 5 | 10 | 10 | 11 | 17 | 17 | 17 | 18 | 19 | 20 |  |
| MCS.36-4 | 50 | 680 | 4 | DTX | 0 | 0 | 0 | 2 | 4 | 6 | 10 | 10 | 12 | 17 | 17 | 18 | 19 | 21 | 21 |  |
| MCS.36-5 | 50 | 1212 | 5 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 13 | 15 | 16 | 20 | 22 | 25 | 28 | 28 | 28 |  |
| MCS.37-1 | 50 | 1180 | 1 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 12 | 15 | 16 | 20 | 22 | 24 | 27 | 28 | 28 | DMRS-based subslot (with CSI-RS) |
| MCS.37-2 | 50 | 612 | 2 | DTX | 0 | 0 | 0 | 2 | 3 | 5 | 10 | 10 | 11 | 17 | 17 | 17 | 18 | 19 | 20 |  |
| MCS.37-3 | 50 | 612 | 3 | DTX | 0 | 0 | 0 | 2 | 3 | 5 | 10 | 10 | 11 | 17 | 17 | 17 | 18 | 19 | 20 |  |
| MCS.37-4 | 50 | 680 | 4 | DTX | 0 | 0 | 0 | 2 | 4 | 6 | 10 | 10 | 12 | 17 | 17 | 18 | 19 | 21 | 21 |  |
| MCS.37-5 | 50 | 1212 | 5 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 13 | 15 | 16 | 20 | 22 | 25 | 28 | 28 | 28 |  |
| Note 1: Mapping between Imcs and CQI Index according to Table 7.1.7.1-1 in TS 36.213. Note 2: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1. Adopt Table 1 for subslot-PDSCH tests in Section 9.12 of TS 36.101.

Figure 1 below compares the effective code rate of subslot sTTI 1-5 with target code rate for different CQI indices for CRS-based rows of Table 1 (MCS.35). It can be seen that sTTI#1 and sTTI#5 corresponding to available number of RE’s 1408 and 1472 deviate significantly from target code rate in mid as well as high CQI regions. Table 1 also shows that 2 or more successive CQI are mapped to the same MCS which is undesirable for fading CQI tests.

Observation 1. For CRS-based subslot CQI to MCS table, sTTI#1 and sTTI#5 have significant deviation from target code rate with successive CQI’s mapping to the same MCS.

Based on this observation, we propose to not use sTTI#1 and sTTI#5 for CRS-based subslot-PDSCH fading tests.

Proposal 2. For CRS-based subslot-PDSCH fading CQI tests, avoid using sTTI#1 and sTTI#5.

Figure 1 Effective code rate vs. target code rate comparison for CRS-based subslot sTTI

Figure 2 below compares the effective code rate of subslot sTTI 1-5 with target code rate for different CQI indices for DMRS-based rows of Table 1 (MCS36 and MCS37). It can be seen that sTTI#2, sTTI#3, sTTI#4 corresponding to available number of RE’s 612 and 680 deviate significantly from target code rate in low, mid as well as high CQI regions. Table 1 also shows that 2 or more successive CQI’s for these sTTI’s are mapped to the same MCS which is undesirable for fading CQI tests.

Observation 2. For DMRS-based subslot CQI to MCS table, sTTI#2, sTTI#3, sTTI#4 have significant deviation from target code rate with successive CQI’s mapping to the same MCS.

Based on this observation, we propose to only use sTTI#1 and sTTI#5 for DMRS-based subslot-PDSCH fading tests.

Proposal 3. For DMRS-based subslot-PDSCH fading CQI tests, only use sTTI#1 and sTTI#5.

Figure 2 Effective code rate vs. target code rate comparison for DMRS-based subslot sTTI

# Conclusions

Proposal 1. Adopt Table 1 for subslot-PDSCH tests in Section 9.12 of TS 36.101.

Observation 1. For CRS-based subslot CQI to MCS table, sTTI#1 and sTTI#5 have significant deviation from target code rate with successive CQI’s mapping to the same MCS.

Proposal 2. For CRS-based subslot-PDSCH fading CQI tests, avoid using sTTI#1 and sTTI#5.

Observation 2. For DMRS-based subslot CQI to MCS table, sTTI#2, sTTI#3, sTTI#4 have significant deviation from target code rate with successive CQI’s mapping to the same MCS.

Proposal 3. For DMRS-based subslot-PDSCH fading CQI tests, only use sTTI#1 and sTTI#5.

# References

[1] R4-1813712