3GPP TSG-RAN WG4 Meeting #89	R4-1815872

Spokane, WA, USA, 12th – 16th Nov, 2018

Agenda item:	6.12.2.1

Source: 	Qualcomm Incorporated

Title: 	Remaining items for applicability rule for 8Rx capable UE

Document for:	Discussion

# Introduction

In the previous two meetings (RAN4#88 and RAN4#88bis), applicability rules for 8Rx capable UE were discussed and agreed upon for all scenarios except 2Rx test cases for which 2Rx supported bands do not exist yet but both 4Rx supported bands and 8Rx supported bands exist [1][2]. In this contribution, we discuss the remaining scenario in the applicability rule for 8Rx capable UE from PDSCH/PDCCH demodulation and RLM perspectives.

# Applicability Rule

## PDSCH demodulation

Applicability rule agreed in the last RAN4#88 meetings is summarized in the Table 1, where the applicability rule for the scenario 3 is left as FFS. For the scenario 3, our view is that the 2Rx demodulation test should be run on 4Rx-supported band, instead of 8Rx-supported band, in order to minimize the approximation in the requirement. Note that when 2Rx demodulation requirement is tested on 8Rx-supported band, a larger margin is added with respect to the theoretical SNR gain, i.e., 3dB tightening versus 6dB theoretical SNR gain (3dB gap), while the gap is only 1.5dB when the same test is run on 4Rx-supported band.

Proposal 1. For a UE that has both 4Rx- and 8Rx-supported bands, but without any 2Rx-supported band, existing demodulation requirement defined based on 2Rx UE is run on the 4Rx-supported band with SNR requirement tightened by 1.5dB.

Table 1. Applicability rule for 2Rx/4Rx PDSCH demodulation test for 8Rx capable UE

| Scenario | Demod test case | Exists 2Rx supported bands | Exists 4Rx supported bands | UE tested on | SNR requirement tightening |
| --- | --- | --- | --- | --- | --- |
| 1 | 2Rx | Yes | Yes/No | 2Rx band | N/A |
| 2 | 2Rx | No | No | 8Rx band | 3dB |
| 3 | 2Rx | No | Yes | FFS: 4Rx or 8Rx band | 1.5dB if run on 4Rx band. 3dB otherwise |
| 4 | 4Rx | Yes/No | No | 8Rx band | 1.5dB |
| 5 | 4Rx | Yes/No | Yes | 4Rx band | N/A |

## PDCCH demodulation

In the RAN4 #88 meeting, it was agreed that UE is not required to use 8Rx for the demodulation of broadcast/control channels. Therefore, in terms of UE demodulation requirement for control channels, there would be no additional tightening with respect to the 4Rx-capable UE. Table 2 summarizes the applicability rule and SNR tightening for 2Rx/4Rx-based PDCCH demodulation requirement tested on the 8Rx-capable UE. In the last RAN4 #88bis meeting, the applicability rule for scenarios other than 3 was agreed based on the Table 2 [1].

Proposal 2. Adopt Table 2 as the applicability rule for 2Rx/4Rx PDCCH demodulation requirement for 8Rx-capable UE

Table 2. Applicability rule for 2Rx/4Rx PDCCH demodulation test for 8Rx capable UE

| Scenario | Demod test case | Exists 2Rx supported bands | Exists 4Rx supported bands | UE tested on | SNR requirement tightening |
| --- | --- | --- | --- | --- | --- |
| 1 | 2Rx | Yes | Yes/No | 2Rx band | N/A |
| 2 | 2Rx | No | No | 8Rx band | 1.5dB |
| 3 | 2Rx | No | Yes | 4Rx band | 1.5dB |
| 4 | 4Rx | Yes/No | No | 8Rx band | 0 dB |
| 5 | 4Rx | Yes/No | Yes | 4Rx band | N/A |

## Radio link monitoring

RAN4 has already agreed that no new RLM requirement shall be defined for 8Rx-capable UE and UE is not required to use 8Rx for RLM measurement. Furthermore, RAN4 agreed that UE is not required to use 8Rx for the demodulation of broadcast/control channels. For an 8-Rx capable UE without any 2Rx or 4Rx supported bands, RLM is tested on 8Rx supported band. In this scenario, we propose using existing applicability rules for 4Rx UE without any further modifications.

Proposal 3. Applicability rule for RLM requirement for 8Rx-capable UE is defined as follows:

- For an 8Rx-capable UE with 2Rx supported band, RLM requirement is tested on 2Rx-supported band.

- For an 8Rx-capable UE with 4Rx supported band but without any 2Rx supported band, RLM requirement is tested on 4Rx-supported band.

  - Existing applicability rules for 4Rx UE apply.

- For an 8Rx-capable UE without any 2Rx or 4Rx supported bands, RLM requirement is tested on 8Rx-supported band

  - Existing applicability rules for 4Rx UE apply.

# Conclusions

In this paper, we discussed the open issues in the applicability rules for 8Rx-capable UE. The proposals made in this paper is summarized as follows.

Proposal 1. For a UE that has both 4Rx- and 8Rx-supported bands, but without any 2Rx-supported band, existing demodulation requirement defined based on 2Rx UE is run on the 4Rx-supported band with SNR requirement tightened by 1.5dB.

Proposal 2. Adopt Table 2 as the applicability rule for 2Rx/4Rx PDCCH demodulation requirement for 8Rx-capable UE

Table 2. Applicability rule for 2Rx/4Rx PDCCH demodulation test for 8Rx capable UE

| Scenario | Demod test case | Exists 2Rx supported bands | Exists 4Rx supported bands | UE tested on | SNR requirement tightening |
| --- | --- | --- | --- | --- | --- |
| 1 | 2Rx | Yes | D/C | 2Rx band | N/A |
| 2 | 2Rx | No | No | 8Rx band | 1.5dB |
| 3 | 2Rx | No | Yes | 4Rx band | 1.5dB |
| 4 | 4Rx | Yes/No | No | 8Rx band | 0 dB |
| 5 | 4Rx | Yes/No | Yes | 4Rx band | N/A |

Proposal 3. Applicability rule for RLM requirement for 8Rx-capable UE is defined as follows:

- For an 8Rx-capable UE with 2Rx supported band, RLM requirement is tested on 2Rx-supported band.

- For an 8Rx-capable UE with 4Rx supported band but without any 2Rx supported band, RLM requirement is tested on 4Rx-supported band.

  - Existing applicability rules and principles of testing defined for 4Rx UE in TS36.133 A3.8 apply.

- For an 8Rx-capable UE without any 2Rx or 4Rx supported bands, RLM requirement is tested on 8Rx-supported band

  - Existing applicability rules for 4Rx UE apply.

# References

[1] R4-1811868

[2] R4-1813491