3GPP TSG-RAN WG4 Meeting #89	R4-1815874

Spokane, WA, USA, 12th – 16th Nov, 2018

Agenda item:	6.5.4.1

Source: 	Qualcomm Incorporated

Title: 	On simulation assumption for Rel-15 MTC WUS

Document for:	Discussion

# Introduction

In the previous RAN4#88 and #88bis meeting, simulation assumption for Rel-15 MTC WUS were discussed [1][2]. In this paper, we discuss the simulation assumption and present a preliminary simulation result.

# Discussion

The following table summarizes the simulation assumptions proposed in [1].

Table 1 Simulation assumption for WUS performance requirement in MTC [1]

| Parameter | Value |
| --- | --- |
| BS Tx ant configuration | 1 Tx, 2Tx with Tx diversity, Note:TxD scheme will depend on RAN1 agreement |
| UE Rx ant configuration | 1 Rx |
| WUS BW | 2 PRBs |
| Channel model | EPA 1Hz, ETU 1Hz, EPA 5Hz, ETU 30Hz |
| SNR | -6 dB, -15 dB |
| False alarm rate | 1% |
| Detection probability | 99% |

For enhanced coverage point, the selected SNR is -15 dB. However, with fading channel models EPA 1Hz, ETU 1Hz, EPA 5Hz, ETU 30Hz, UE experiences deep fades with effective SNR significantly below -15 dB and reaching ~ -20 dB where UE’s datapath and tracking loops may not properly function. We propose to use SNR of -12 dB for the enhanced coverage point with the channel models in Table 1.

Proposal 1. Use SNR -12 dB instead of -15 dB for the simulation assumption for the enhanced coverage.

The following table summarizes the frequency and time drifts for MTC WUC simulations. Here, a 45 Hz per second frequency drift with an initial frequency error of 30 Hz is assumed. The frequency drift of 45 Hz corresponds to 0.05 ppm if the carrier frequency is assumed to be 900 MHz. The 0.05 ppm assumption is in line with agreed RAN1 model [3].

Table 2 Frequency/time drifts based on carrier frequency of 900 MHz

| DRX cycle (s) | Frequency Drift [Hz] | Timing Drift [us] |
| --- | --- | --- |
| 0.32 | 44.4 | 4.69006 |
| 0.64 | 58.8 | 4.69774 |
| 1.28 | 87.6 | 4.72846 |
| 2.56 | 145.2 | 4.85134 |

However, according to TS36.101 Section 5.5E, category M1 and M2 are designed to operate in band 1 and band 66 with high end downlink carrier frequency of 2.2 GHz which results in higher frequency drifts compared to Table 2. Based on the methodology of [3], we propose Table 3 to be used for frequency/time drifts of NB-IoT WUS simulations that is based on carrier frequency of 2.2 GHz. It is noted that no initial frequency error is assumed in Table 3.

Table 3 Frequency/time drifts based on carrier frequency of 2.2 GHz

| DRX cycle (s) | Frequency Drift [Hz] | Timing Drift [us] |
| --- | --- | --- |
| 0.32 | 35.2 | 4.69006 |
| 0.64 | 70.4 | 4.69774 |
| 1.28 | 140.8 | 4.72846 |
| 2.56 | 281.6 | 4.85134 |

Proposal 2. To adopt frequency/time drift in Table 3 for WUS simulation.

Table 4 presents a preliminary simulation result for the number of WUS subframes required to reach 1% false alarm and 99% detection probability for DRX cycle of 0.32s for SNR = -6 dB, where the frequency and time drifts in Table 3 is assumed. Ncoh represents the number of subframes used for coherent combining. When number of Tx antennas is 1, Ncoh = 1 requires half of the number of WUS subframes to reach the required performance level compared to Ncoh = 0. However, for 2 Tx antennas that delivers Tx diversity, Ncoh = 0 and 1 perform similarly.

Table 4 Number of WUS subframes needed for 1% FA/MD for DRX cycle of 0.32s and SNR= -6 dB

|  | Ncoh | NTx=1 | NTx=2 |
| --- | --- | --- | --- |
| EPA1 | 1 | 64 | 8 |
|  | 0 | 128 | 8 |
| EPA5 | 1 | 32 | 8 |
|  | 0 | 64 | 8 |
| ETU1 | 1 | 16 | 4 |
|  | 0 | 32 | 4 |
| ETU30 | 1 | 8 | 4 |
|  | 0 | 8 | 4 |

Observation 1. For DRX cycle of 0.32s and SNR = -6 dB and 1Tx antenna, number of WUS subframes is halved if 1-subframe of coherent combining is used with non-coherent combining across subframes compared to full non-coherent combining. For DRX cycle of 0.32s and SNR = -6 dB and 2Tx antenna, number of WUS subframes is the same between 1-subframe of coherent combining with non-coherent combining across subframes and full non-coherent combining.

Considering the above preliminary simualtion result, we propose to consider two coherent combining options for WUS simulations: 1) no coherent combining, 2) up to 2-sf coherent combining.

Proposal 3. For MTC WUS simulation assumptions, consider two coherent combining options: 1) no coherent combining, 2) up to 2-sf coherent combining.

# Conclusions

Proposal 1. Use SNR -12 dB instead of -15 dB for the simulation assumption for the enhanced coverage.

Proposal 2. To adopt frequency/time drift in Table 3 for WUS simulation.

Table 3 Frequency/time drifts based on carrier frequency of 2.2 GHz

| DRX cycle (s) | Frequency Drift [Hz] | Timing Drift [us] |
| --- | --- | --- |
| 0.32 | 35.2 | 4.69006 |
| 0.64 | 70.4 | 4.69774 |
| 1.28 | 140.8 | 4.72846 |
| 2.56 | 281.6 | 4.85134 |

Observation 1. For DRX cycle of 0.32s and SNR = -6 dB and 1Tx antenna, number of WUS subframes is halved if 1-subframe of coherent combining is used with non-coherent combining across subframes compared to full non-coherent combining. For DRX cycle of 0.32s and SNR = -6 dB and 2Tx antenna, number of WUS subframes is the same between 1-subframe of coherent combining with non-coherent combining across subframes and full non-coherent combining (Table 4).

Table 4 Number of WUS subframes needed for 1% FA/MD for DRX cycle of 0.32s and SNR= -6 dB

|  | Ncoh | NTx=1 | NTx=2 |
| --- | --- | --- | --- |
| EPA1 | 1 | 64 | 8 |
|  | 0 | 128 | 8 |
| EPA5 | 1 | 32 | 8 |
|  | 0 | 64 | 8 |
| ETU1 | 1 | 16 | 4 |
|  | 0 | 32 | 4 |
| ETU30 | 1 | 8 | 4 |
|  | 0 | 8 | 4 |

Proposal 3. For MTC WUS simulation assumptions, consider two coherent combining options: 1) no coherent combining, 2) up to 2-sf coherent combining.

# References

[1] R4-1814031

[2] R4-1811690

[2] R1-1714992