3GPP TSG RAN WG4 Meeting #89	R4-1815915

Spokane, US, November 12 – 16, 2018

Agenda Item:	7.11.3.1.2

Source: 	Ericsson

Title:	On UE behaviour before and after measurement gap

Document for:	Discussion

# Introduction

At RAN4#88bis, based on a way forward from earlier meeting, [1], the UE behaviour before and after measurement gaps was discussed. Proposals were put forward by network vendors on allowing UEs to be scheduled also in the non-overlapped half-slots that arise for SCS 15kHz when a measurement gap is shifted by MGTA 0.5ms [2][3], but no conclusion was reached. In a companion contribution [4] we are proposing that the non-overlapped half-slots are recognized as candidates for scheduling. In this contribution we are discussing how those half-slots can be used in Rel-15.

# Discussion

As pointed out in the companion contribution [4], the loss of scheduling opportunities may be up to 35% in NR for SCS 15kHz when MGRP 20ms and MGTA 0.5ms is used. This exceeds the 25% loss in LTE for MGRP 40ms which eventually lead to RAN4-work on measurement gap enhancements. When allowing the half-frames to be used for scheduling, the loss of scheduling opportunities reduces to 25% and thereby is on par with the LTE baseline.

Given that the issue regarding scheduling opportunities was identified late in Rel-15, it is understandable that this may cause concern for development. It is however inevitable that at some point, the half-slots will be used for scheduling as a loss of scheduling opportunities of as much as 35% is undesirable. Our preference is that Rel-15 UEs support scheduling in half-slots – as is already supported in the NR baseline (see [3]). Hence, we propose the following UE behaviour, as captured in [5]:

When non-zero measurement gap timing advance is configured, in slots that are partially overlapped by

- the beginning of per-UE or per-FR measurement gaps, the UE

-	shall receive channels and signals for which time domain resource allocations are comprised in the non-overlapped first half of the slot

-	shall transmit channels and signals for which time domain resource allocations are comprised in the non-overlapped first half of the slot

- the end of per-UE or per-FR measurement gaps, the UE

- 	shall receive channels and signals for which time domain resource allocations are comprised in the non-overlapped second half of the slot

In case consensus cannot be reached, we can consider conditional support in Rel-15 such as for instance support only when in SA operation, etc. Leaving it up to UE implementation in Rel-15 is also a possibility but comes at a cost with respect to system throughput since scheduling resources may be wasted during the time the gNB establishes whether the UE can receive and/or transmit in half-slots. Therefor our preference is that it shall be possible to determine beforehand whether a UE supports operation in half-slots around measurement gaps.

# Summary and Conclusions

In this contribution we have provided our view on scheduling of UEs in half-slots before and after measurement gaps in Rel-15. Our preference is captured a draft CR [5], but we are open to discussions on whether support shall be conditioned on e.g. certain capabilities or use cases.

# References

- R4-1811869 “Way forward on UE behavior before or after measurement gap”, ZTE

- R4-1812859 “Discussion on UE behaviour before and after measurement gap”, ZTE

- R4-1813427 “On UE behaviour before and after measurement gap”, Ericsson

- R4-1815913 “On Measurement Gaps and Scheduling Opportunities”, Ericsson

- R4-1815916 “DraftCR 38.133 (9.1.2) UE behaviour before and after measurement gap”, Ericsson