3GPP TSG-RAN WG4 Meeting #89	R4-181aaaa

# Spokane, USA, 12th – 16th November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-3 rel. 15 to fix MPR issue |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel. 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently the MPR chapter for DC combinations is empty, that means not even the power reduction for the single E-UTRA or LTE carrier is allowed resulting in issues to fulfill EVM, inband emissions, ACLR etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a sentence allowing to use the single carrier MPR also for EN-DC combinations similarly to what has been introduced for NR CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs will fail some TX tests where MPR is needed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-3 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6.2B.2	UE maximum output power reduction and EN-DC

#### 6.2B.2.1	Intra-band contiguous EN-DC

#### For intra-band contiguous EN-DC, single carrier UE maximum output power reduction specified in TS 36.101 [4] for E-UTRA and TS 38.101-1 [2] for NR apply for E-UTRA and NR carriers respectively, unless additional MPR is specified below for IMD reasons.

#### 6.2B.2.2	Intra-band non-contiguous EN-DC

#### For intra-band non-contiguous EN-DC, single carrier UE maximum output power reduction specified in TS 36.101 [4] for E-UTRA and TS 38.101-1 [2] for NR apply for E-UTRA and NR carriers respectively, unless additional MPR is specified below for IMD reasons.

#### 6.2B.2.3	Inter-band EN-DC within FR1

#### For inter-band EN-DC within FR1, single carrier UE maximum output power reduction specified in TS 36.101 [4] for E-UTRA and TS 38.101-1 [2] for NR apply for E-UTRA and NR carriers respectively.

< conducted requirements >

#### 6.2B.2.4	Inter-band EN-DC including FR2

#### For inter-band EN-DC including  FR2, single carrier UE maximum output power reduction specified in TS 36.101 [4] and TS 38.101-2 [3] apply for each frequency range and RAT respectively.

< OTA requirements >

#### 6.2B.2.5	Inter-band EN-DC including both FR1 and FR2

#### For inter-band EN-DC including both, FR1 and FR2, single carrier UE maximum output power reduction specified in TS 36.101 [4], TS 38.101-1 [2] and TS 38.101-2 [3] apply for each frequency range and RAT respectively.

< OTA requirements >