3GPP TSG-RAN WG4 Meeting #89	R4-1815987

# Spokane, Washington, USA, 12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101 | CR | 5310 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Addition of CQI tests for 8Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_8Rx_AP_DL-Perf |  |  |  |  |  | Date: |  |  | 2018-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed to introduce CQI reporting requirements for 8Rx capable UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the CQI reporting requirements for 8Rx capable UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No CQI reporting requirement for 8Rx UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 9.12, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

## 9.12	CSI reporting for 8Rx UE

### 9.12.1	CQI reporting definition under AWGN conditions

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 36.213 [6]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

#### 9.12.1.1		Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

##### 9.12.1.2.1	FDD

The following requirements apply to UE Category 8 and DL Category 14, 17~20, ≥22. For the parameters specified in table 9.12.1.2.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.12.1.2.1-1: PUCCH 1-1 submode 1 static test for 8x8 (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | -3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (8x8 for Rank8) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | Rank 8: 0x0000 0000 0000 0010 0000 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note3) |  |  |  |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |  |  |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note4) |  |  |  |
| Note 1: Reference measurement channel RC.23 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

##### 9.12.1.2.2	TDD

The following requirements apply to UE Category 8 and DL Category 14, 17~20, ≥22. For the parameters specified in table 9.12.1.2.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.12.1.2.2-1: PUCCH 1-1 submode 1 static test 8x8 (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | -3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (8x8 for Rank8) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | Rank 8: 0x0000 0000 0000 0010 0000 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |  |  |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.23 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

<< Unchanged part omitted >>

# A.4	CSI reference measurement channels

This section defines the DL signal applicable to the reporting of channel status information (Clause 9.2, 9.3 and 9.5).

In Table A.4-1 are specified the reference channels. Table A.4-13 specifies the mapping of CQI index to modulation coding scheme, which complies with the CQI definition specified in Section 7.2.3 of [6].

Table A.4-0: Void

Table A.4-1: CSI reference measurement channels

| RMC Name | Duplex | CH-BW | Alloc. RB-s | UL/DL Config | Alloc. SF-s | MCS Scheme | Nr. HARQ Proc. | Max. nr HARQ Trans. | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 CRS Port |  |  |  |  |  |  |  |  |  |
| RC.1 FDD | FDD | 10 | 50 | - |  | MCS.1 | 8 | 1 |  |
| RC.1A FDD | FDD | 10 | 50 |  |  | MCS.1A | 8 | 1 |  |
| RC.1 TDD | TDD | 10 | 50 | Note 3 |  | MCS.1 | 10 | 1 |  |
| RC.1A TDD | TDD | 20 | 100 | Note 3 |  | MCS.1B | 10 | 1 |  |
| RC.3 FDD | FDD | 10 | 6 | - |  | MCS.10 | 8 | 1 |  |
| RC.3 TDD | TDD | 10 | 6 | Note 3 |  | MCS.10 | 10 or 7 (Note 9) | 1 |  |
| RC.4 FDD | FDD | 10 | 15 | - |  | MCS.15 | 8 | 1 | Note 6 |
| RC.4 TDD | TDD | 10 | 15 | Note 3 |  | MCS.15 | 10 | 1 | Note 6 |
| RC.5 FDD | FDD | 10 | 3 | - |  | MCS.17 | 8 | 1 |  |
| RC.5 TDD | TDD | 10 | 3 | Note 3 |  | MCS.17 | 10 | 1 |  |
| RC.14 FDD | FDD | 5 | 25 | - |  | MCS.14 | 8 | 1 |  |
| RC.15 FDD | FDD | 5 | 15 | - |  | MCS.15 | 8 | 1 | Note 6 |
| RC.16 FDD | FDD/HD-FDD | 10 | 2 |  |  | MCS.20 | 8 | 1 | Note 8,10 |
| RC.16 TDD | TDD | 10 | 2 | Note 3 |  | MCS.20 | 10 | 1 | Note 8 |
| RC.23FDD | FDD/HD-FDD | 10 | 3 |  |  | MCS.28 | 8 | 1 | Note 12, 13 |
| RC.23 TDD | TDD | 10 | 3 |  |  | MCS.28 | 10 | 1 | Note 12 |
| RC.25 FDD | FDD/HD-FDD | 10 | 3 |  |  | MCS.28 | 8 | 1 | Note 12, 14 |
| RC.25 TDD | TDD | 10 | 3 |  |  | MCS.28 | 10 | 1 | Note 12, 15 |
| 2 CRS Ports |  |  |  |  |  |  |  |  |  |
| RC.2 FDD | FDD | 10 | 50 | - |  | MCS.2 | 8 | 1 |  |
| RC.2A FDD | FDD | 20 | 100 |  |  | MCS.2A | 8 | 1 |  |
| RC.2 TDD | TDD | 10 | 50 | Note 3 |  | MCS.2 | 10 or 7 (Note 9) | 1 |  |
| RC.4A FDD | FDD | 20 | 15 | - |  | MCS.16 | 8 | 1 | Note 6 |
| RC.6 FDD | FDD | 10 | 15 | - |  | MCS.16 | 8 | 1 | Note 6 |
| RC.6 TDD | TDD | 10 | 15 | Note 3 |  | MCS.16 | 7 | 1 | Note 6 |
| 4 CRS Ports |  |  |  |  |  |  |  |  |  |
| RC.17 FDD | FDD | 10 | 50 | - |  | MCS.18 | 8 | 1 |  |
| RC.17 TDD | TDD | 10 | 50 | Note 3 |  | MCS.18 | 7 | 1 |  |
| RC.21 FDD | FDD | 10 | 50 | - |  | MCS.26 | 8 | 1 |  |
| RC.21 TDD | TDD | 10 | 50 | Note 3 |  | MCS.26 | 7 | 1 |  |
| 1 CRS Port + CSI-RS |  |  |  |  |  |  |  |  |  |
| RC.8 FDD | FDD | 10 | 6 | - | Non CSI-RS | MCS.11 | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.12 |  |  |  |
| RC.8A FDD | FDD | 10 | 6 | - | Non CSI-RS | MCS.11A | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.12A |  |  |  |
| RC.8 TDD | TDD | 10 | 6 | Note 3 | Non CSI-RS | MCS.11 | 10 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.12 |  |  |  |
| RC.8A TDD | TDD | 20 | 8 | Note 3 | Non CSI-RS | MCS.11B | 10 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.12B |  |  |  |
| RC.9 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.3 | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.4 |  |  |  |
| RC.9A FDD | FDD | 20 | 100 | - | Non CSI-RS | MCS.3A | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.4A |  |  |  |
| RC.9B FDD | FDD | 10 | 50 | - | Non CSI-RS, rank 1/2 | MCS.3 | 8 | 1 |  |
|  |  |  |  |  | Non CSI-RS, rank 3/4 | MCS.30 |  |  |  |
|  |  |  |  |  | 4 CSI-RS, rank 1/2 | MCS.29 |  |  |  |
|  |  |  |  |  | 4 CSI-RS, rank 3/4 | MCS.31 |  |  |  |
| RC.9 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.3 | 7 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.4 |  |  |  |
| RC.9B TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS, rank 1/2 | MCS.3 | 7 | 1 |  |
|  |  |  |  |  | Non CSI-RS, rank 3/4 | MCS.30 |  |  |  |
|  |  |  |  |  | 4 CSI-RS, rank 1/2 | MCS.29 |  |  |  |
|  |  |  |  |  | 4 CSI-RS, rank 3/4 | MCS.31 |  |  |  |
| 2 CRS Port + CSI-RS |  |  |  |  |  |  |  |  |  |
| RC.7 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.5 | 8 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.7 |  |  |  |
| RC.7 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.5 | 10 | 1 |  |
|  |  |  |  |  | 8 CSI-RS | MCS.8 |  |  |  |
| RC.11 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.5 | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.6 |  |  |  |
| RC.11 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.5 | 10 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.6 |  |  |  |
| RC.18 FDD | FDD | 10 | 6 | - | Non CSI-RS | MCS.13 | 8 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.19 |  |  |  |
| RC.18 TDD | TDD | 10 | 6 | Note 3 | Non CSI-RS | MCS.13 | 7 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.19 |  |  |  |
| RC.17 TDD | TDD | 10 | 6 | Note 3 | 4 ZP-CSI-RS | MCS.21 | 10 | 1 |  |
| RC.18 TDD | TDD | 10 | 6 | Note 3 | 4 ZP-CSI-RS | MCS.22 | 10 | 1 |  |
| RC.19 TDD | TDD | 10 | 41 | Note3 | 4 ZP-CSI-RS | MCS.23 | 10 | 1 | Note 11 |
| RC.20 TDD | TDD | 10 | 50 | Note3 | Non CSI-RS | MCS.24 | 10 | 1 |  |
|  |  |  |  |  | 2 CSI-RS, 4 ZP-CSI-RS | MCS.25 |  |  |  |
| RC.22 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.5 | 8 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.27 |  |  |  |
| RC.22 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.5 | 10 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.27 |  |  |  |
| RC.23 FDD | FDD | 10 | 50 |  | Non CSI-RS | MCS.9 | 8 | 1 | Rank 8 |
|  |  |  |  |  | 8 CSI-RS | MCS.32 |  |  |  |
| RC.23 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.9 | 10 | 1 | Rank 8 |
|  |  |  |  |  | 8 CSI-RS | MCS.32 |  |  |  |
| 1 CRS Port + CSI-RS + CSI-IM |  |  |  |  |  |  |  |  |  |
| RC.13 FDD | FDD | 10 | 50 | - | Non CSI-RS/IM | MCS.3 | 8 | 1 |  |
|  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |
| RC.13 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS/IM | MCS.3 | 10 | 1 |  |
|  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |
| 2 CRS Port + CSI-RS + CSI-IM |  |  |  |  |  |  |  |  |  |
| RC.10 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.5 | 8 | 1 |  |
|  |  |  |  |  | 4 CSI-RS, 1 CSI process | MCS.8 |  |  |  |
| RC.10 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.5 | 10 | 1 |  |
|  |  |  |  |  | 8 CSI-RS, 1 CSI process | MCS.9 |  |  |  |
| RC.12 FDD | FDD | 10 | 6 | - | Non CSI-RS/IM | MCS.13 | 8 | 1 |  |
|  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |
| RC.12 TDD | TDD | 10 | 6 | Note 3 | Non CSI-RS/IM | MCS.13 | 10 | 1 |  |
|  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |
| Note 1: 3 symbols allocated to PDCCH. Note 2: For FDD only subframes 1, 2, 3, 4, 6, 7, 8 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 3: TDD UL-DL configuration as specified in the individual tests. Note 4: For TDD when UL-DL configuration 1 is used only subframes 4 and 9 are allocated to avoide PBCH and synchronizaiton signal overhead. Note 5: For TDD when UL-DL configuration 2 is used only subframes 3, 4, 8, and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 6: Centered within the Transmission Bandwidth Configuration (Figure 5.6-1). Note 7: Only subframes 2, 3, 4, 7, 8 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 8: Allocate PDSCH on 5th and 6th PRBs within a subband. Note 9: The number of HARQ processes is 10 for TDD UL/DL configuration 2 and 7 for TDD UL/DL configuration 1. Note 10: The downlink subframes are scheduled at the 1st, 2nd, 8th, 9th, 16th, 17th, 18th, 24th, 26th, 32nd, 33rd, 34th subframes every 40ms. Information bit payload is available if downlink subframe is scheduled.(starting from 0th subframe) Note 11: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in subframe 0 and 5 in RC.19 TDD. Note 12: Allocate PDSCH on 3th, 4th and 5th PRBs within a narrowband. Allocate MPDCCH on the 0th and 1st PRBs within a narrowband. Note 13: The PDSCH subframes are scheduled at the 0th and 1st subframes every 10ms. Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe). MPDCCH subframes are scheduled at the 8th and 9th subframes every 10ms. Note 14: The downlink subframes are scheduled at the 0th to 4th subframes every 20ms. Information bit payload is scheduled at the 4th subframe (starting from 0th subframe). MPDCCH and Information bit payload are not scheduled in the radio frames where systemInformation1-BR is scheduled and = 4 with the set of frames and subframes for SIB1-BR defined in TS 36.211 [16] Table 6.4.1-2. Note 15: Information bit payload is scheduled at the 8th subframe every 20ms (starting from 0th subframe). |  |  |  |  |  |  |  |  |  |

Table A.4-1a: Void

Table A.4-1b: Void

Table A.4-1c: Void

Table A.4-1d: Void

Table A.4-1e: Void

Table A.4-2: Void

Table A.4-2a: Void

Table A.4-2b: Void

Table A.4-2c: Void

Table A.4-2d: Void

Table A.4-2e: Void

Table A.4-3: Void

Table A.4-3a: Void

Table A.4-3b: Void

Table A.4-3c: Void

Table A.4-3d: Void

Table A.4-3e: Void

Table A.4-3f: Void

Table A.4-3g: Void

Table A.4-3h: Void

Table A.4-3i: Void

Table A.4-3j: Void

Table A.4-3k: Void

Table A.4-3l: Void

Table A.4-3m: Void

Table A.4-4: Void

Table A.4-4a: Void

Table A.4-4b: Void

Table A.4-5: Void

Table A.4-5a: Void

Table A.4-5b: Void

Table A.4-6: Void

Table A.4-6a: Void

Table A.4-6b: Void

Table A.4-6c: Void

Table A.4-6d: Void

Table A.4-6e: Void

Table A.4-6f: Void

Table A.4-7: Void

Table A.4-8: Void

Table A.4-9: Void

Table A.4-10: Void

Table A.4-11: Void

Table A.4-12: Void

Table A.4-13: Mapping of CQI Index to Modulation coding scheme (MCS)

| CQI Index |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  |  | OOR | 0.0762 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6016 | 0.4551 | 0.5537 | 0.6504 | 0.7539 | 0.8525 | 0.9258 |  |
| Modulation |  |  |  | OOR | QPSK |  |  |  |  |  | 16QAM |  |  | 64QAM |  |  |  |  |  |  |
| MCS Scheme |  | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.1 |  | 50 | 6300 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.2 |  | 50 | 6000 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.2A |  | 100 | 12000 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.3 |  | 50 | 5700 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 10 | 13 | 15 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.3A |  | 100 | 11400 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.4 |  | 50 | 5600 | DTX | 0 | 0 | 2 | 4 | 6 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.4A |  | 100 | 11200 | DTX | 0 | 0 | 2 | 4 | 6 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.5 |  | 50 | 5400 | DTX | 0 | 0 | 2 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 24 | 25 |  |
| MCS.6 |  | 50 | 5300 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 22 | 24 | 25 |  |
| MCS.7 |  | 50 | 5200 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.8 |  | 50 | 5000 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 13 | 17 | 18 | 20 | 22 | 23 | 24 |  |
| MCS.9 |  | 50 | 4800 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 13 | 17 | 18 | 20 | 22 | 23 | 24 |  |
| MCS.10 |  | 6 | 756 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 19 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.11 |  | 6 | 684 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 14 | 17 | 20 | 21 | 23 | 25 | 27 |  |
| MCS.12 |  | 6 | 672 | DTX | 0 | 0 | 1 | 4 | 6 | 8 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.13 |  | 6 | 648 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 22 | 24 | 25 |  |
| MCS.14 |  | 25 | 3150 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.15 |  | 15 | 1890 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.16 |  | 15 | 1800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.17 |  | 3 | 378 | DTX | 0 | 1 | 2 | 5 | 7 | 9 | 12 | 13 | 16 | 19 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.18 |  | 50 | 5800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 17 | 20 | 22 | 23 | 26 | 27 |  |
| MCS.19 |  | 6 | 624 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.20 |  | 2 | 252 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 19 | 21 | 23 | 23 | 23 | 23 |  |
| MCS.21 |  | 6 | 696 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 21 | 24 | 25 | 27 |  |
| MCS.22 |  | 6 | 624 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 19 | 20 | 22 | 24 | 24 |  |
| MCS.23 |  | 41 | 4264 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 18 | 20 | 22 | 24 | 24 |  |
| MCS.24 |  | 50 | 5400 | DTX | 0 | 0 | 2 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 19 | 21 | 23 | 24 | 25 |  |
| MCS.25 |  | 50 | 5100 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 12 | 13 | 15 | 18 | 20 | 22 | 23 | 24 |  |
| MCS 26 |  | 50 | 5800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.27 | CW0 | 50 | 4600 | DTX | 0 | 0 | 1 | 3 | 5 | 6 | 10 | 11 | 13 | 17 | 18 | 19 | 21 | 23 | 23 |  |
|  | CW1 | 50 | 4600 | DTX | 0 | 0 | 1 | 3 | 5 | 6 | 10 | 11 | 13 | 17 | 18 | 19 | 21 | 22 | 23 |  |
| MCS 29 |  | 50 | 5500 | DTX | 0 | 0 | 2 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 19 | 21 | 23 | 24 | 25 |  |
| MCS.30 |  | 50 | 10200 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 12 | 14 | 15 | 18 | 20 | 22 | 23 | 24 |  |
| MCS.31 |  | 50 | 9800 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 11 | 13 | 14 | 18 | 20 | 21 | 23 | 23 |  |
| MCS.32 |  | 50 | 4400 | DTX | 0 | 0 | 1 | 3 | 4 | 6 | 10 | 11 | 12 | 17 | 18 | 19 | 20 | 22 | [23] | 4Layer1CW |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement except for [MCS.23]. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-14: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 1.9141 | 2.4063 | 2.7305 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.1A | 50 | 6300 | DTX | 0 | 1 | 3 | 5 | 7 | 10 | 11 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 26 |  |
| MCS.1B | 100 | 12600 | DTX | 0 | 1 | 3 | 5 | 7 | 10 | 11 | 14 | 15 | 18 | 20 | 22 | 24 | 26 | 26 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1A, 7.1.7.2.1-1 and 7.2.3-2 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-15: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 1.9141 | 2.4063 | 2.7305 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.11A | 6 | 684 | DTX | 0 | 1 | 3 | 5 | 7 | 8 | 10 | 13 | 14 | 16 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.12A | 6 | 672 | DTX | 0 | 1 | 3 | 5 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.11B | 8 | 912 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 10 | 13 | 14 | 16 | 18 | 19 | 22 | 24 | 26 |  |
| MCS.12B | 8 | 896 | DTX | 0 | 1 | 3 | 5 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 19 | 22 | 24 | 25 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1A, 7.1.7.2.1-1 and 7.2.3-2 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-16: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS indx Table 3)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  | OOR | 0.0391 | 0.0762 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6015 | Reserved | Reserved | Reserved | Reserved | Reserved |  |
| Modulation |  |  | OOR | QPSK |  |  |  |  |  |  | 16QAM |  |  |  |  |  |  |  |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.28 | 3 | 378 | DTX | 0 | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | N/A | N/A | N/A | N/A | N/A |  |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: startSymbolBR = 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>