3GPP TSG-RAN WG4 Meeting #89                                                                  R4-1816024

Spokane, Washington, USA, 12th – 16nd November, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.104 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft - Addition parameters about n75 & n76 in TS 37.104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition parameters about n75 & n76 in TS 37.104 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Paragraphs: 3.2 => update the symbols 4.5-1 => Update table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some additional technical precisions about Bands n75 & n76 will not be include in the TS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2; 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 37.113 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 37.141 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BWChannel	Channel bandwidth (for E-UTRA and NR)

BWConfig	Transmission bandwidth configuration (for E-UTRA), where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink. Transmission bandwidth configuration (for NR), where BWConfig = NRB x SCS x 12.

BWRF 	Base Station RF Bandwidth, where BWRF = FBW RF,high – FBW RF,low

BWRF,max 	Maximum Base Station RF Bandwidth

DwPTS	Downlink part of the special subframe (for E-UTRA TDD operation)

f	Frequency

f	Separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	The largest value of f used for defining the requirement

ΔfOBUE 	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

FC	Carrier centre frequency

Ffilter	Filter centre frequency

f_offset 	Separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter

f_offsetmax 	The maximum value of f_offset used for defining the requirement

Fblock,high	Upper sub-block edge, where Fblock,high = FC,block,high + Foffset, RAT

Fblock,low	Lower sub-block edge, where Fblock,low = FC,block,low - Foffset, RAT

FBW RF,high  	Upper Base Station RF Bandwidth edge, where FBW RF,high  = FC,high + Foffset, RAT

FBW RF,low  	Lower Base Station RF Bandwidth edge, where FBW RF,low  = FC,low - Foffset, RAT

FC band, high		Center frequency of the highest transmitted/received carrier in a band.

FC band, low		Center frequency of the lowest transmitted/received carrier in a band.

FC,block, high		Centre frequency of the highest transmitted/received carrier in a sub-block.

FC,block, low		Centre frequency of the lowest transmitted/received carrier in a sub-block.

FC,high		Centre frequency of the highest transmitted/received carrier.

FC,low		Centre frequency of the lowest transmitted/received carrier.

Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper Base Station RF Bandwidth edge, sub-block edge or Inter-RF Bandwidth edge, or from the centre frequency of the lowest transmitted/received to the lower Base Station RF Bandwidth edge, sub-block edge or Inter-RF Bandwidth edge for a specific RAT.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

PEM,N	Declared emission level for channel N

PEM,B32,B75,B76,ind	Declared emission level in Band 32, Band 75/n75 and Band 76/n76, ind=a, b, c

PEM,B32,ind	Declared emission level in Band 32, ind= d, e

PEM,B50,B74,B75,ind	Declared emission level for Band 50/n50, Band 74 and Band 75/n75, ind=a,b

Pmax	Maximum total output power

Pmax,RAT	Maximum RAT output power

Pmax,c	Maximum carrier output power

PRated,c	Rated carrier output power

PREFSENS		Reference Sensitivity power level

Wgap	Sub-block gap or Inter RF Bandwidth gap size

<Unchanged sections omitted>

Table 4.5-1: Paired bands in NR, E-UTRA, UTRA and GSM/EDGE.

| MSR and E-UTRA Band number |  | NR Band number |  | UTRA Band number |  | GSM/EDGE Band designation |  | Uplink (UL) BS receive UE transmit |  |  |  |  |  | Downlink (DL) BS transmit UE receive |  |  |  |  |  | Band category |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | n1 |  | I |  | - |  | 1920 MHz |  |  | – | 1980 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | 1 |  |
| 2 |  | n2 |  | II |  | PCS 1900 |  | 1850 MHz |  |  | – | 1910 MHz |  | 1930 MHz |  | – |  | 1990 MHz |  | 2 |  |
| 3 |  | n3 |  | III |  | DCS 1800 |  | 1710 MHz |  |  | – | 1785 MHz |  | 1805 MHz |  | – |  | 1880 MHz |  | 2 |  |
| 4 |  |  |  | IV |  | - |  | 1710 MHz |  |  | – | 1755 MHz |  | 2110 MHz |  | – |  | 2155 MHz |  | 1 |  |
| 5 |  | n5 |  | V |  | GSM 850 |  | 824 MHz |  |  | – | 849 MHz |  | 869 MHz |  | – |  | 894MHz |  | 2 |  |
| 6 (NOTE 1) |  |  |  | VI |  | - |  | 830 MHz |  |  | – | 840 MHz |  | 875 MHz |  | – |  | 885 MHz |  | 1 (NOTE 1) |  |
| 7 |  | n7 |  | VII |  | - |  | 2500 MHz |  |  | – | 2570 MHz |  | 2620 MHz |  | – |  | 2690 MHz |  | 1 (NOTE 3) |  |
| 8 |  | n8 |  | VIII |  | E-GSM |  | 880 MHz |  |  | – | 915 MHz |  | 925 MHz |  | – |  | 960 MHz |  | 2 |  |
| 9 |  |  |  | IX |  | - |  | 1749.9 MHz |  |  | – | 1784.9 MHz |  | 1844.9 MHz |  | – |  | 1879.9 MHz |  | 1 (NOTE 3) |  |
| 10 |  |  |  | X |  | - |  | 1710 MHz |  |  | – | 1770 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | 1 (NOTE 3) |  |
| 11 |  |  |  | XI |  | - |  | 1427.9 MHz |  |  | – | 1447.9 MHz |  | 1475.9 MHz |  | – |  | 1495.9 MHz |  | 1 |  |
| 12 |  | n12 |  | XII |  | - |  | 699 MHz |  |  | – | 716 MHz |  | 729 MHz |  | – |  | 746 MHz |  | 1 |  |
| 13 |  |  |  | XIII |  | - |  | 777 MHz |  |  | – | 787 MHz |  | 746 MHz |  | – |  | 756 MHz |  | 1 |  |
| 14 |  |  |  | XIV |  | - |  | 788 MHz |  |  | – | 798 MHz |  | 758 MHz |  | – |  | 768 MHz |  | 1 |  |
| 15 |  |  |  | XV |  | - |  | Reserved |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |
| 16 |  |  |  | XVI |  | - |  | Reserved |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |
| 17 |  |  |  | - |  | - |  | 704 MHz |  |  | – | 716 MHz |  | 734 MHz |  | – |  | 746 MHz |  | 1 (NOTE 4) |  |
| 18 |  |  |  | - |  | - |  | 815 MHz |  |  | – | 830 MHz |  | 860 MHz |  | – |  | 875 MHz |  | 1 (NOTE 4) |  |
| 19 |  |  |  | XIX |  | - |  | 830 MHz |  |  | – | 845 MHz |  | 875 MHz |  | – |  | 890 MHz |  | 1 |  |
| 20 |  | n20 |  | XX |  | - |  | 832 MHz |  |  | – | 862 MHz |  | 791 MHz |  | – |  | 821 MHz |  | 1 |  |
| 21 |  |  |  | XXI |  | - |  | 1447.9 MHz |  |  | – | 1462.9 MHz |  | 1495.9 MHz |  | – |  | 1510.9 MHz |  | 1 |  |
| 22 |  |  |  | XXII |  | - |  | 3410 MHz |  |  | – | 3490 MHz |  | 3510 MHz |  | – |  | 3590 MHz |  | 1 (NOTE 3) |  |
| 238 |  |  |  | - |  | - |  | 2000 MHz |  |  | – | 2020 MHz |  | 2180 MHz |  | – |  | 2200 MHz |  | 1 (NOTE 2) |  |
| 24 |  |  |  | - |  | - |  | 1626.5 MHz |  |  | – | 1660.5 MHz |  | 1525 MHz |  | – |  | 1559 MHz |  | 1 (NOTE 2) |  |
| 25 |  | n25 |  | XXV |  | - |  | 1850 MHz |  |  | – | 1915 MHz |  | 1930 MHz |  | – |  | 1995 MHz |  | 1 |  |
| 26 |  |  |  | XXVI |  | - |  | 814 MHz |  |  | – | 849 MHz |  | 859 MHz |  | – |  | 894 MHz |  | 1 |  |
| 27 |  |  |  | - |  | - |  | 807 MHz |  |  | – | 824 MHz |  | 852 MHz |  | – |  | 869 MHz |  | 1 (NOTE 2) |  |
| 28 |  | n28 |  | - |  | - |  | 703 MHz |  |  | – | 748 MHz |  | 758 MHz |  | – |  | 803 MHz |  | 1 (NOTE 4) |  |
| 29 |  |  |  | - |  | - |  | N/A |  |  |  |  |  | 717 MHz |  | – |  | 728 MHz |  | 1 (NOTE 2, NOTE 5) |  |
| 30 |  |  |  | - |  | - |  | 2305 MHz |  |  | – | 2315 MHz |  | 2350 MHz |  | – |  | 2360 MHz |  | 1 (NOTE 2) |  |
| 31 |  |  |  | - |  | - |  | 452.5 MHz |  |  | – | 457.5 MHz |  | 462.5 MHz |  | – |  | 467.5 MHz |  | 1 (NOTE 4) |  |
| 32 (NOTE 5) |  |  |  | XXXII (NOTE 6) |  | - |  |  | N/A |  |  |  |  | 1452 MHz |  | – |  | 1496 MHz |  | 1 (NOTE 3) |
| 64 |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |
| 65 |  |  |  | - |  | - |  | 1920 MHz | – |  |  | 2010 MHz |  | 2110 MHz |  | – |  | 2200 MHz |  | 1 (NOTE 2) |
| 66 (NOTE 7) |  | n66 |  | - |  | - |  | 1710 MHz | – |  |  | 1780 MHz |  | 2110 MHz |  | – |  | 2200 MHz |  | 1 (NOTE 4) |
| 67 (NOTE 5) |  |  |  | - |  | - |  |  | N/A |  |  |  |  | 738 MHz |  | - |  | 758 MHz |  | 1 (NOTE 2) |
| 68 |  |  |  | - |  | - |  | 698 MHz | – |  |  | 728 MHz |  | 753 MHz |  | – |  | 783 MHz |  | 1 (NOTE 2) |
| 69 |  |  |  | - |  | - |  | N/A |  |  |  |  |  | 2570 MHz |  | – |  | 2620 MHz |  | 1 (NOTE 2, NOTE 5) |
| 70 (NOTE 9) |  | n70 |  | - |  | - |  | 1695 MHz | – |  |  | 1710 MHz |  | 1995 MHz |  | – |  | 2020 MHz |  | 1 (NOTE 4) |
| 71 |  | n71 |  | - |  | - |  | 663 MHz | – |  |  | 698 MHz |  | 617 MHz |  | – |  | 652 MHz |  | 1 (NOTE 4) |
| 72 |  |  |  | - |  | - |  | 451 MHz | – |  |  | 456 MHz |  | 461 MHz |  | – |  | 466 MHz |  | 1 (NOTE 4) |
| 73 |  |  |  | - |  | - |  | 450 MHz | – |  |  | 455 MHz |  | 460 MHz |  | – |  | 465 MHz |  | 1 (NOTE 4) |
| 74 |  |  |  | - |  | - |  | 1427 MHz | – |  |  | 1470 MHz |  | 1475 MHz |  | – |  | 1518 MHz |  | 1 (NOTE 4) |
| 75 |  | n75 |  | - |  | - |  | N/A |  |  |  |  |  | 1432 MHz |  | – |  | 1517 MHz |  | 1 (NOTE 2, NOTE 5) |
| 76 |  | n76 |  | - |  | - |  | N/A |  |  |  |  |  | 1427 MHz |  | – |  | 1432 MHz |  | 1 (NOTE 2, NOTE 5) |
| 85 |  |  |  | - |  | - |  | 698 MHz | – |  |  | 716 MHz |  | 728 MHz |  | – |  | 746 MHz |  | 1 (NOTE 4) |
| NOTE 1: The band is for UTRA only. NOTE 2: The band is for E-UTRA and/or NR only. NOTE 3: The band is for NR, E-UTRA and/or UTRA only. NOTE 4: The band is for NR, E-UTRA and/or NB-IoT only. NOTE 5: Restricted to NR and/or E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 6: Restricted to UTRA operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequenc(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 7: The range 2180-2200 MHz of the DL operating band is restricted to NR and/or E-UTRA operation when carrier aggregation is configured. NOTE 8: Band 23 is not applicable. NOTE 9: The range 2010-2020 MHz of the DL operating band is restricted to NR and/or E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz. The range 2005-2020 MHz of the DL operating band is restricted to NR and/or E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of the change>