3GPP TSG-RAN WG4 Meeting #89                                                                  R4-1816028

Spokane, Washington, USA, 12th – 16nd November, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft - Addition parameters about n50 and n51 in TS 37.141 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Etisalat |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition parameters about n50 and n51 in TS 37.141 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Paragraphs: 3.2 Update the symbols in include bands n50 & n51 4.4 Update the table to include n50 & n51 4.6.3 Add n50 & n51 to the additions unwanted emissions 6.6.1.5.5 Update the table to include n50 & n51 6.6.1.5.6 Update the table to include n50 & n51 6.6.2.5.4.6 Add n50 & n51 to the additions unwanted emissions 7.5.5.2 Update the table to include n50 & n51 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some additional technical parameters about Bands n50 and n51 will not be include in the TS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2; 4.4; 4.6.3; 6.6.1.5.5; 6.6.1.5.6; 6.6.2.5.4.6; 7.5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 37.104 , TS 37.113 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Roll-off factor

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BWChannel	Channel bandwidth (for E-UTRA)

BWConfig	Transmission bandwidth configuration (for E-UTRA), expressed in MHz, where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink.

BWRF 	Base Station RF Bandwidth, where BWRF = FBW RF,high – FBW RF,low

BWRF,max 	Maximum Base Station RF Bandwidth

CA_X	CA for band X where X is the applicable E-UTRA operating band

CA_X-Y	CA for band X and Band Y where X and Y are the applicable E-UTRA operating band

DwPTS	Downlink part of the special subframe (for E-UTRA TDD operation

f	Frequency

f	Separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	The largest value of f used for defining the requirement

FC	Carrier centre frequency

Ffilter	Filter centre frequency

f_offset 	Separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter

f_offsetmax 	The maximum value of f_offset used for defining the requirement

Fblock,high	Upper sub-block edge, where Fblock,high = FC,block,high + Foffset, RAT

Fblock,low	Lower sub-block edge, where Fblock,low = FC,block,low - Foffset, RAT

FBW RF,high 	Upper Base Station RF Bandwidth edge, where FBW RF,high = FC,high + Foffset, RAT

FBW RF,low 	Lower Base Station RF Bandwidth edge, where FBW RF,low = FC,low - Foffset, RAT

FC band, high		Center frequency of the highest transmitted/received carrier in a band.

FC band, low		Center frequency of the lowest transmitted/received carrier in a band.

FC,block, high		Centre frequency of the highest transmitted/received carrier in a sub-block.

FC,block, low		Centre frequency of the lowest transmitted/received carrier in a sub-block.

FC,high		Centre frequency of the highest transmitted/received carrier.

FC,low		Centre frequency of the lowest transmitted/received carrier.

Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper Base Station RF Bandwidth edge, sub-block edge or Inter RF Bandwidth edge, or from the centre frequency of the lowest transmitted/received carrier to the lower Base Station RF Bandwidth edge, sub-block edge or Inter RF Bandwidth edge for a specific RAT.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

NRB	Transmission bandwidth configuration, expressed in units of resource blocks (for E-UTRA)

PEM,B32,B75,B76,ind	Declared emission level in Band 32, Band 75 and Band 76, ind=a, b, c

PEM,B32,ind	Declared emission level in Band 32, ind= d, e

PEM,B50,B74,B75,ind	Declared emission level for Band 50/n50, Band 74 and Band 75, ind=a,b

Pmax	Maximum total output power

Pmax,c	Maximum carrier output power

Pmax,RAT	Maximum RAT output power

PRated,c	Rated carrier output power

PREFSENS		Reference Sensitivity power level

Wgap	Sub-block gap size or Inter RF Bandwidth gap size

<Unchanged sections omitted>

Table 4.4-2: Unpaired bands in NR, E-UTRA and UTRA

| MSR and E-UTRA Band number |  | NR Band number | UTRA Band number | Uplink (UL) BS receive UE transmit |  |  | Downlink (DL) BS transmit UE receive |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 |  |  | a) | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | 3 |
| 34 |  |  | a) | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | 3 |
| 35 |  |  | b) | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | 3 |
| 36 |  |  | b) | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | 3 |
| 37 |  |  | c) | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | 3 |
| 38 |  |  | d) | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | 3 |
| 39 |  |  | f) | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | 3 |
| 40 |  |  | e) | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | 3 |
| 41 |  |  | - | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | 3 (NOTE 1) |
| 42 |  |  | - | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | 3 |
| 43 |  |  | - | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | 3 |
| 44 |  |  | - | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | 3 |
| 45 |  |  | - | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | 3 |
| 48 |  |  | - | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | 3 |
| 50 |  | n50 | - | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | 3 |
| 51 |  | n51 | - | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | 3 |
| 52 |  |  | - | 3300 MHz | – | 3400 MHz | 3300 MHz | – | 3400 MHz | 3 |
|  | NOTE 1: The band supports NB-IoT (in certain regions). |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

### 4.6.3	Additional operating band unwanted emissions

The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.2.4 of TS 37.104 [2] apply. If this is the case, conformance with the applicable emission limits shall be demonstrated.

NOTE: 	For the emission limits established by FCC Title 47 [8], there is no test method or requirement defined in the present specification.

For a BS declared to support Band 20 and to operate in geographic areas within the CEPT in which frequencies are allocated to broadcasting (DTT) service, the manufacturer shall additionally declare the following quantities associated with the applicable test conditions of Table 6.6.2.5.4.4-1 and information in annex G of TS 36.104 [5] :

PEM,N		Declared emission level for channel N

P10MHz		Maximum output Power in 10 MHz

Conformance with the declared emission level PEM,N shall be demonstrated.

For a BS declared to support Band 32, 75 or 76 and to operate in geographic areas within the CEPT, the manufacturer shall additionally declare the following quantities associated with the applicable test conditions of Table 6.6.2.5.4.6-1 and Table 6.6.2.5.4.6-2:

PEM,B32,B75,B76,ind	Declared emission level in Band 32, Band 75 and Band 76, ind=a, b, c

PEM,B32,ind	Declared emission level in Band 32, ind= d, e

Conformance with the declared emission level PEM,B32,B75,B76,ind and PEM,B32,ind shall be demonstrated.

For a BS declared to support Band 50/n50, 74 or 75 and to operate in geographic areas where the additional unwanted emission limit defined in Table 6.6.2.5.4.6-3 applies, the manufacturer shall additionally declare the following quantity associated with the applicable test conditions of Table 6.6.2.5.4.6-3:

PEM,B50,B74,B75,ind	Declared emission level for Band 50/n50, Band 74 and Band 75, ind=a,b

Conformance with the declared emission level PEM,B50,B74,B75,ind  shall be demonstrated.

### 4.6.4	Co-existence with other systems

<Unchanged sections omitted>

Table 6.6.1.5.5-1: BS Spurious emissions limits for co-existence with systems operating in other frequency bands

| System type to co-exist with |  | Frequency range for co-existence requirement |  | Maximum Level |  |  | Measurement Bandwidth |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 |  | 921 - 960 MHz |  | -57 dBm |  |  | 100 kHz |  | This requirement does not apply to BS operating in band 8 |  |
|  |  | 876 - 915 MHz |  | -61 dBm |  |  | 100 kHz |  | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band 8, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| DCS1800 (Note 3) |  | 1805 - 1880 MHz |  | -47 dBm |  |  | 100 kHz |  | This requirement does not apply to BS operating in band 3. |  |
|  |  | 1710 - 1785 MHz |  | -61 dBm |  |  | 100 kHz |  | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| PCS1900 |  | 1930 - 1990 MHz |  | -47 dBm |  |  | 100 kHz |  | This requirement does not apply to BS operating in band 2, 25, band 36 or band 70. |  |
|  |  | 1850 - 1910 MHz |  | -61 dBm |  |  | 100 kHz |  | This requirement does not apply to BS operating in band 2 or 25, since it is already covered by the requirement in sub-clause 6.6.1.5.4. This requirement does not apply to BS operating in band 35. |  |
| GSM850 or CDMA850 |  | 869 - 894 MHz |  | -57 dBm |  |  | 100 kHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 824 - 849 MHz |  | -61 dBm |  |  | 100 kHz |  | This requirement does not apply to BS operating in band 5 or 26, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band I or E-UTRA Band 1 |  | 2110 - 2170 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65 . |  |
|  |  | 1920 - 1980 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band II or E-UTRA Band 2 |  | 1930 - 1990 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 2, 25 or 70. |  |
|  |  | 1850 - 1910 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 2 or 25, since it is already covered by the requirement in sub-clause 6.6.1.5.4 |  |
| UTRA FDD Band III or E-UTRA Band 3 (Note 3) |  | 1805 - 1880 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9. |  |
|  |  | 1710 - 1785 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band IV or E-UTRA Band 4 |  | 2110 - 2155 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10 or 66. |  |
|  |  | 1710 - 1755 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10 or 66, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band V or E-UTRA Band 5 |  | 869 - 894 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 824 - 849 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 |  | 860 - 890 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 6, 18, 19 |  |
|  |  | 815 - 830 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 18 since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
|  |  | 830 - 845 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 6, 19, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band VII or E-UTRA Band 7 |  | 2620 - 2690 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 7. |  |
|  |  | 2500 - 2570 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 7, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band VIII or E-UTRA Band 8 |  | 925 - 960 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 8. |  |
|  |  | 880 - 915 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band IX or E-UTRA Band 9 |  | 1844.9 - 1879.9 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9. |  |
|  |  | 1749.9 - 1784.9 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band X or E-UTRA Band 10 |  | 2110 - 2170 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10 or 66. |  |
|  |  | 1710 - 1770 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 10 or 66, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 |  | 1475.9 - 1510.9 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, 21, 32, 50/n50, 74 or 75. |  |
|  |  | 1427.9 - 1447.9 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 11 or 74, since it is already covered by the requirement in sub-clause 6.6.1.5.4. This requirement does not apply to BS operating in band 32, 50/n50, 51/n51, 75 or 76. |  |
|  |  | 1447.9 – 1462.9 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 21 or 74, since it is already covered by the requirement in sub-clause 6.6.1.5.4. This requirement does not apply to BS operating in band 32, 50/n50 or 75. |  |
| UTRA FDD Band XII or E-UTRA Band 12 |  | 729 - 746 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 12 or 85. |  |
|  |  | 699 - 716 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 12 or 85, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |  |
| UTRA FDD Band XIII or E-UTRA Band 13 |  | 746 - 756 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 13. |  |
|  |  | 777 - 787 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 13, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band XIV or E-UTRA Band 14 |  | 758 - 768 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 14. |  |
|  |  | 788 - 798 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 14, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| E-UTRA Band 17 |  | 734 - 746 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 17. |  |
|  |  | 704 - 716 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 17, since it is already covered by the requirement in subclause 6.6.1.5.4. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |  |
| UTRA FDD Band XX or E-UTRA Band 20 |  | 791 - 821 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 20 or 28. |  |
|  |  | 832 - 862 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in subclause 6.6.1.5.4. |  |
| UTRA FDD Band XXII or E-UTRA Band 22 |  | 3510 – 3590 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 22, 42, 48, or 49. |  |
|  |  | 3410 – 3490 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 22, since it is already covered by the requirement in subclause 6.6.1.5.4. This requirement does not apply to Band 42 |  |
| E-UTRA Band 24 |  | 1525 – 1559 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 24. |  |
|  |  | 1626.5 – 1660.5 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 24, since it is already covered by the requirement in subclause 6.6.1.5.4. |  |
| UTRA FDD Band XXV or E-UTRA Band 25 |  | 1930 - 1995 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 2, 25 or 70. |  |
|  |  | 1850 - 1915 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 25, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 6.6.1.5.4. |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 |  | 859 - 894 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 814 - 849 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 26, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in sub-clause 6.6.1.5.4. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| E-UTRA Band 27 |  | 852 – 869 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 5, 26 or 27. |  |
|  |  | 807 – 824 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 27, since it is already covered by the requirement in subclause 6.6.1.5.4. For BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in sub-clause 6.6.1.5.4. This requirement also applies to BS operating in Band 28, starting 4 MHz above the Band 28 downlink operating band (Note 6). |  |
| E-UTRA Band 28 |  | 758 - 803 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, 28, 44 or 67. |  |
|  |  | 703 - 748 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 28, since it is already covered by the requirement in sub-clause 6.6.1.5.4. This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733MHz. |  |
| E-UTRA Band 29 |  | 717 – 728 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 29 or 85. |  |
| E-UTRA Band 30 |  | 2350 - 2360 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 30 or 40. |  |
|  |  | 2305 - 2315 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 30, since it is already covered by the requirement in sub-clause 6.6.1.5.4. This requirement does not apply to BS operating in Band 40. |  |
| E-UTRA Band 31 |  | 462.5 – 467.5 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  |  | 452.5 – 457.5 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, since it is already covered by the requirement in sub-clause 6.6.1.5.4. This requirement does not apply to BS operating in band 72 or 73. |  |
| UTRA FDD Band XXXII or E-UTRA Band 32 |  | 1452 - 1496 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, 21, 32, 50/n50, 74 or 75. |  |
| UTRA TDD Band a) or E-UTRA Band 33 |  | 1900 - 1920 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 33 |  |
| UTRA TDD Band a) or E-UTRA Band 34 |  | 2010 - 2025 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 34 |  |
| UTRA TDD Band b) or E-UTRA Band 35 |  | 1850 – 1910 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 35 |  |
| UTRA TDD Band b) or E-UTRA Band 36 |  | 1930 - 1990 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 2, 25 or 36 |  |
| UTRA TDD in Band c) or E-UTRA Band 37 |  | 1910 - 1930 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |  |
| UTRA TDD Band d) or E-UTRA Band 38 |  | 2570 – 2620 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 38 or 69. |  |
| UTRA TDD Band f) or E-UTRA Band 39 |  | 1880 – 1920MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 39 |  |
| UTRA TDD Band e) or E-UTRA Band 40 |  | 2300 – 2400MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 30 or 40 |  |
| E-UTRA Band 41 |  | 2496 – 2690MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 41 |  |
| E-UTRA Band 42 |  | 3400 – 3600 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42, 43, 48, 49 or 52. |  |
| E-UTRA Band 43 |  | 3600 – 3800 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48 or 49. |  |
| E-UTRA Band 44 |  | 703 - 803 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 28 or 44 |  |
| E-UTRA Band 45 |  | 1447 - 1467 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 45 |  |
| E-UTRA Band 46 |  | 5150 - 5925 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 46 |  |
| E-UTRA Band 47 |  | 5855 - 5925 MHz |  | -52 dBm |  |  | 1 MHz |  |  |  |
| E-UTRA Band 48 |  | 3550 – 3700 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42, 43, 48 or 49 |  |
| E-UTRA Band 49 |  | 3550 – 3700 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42, 43, 48 or 49 |  |
| E-UTRA Band 50 or NR band n50 |  | 1432 - 1517 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in Band 11, 21, 32, 45, 50/n50, 51/n51, 74, 75 or 76. |  |
| E-UTRA Band 51 or NR band n51 |  | 1427 - 1432 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in Band 50/n50, 51/n51, 75 or 76. |  |
| E-UTRA Band 52 |  | 3300 – 3400 MHz |  | -52 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 42 or 52. |  |
| E-UTRA Band 65 |  | 2110 - 2200 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, |  |
|  |  | 1920 - 2010 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 65, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in sub-clause 6.6.1.5.4. |  |
| E-UTRA Band 66 |  | 2110 - 2200 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10, 23 or 66. |  |
|  |  | 1710 - 1780 MHz |  | -49 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 66, since it is already covered by the requirement in clause 6.6.1.5.4. For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in clause 6.6.1.5.4. For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in clause 6.6.1.5.4. |  |
| E-UTRA Band 67 |  | 738 – 758 MHz |  | -52 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 28 or 67. |  |
| E-UTRA Band 68 |  | 753 -783 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in band 28, or 68. |  |
|  |  | 698-728 MHz |  | -49 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in band 68, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For E-UTRA BS operating in Band 28, it applies between 698 MHz and 703 MHz, while the rest is covered in sub-clause 6.6.1.5.4. |  |
| E-UTRA Band 69 |  | 2570 - 2620 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in Band 38 or 69. |  |
| E-UTRA Band 70 |  | 1995 - 2020 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in band 2, 25 or 70 |  |
|  |  | 1695 – 1710 MHz |  | -49 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in band 70, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| E-UTRA Band 71 |  | 617 - 652 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in band 71. |  |
|  |  | 663 – 698 MHz |  | -49 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in band 71, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| E-UTRA Band 72 |  | 461 - 466 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  |  | 451 - 456 MHz |  | -49 dBm |  | 1 MHz |  |  | This requirement does not apply to BS operating in band 72, since it is already covered by the requirement in sub-clause 6.6.1.5.4. This requirement does not apply to BS operating in band 73. |  |
| E-UTRA Band 73 |  | 460 - 465 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  |  | 450 - 455 MHz |  | -49 dBm |  | 1 MHz |  |  | This requirement does not apply to BS operating in band 73, since it is already covered by the requirement in sub-clause 6.6.1.5.4. |  |
| E-UTRA Band 74 |  | 1475 – 1518 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in band 11, 21, 32, 50/n50, 74 or 75. |  |
|  |  | 1427 – 1470 MHz |  | -49 dBm |  | 1MHz |  |  | This requirement does not apply to E-UTRA BS operating in Band 74, since it is already covered by the requirement in sub-clause 6.6.1.5.4. This requirement does not apply to BS operating in band 32, 45, 50/n50, 51/n51, 75 or 76. |  |
| E-UTRA Band 75 |  | 1432 - 1517 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in Band 11, 21, 32, 45, 50/n50, 51/n51, 74, 75 or 76. |  |
| E-UTRA Band 76 |  | 1427 - 1432 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to E-UTRA BS operating in Band 50/n50, 51/n51, 75 or 76. |  |
| E-UTRA Band 85 |  | 728 - 746 MHz |  | -52 dBm |  | 1 MHz |  |  | This requirement does not apply to BS operating in band 12, 29 or 85. |  |
|  |  | 698 - 716 MHz |  | -49 dBm |  | 1 MHz |  |  | This requirement does not apply to BS operating in band 85, since it is already covered by the requirement in sub-clause 6.6.1.5.4. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |  |
| NOTE 5: Void |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

Table 6.6.1.5.6-1: BS Spurious emissions limits for BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level (WA BS) | Maximum Level (MR BS) | Maximum Level (LA BS) | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876-915 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| DCS1800 | 1710 - 1785 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| PCS1900 | 1850 - 1910 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 - 849 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 | 1920 - 1980 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 | 1850 - 1910 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 | 1710 - 1785 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 | 824 - 849 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 845 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 | 2500 - 2570 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VIII or E-UTRA Band 8 | 880 - 915 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50/n50, 51/n51, 75 or 76 |
| UTRA FDD Band XII or E-UTRA Band 12 | 699 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 18 | 815 - 830 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 | 832 - 862 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 – 1462.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 32, 50/n50 or 75 |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42 |
| E-UTRA Band 23 | 2000 - 2020 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1850 - 1915 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814 - 849 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 27 | 807 - 824 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 28 | 703 – 748 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 44 |
| E-UTRA Band 30 | 2305 - 2315 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 40 |
| E-UTRA Band 31 | 452.5 – 457.5 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 2 and 36 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 | 2570 – 2620 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 38. |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880 – 1920MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 33 and 39 |
| UTRA TDD Band e) or E-UTRA Band 40 | 2300 – 2400MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 30 or 40 |
| E-UTRA Band 41 | 2496 – 2690MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 41 |
| E-UTRA Band 42 | 3400 – 3600 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 22, 42, 43, 48, 49 or 52. |
| E-UTRA Band 43 | 3600 – 3800 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48 or 49. |
| E-UTRA Band 44 | 703 – 803 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 28 or 44 |
| E-UTRA Band 45 | 1447 – 1467 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 45 |
| E-UTRA Band 46 | 5150 – 5925 MHz | N/A | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 46 |
| E-UTRA Band 48 | 3550 – 3700 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48 or 49 |
| E-UTRA Band 49 | 3550 – 3700 MHz | N/A | N/A | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48 or 49 |
| E-UTRA Band 50 | 1432 – 1517 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 11, 21, 32, 51/n51, 74, 75 or 76 |
| E-UTRA Band 51 | 1427 – 1432 MHz | N/A | N/A | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50/n50, 75 or 76 |
| E-UTRA Band 52 | 3300 – 3400 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42 or 52. |
| E-UTRA Band 65 | 1920 - 2010 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 66 | 1710 – 1780 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 68 | 698 – 728 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 70 | 1695 – 1710 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 71 | 663 – 698 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 72 | 451 – 456 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 73 | 450 – 455 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 74 | 1427 – 1470 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicabe to E-UTRA BS operating in Band 50 or 51 |
| E-UTRA Band 85 | 698 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

<Unchanged sections omitted>

6.6.2.5.4.6	Additional band 32, 50/n50, 51/n51, 74, 75 and 76 unwanted emissions

In certain regions, the following requirements may apply to BS operating in Band 32 within 1452-1492 MHz, in Band 75 within 1432-1517 MHz and in Band 76 within 1427-1432 MHz. The level of operating band unwanted emissions, measured on centre frequencies f_offset with filter bandwidth, according to Table 6.6.2.5.4.6-1, shall neither exceed the maximum emission level PEM,B32,B75,B76,a ,  PEM,B32,B75,B76,b nor PEM,B32,B75,B76,c declared by the manufacturer.

<Unchanged sections omitted>

In certain regions, the following requirement may apply to BS operating in Band 50/n50 and Band 75 within 1492-1517 MHz and in Band 74 within 1492-1518 MHz. The level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to Table 6.6.2.5.4.6-3, shall neither exceed the maximum emission level PEM,B50,B74,B75,a nor PEM,B50,B74,B75,b  declared by the manufacturer.

Table 6.6.2.5.4.6-3: Operating band 50/n50, 74 and 75 declared emission above 1518 MHz

| Filter centre frequency, Ffilter | Declared emission level [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 1518.5 MHz ≤ Ffilter ≤ 1519.5 MHz | PEM,B50,B74,B75,a | 1 MHz |
| 1520.5 MHz ≤ Ffilter ≤ 1558.5 MHz | PEM,B50,B74,B75,b | 1 MHz |

NOTE:	The regional requirement, included in [25], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in Annex H.

In certain regions, the following requirement may apply to E-UTRA BS operating in Band 50/n50 and Band 75 within 1432-1452 MHz, and in Band 51/n51 and Band 76. Emissions shall not exceed the maximum levels specified in Table 6.6.2.5.4.6-4.

Table 6.6.2.5.4.6-4: Additional operating band unwanted emission limits for BS operating in Band 50/n50 and 75 within 1432-1452 MHz, and in Band 51/n51 and 76

| Filter centre frequency, Ffilter | Maximum Level [dBm] | Measurement Bandwidth |
| --- | --- | --- |
| Ffilter = 1413.5 MHz | -42 | 27 MHz |

###### 6.6.2.5.4.7	Additional requirements for band 48

<Unchanged sections omitted>

Table 7.5.5.2-1: Blocking requirement for co-location with BS in other frequency bands

| Type of co-located BS |  |  | Centre Frequency of Interfering Signal (MHz) |  |  | Interfering Signal mean power for WA BS (dBm) |  | Interfering Signal mean power for MR BS (dBm) |  |  | Interfering Signal mean power for LA BS (dBm) |  |  | Wanted Signal mean power (dBm) |  |  |  |  | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 |  |  | 869 – 894 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| GSM900 |  |  | 921 – 960 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| DCS1800 |  |  | 1805 – 1880 (Note 4) |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| PCS1900 |  |  | 1930 – 1990 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band I or E-UTRA Band 1 |  |  | 2110 – 2170 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band II or E-UTRA Band 2 |  |  | 1930 – 1990 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band III or E-UTRA Band 3 |  |  | 1805 – 1880 (Note 4) |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band IV or E-UTRA Band 4 |  |  | 2110 – 2155 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band V or E-UTRA Band 5 |  |  | 869 – 894 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band VI or E-UTRA Band 6 |  |  | 875 – 885 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band VII or E-UTRA Band 7 |  |  | 2620 – 2690 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band VIII or E-UTRA Band 8 |  |  | 925 – 960 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band IX or E-UTRA Band 9 |  |  | 1844.9 – 1879.9 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band X or E-UTRA Band 10 |  |  | 2110 – 2170 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XI or E-UTRA Band 11 |  |  | 1475.9 - 1495.9 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XII or E-UTRA Band 12 |  |  | 729 - 746 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XIIII or E-UTRA Band 13 |  |  | 746 - 756 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XIV or E-UTRA Band 14 |  |  | 758 - 768 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 17 |  |  | 734 - 746 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 18 |  |  | 860 - 875 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XIX or E-UTRA Band 19 |  |  | 875 - 890 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XX or E-UTRA Band 20 |  |  | 791 - 821 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XXI or E-UTRA Band 21 |  |  | 1495.9 – 1510.9 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XXII or E-UTRA Band 22 |  |  | 3510 – 3590 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 23 |  |  | 2180 - 2200 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 24 |  |  | 1525 – 1559 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XXV or E-UTRA Band 25 |  |  | 1930 – 1995 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XXVI or E-UTRA Band 26 |  |  | 859 – 894 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 27 |  |  | 852 - 869 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 28 |  |  | 758 – 803 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 29 | 717-728 |  | +16** |  |  | +8** |  |  | -6** |  |  | PREFSENS + 6dB* |  |  | CW carrier |  |  |  |
| E-UTRA Band 30 |  |  | 2350-2360 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 31 |  |  | 462.5–467.5 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + 6dB* |  |  |  |  | CW carrier |
| UTRA FDD Band XXXII or E-UTRA Band 32 |  |  | 1452 – 1496 (NOTE 5) |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + 6dB* |  |  |  |  | CW carrier |
| UTRA TDD Band a) or E-UTRA Band 33 |  |  | 1900-1920 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA TDD Band a) or E-UTRA Band 34 |  |  | 2010-2025 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA TDD Band b) or E-UTRA Band 35 |  |  | 1850-1910 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA TDD Band b) or E-UTRA Band 36 |  |  | 1930-1990 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA TDD Band c) or E-UTRA Band 37 |  |  | 1910-1930 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA TDD Band d) or E-UTRA Band 38 |  |  | 2570-2620 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA TDD Band f) or E-UTRA Band 39 |  |  | 1880-1920 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| UTRA TDD Band e) or E-UTRA Band 40 |  |  | 2300-2400 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 41 |  |  | 2496 - 2690 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 42 |  |  | 3400 – 3600 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 43 |  |  | 3600 – 3800 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 44 |  |  | 703 - 803 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 45 |  |  | 1447 - 1467 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 46 |  |  | 5150 - 5925 |  |  | N/A |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 48 |  |  | 3550 - 3700 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 49 |  |  | 3550 - 3700 |  |  | N/A |  | N/A |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 50 or NR band n50 |  |  | 1432 - 1517 |  |  | +16 |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 51 or NR band n51 |  |  | 1427 - 1432 |  |  | N/A |  | N/A |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 52 |  |  | 3300 – 3400 |  |  | +16** |  | +8 |  |  | -6 |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 65 |  |  | 2110 – 2200 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 66 |  |  | 2110 – 2200 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 67 |  |  | 738 – 758 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 68 |  |  | 753 – 783 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 69 |  |  | 2570-2620 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 70 |  |  | 1995 - 2020 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 71 |  |  | 617 - 652 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 72 |  |  | 461 - 466 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 73 |  |  | 460 - 465 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 74 |  |  | 1475 - 1518 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 75 |  |  | 1432 - 1517 |  |  | +16** |  | +8** |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 76 |  |  | 1427 - 1432 |  |  | N/A |  | N/A |  |  | -6** |  |  | PREFSENS + x dB* |  |  |  |  | CW carrier |
| E-UTRA Band 85 |  |  | 728 - 746 |  |  |  | +16** |  |  | +8 |  |  | -6 |  |  | PREFSENS + x dB* |  |  | CW carrier |
| NOTE 1 (*):PREFSENS depends on the RAT, the BS class and the channel bandwidth, see subclause 7.2. "x" is equal to 3 in case of GSM/EDGE wanted signal and equal to 6 in case of UTRA or E-UTRA wanted signals. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797MHz.. NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [14]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805-1850MHz. NOTE 5: For a BS operating in band 11, 21 or 74, the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 6: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 7 (**): For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

<End of the change>