3GPP TSG-RAN WG4 Meeting #89	R4-1816076

# Spokane, USA, November 12th – 16th 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6102 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR introducing in-sync RLM test for NB-IoT TDD in enhanced coverage without DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of In-sync RLM test for NB-IoT TDD in enhanced coverage without DRX |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of In-sync RLM test for NB-IoT TDD in enhanced coverage without DRX |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No In-sync RLM test for NB-IoT TDD in enhanced coverage without DRX |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.7.3.93 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage

#### A.7.3.93.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in enhanced coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.93.1-1, A.7.3.93.1-2, A.7.3.93.1-3, A.7.3.93.1-4 and A.7.3.93.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.93.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is disabled, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2, dT and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.93.1-1: General test parameters for TDD in-sync test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 4 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 16 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 1.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.93.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.14 NB-TDD |  |  |  |  |
| NPDSCH_RA |  | -3 |  |  |  |  |
| NPDSCH_RB |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.93.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -6.3 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.93.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.93.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 0 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.93.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests without DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |

Figure A.7.3.93.1-1: SNR variation for in-sync testing without DRX

#### A.7.3.93.2	Test Requirements

The UE behavior in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.