3GPP TSG-RAN WG4 Meeting #89	R4-1816112

# Spokane, USA, November 12th – 16th 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to section 9.3 NR inter-frequency measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections to section 9.2 NR inter-frequency measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removal of Editor’s note - removal of [] - Update of references - Removal of unecessary rows in tables - section 9.3.7 is Void |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-frequency core requirements are unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

## 9.3	NR inter-frequency measurements

### 9.3.1	Introduction

A measurement is defined as a SSB based inter-frequency measurement provided it is not defined as in intra-frequency measurement according to section 9.2.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-frequency cells if carrier frequency information is provided by PCell or the PSCell, even if no explicit neighbour list with physical layer cell identities is provided.

SSB based measurements are configured along with a measurement timing configuration (SMTC) per carrier, which provides periodicity, duration and offset information on a window of up to 5ms where the measurements on the configured inter-frequency carrier are to be performed. For inter-frequency connected mode measurements, one measurement window periodicity may be configured per inter-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE in EN-DC and SA, or the serving cells are in FR2, the inter-frequency cells are in FR2 and the per-UE gap is configured to the UE in SA, the switching time is 0,25ms Otherwise the switching time is 0.5ms.

### 9.3.2	Requirements applicability

The requirements in Section 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in Sections 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in Sections 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in Sections 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

#### 9.3.2.1	Void

#### 9.3.2.2	Void

### 9.3.3	Number of cells and number of SSB

#### 9.3.3.1	Requirements for FR1

For each inter-frequency layer, the UE shall be capable of monitoring at least 4 cells.

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of monitoring at least 7 SSBs with different SSB index and/or PCI on the inter-frequency layer.

#### 9.3.3.2	Requirements for FR2

For each inter-frequency layer, the UE shall be capable of monitoring at least 4 cells.

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of monitoring at least 10 SSBs with different SSB index and/or PCI on the inter-frequency layer. The UE shall be capable of monitoring at least one SSB per cell.

### 9.3.4	Inter frequency cell identification

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within TSSB_time_index_inter.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1 and table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3 and table 9.3.4-4.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1 and table 9.3.5-2.

Mpss/sss_sync_inter : For a UE supporting power class 1, Mpss/sss_sync_inter=[64] samples. For a UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[40] samples. For a UE supporting power class 3 (handheld), Mpss/sss_sync_inter=[40] samples. For a UE supporting power class 4, Mpss/sss_sync=[TBD] samples.

MSSB_index_inter: For a UE supporting power class 1, MSSB_index_inter =[TBD] samples. For a vehicle mounted UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[TBD] samples.For a UE supporting power class 3 (handheld), MSSB_index_inter =[TBD] samples. For a UE supporting power class 4, Mmeas_period_inter =[TBD] samples.

Mmeas_period_inter: For a UE supporting power class 1, Mmeas_period_inter =[64] samples. For a vehicle mounted UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[40] samples.For a UE supporting power class 3 (handheld), Mmeas_period_inter =[40] samples. For a UE supporting power class 4, Mmeas_period_inter =[TBD] samples.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_i in section 9.1.5.2 for measurement conducted within measurement gaps.

# Table 9.3.4-1: Time period for PSS/SSS detection, (Frequency range FR1)

| Condition | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max[ 600ms, [8] x max(MGRP, SMTC period)x CSSFinter] Note 1, |
| DRX cycle ≤ 320ms | max[ 600ms, ceil(8x1.5) x max(MGRP, SMTC period, DRX cycle)x CSSFinter] Note 1, |
| DRX cycle > 320ms | [8] x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified |  |

Table 9.3.4-2: Time period for PSS/SSS detection, (Frequency range FR2)

| Condition | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max[ 600ms, Mpss/sss_sync_inter x max(MGRP, SMTC period)x CSSFinter] Note 1, |
| DRX cycle ≤ 320ms | max[ 600ms, Mpss/sss_sync_inter x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > 320ms | Mpss/sss_sync_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified |  |

Table 9.3.4-3: Time period for time index detection (Frequency range FR1)

| Condition | TSSB_time_index_inter |
| --- | --- |
| No DRX | max[ 120ms, [3] x max(MGRP, SMTC period)x CSSFinter ] Note 1, |
| DRX cycle ≤ 320ms | max[ 120ms, ceil(3 x 1.5) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > 320ms | [3] x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified |  |

Table 9.3.4-4: Time period for time index detection (Frequency range FR2)

| Condition | TSSB_time_index_inter |
| --- | --- |
| No DRX | max[ 200ms, MSSB_index_inter x max(MGRP, SMTC period)x CSSFinter] Note 1, |
| DRX cycle ≤ 320ms | max[ 200ms, MSSB_index_inter x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > 320ms | MSSB_index_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified |  |

#### 9.3.4.1	Void

#### 9.3.4.2	Void

### 9.3.5	Inter frequency measurements

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively,  as shown in table 9.3.5-1 and 9.3.5-2:

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max[ 200ms, [8] x max(MGRP, SMTC period)x CSSFinter] Note 1, |
| DRX cycle ≤ 320ms | max[ 200ms, ceil(8 x 1.5) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > 320ms | [8] x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being measured |  |

Table 9.3.5-2: Measurement period for inter-frequency measurements with gaps (Frequency FR2)

| Condition | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max[ 400ms, Mmeas_period_inter x max(MGRP, SMTC period) x CSSFinter] Note 1, |
| DRX cycle ≤ 320ms | max[ 400ms, Mmeas_period_inter x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > 320ms | Mmeas_period_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being measured |  |

#### 9.3.5.1	Void

#### 9.3.5.2	Void

#### 9.3.5.3	Void

### 9.3.6	NR Inter frequency measurements reporting requirements

#### 9.3.6.1	Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodically triggered measurement reports shall meet the requirements in sections 10.1.4.1, 10.1.5.1, 10.1.9.1, 10.1.10.1, 10.1.14.1 and 10.1.15.1, respectively.

#### 9.3.6.2	Event-triggered Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered periodic measurement reports shall meet the requirements in sections 10.1.4.1, 10.1.5.1, 10.1.9.1, 10.1.10.1, 10.1.14.1 and 10.1.15.1, respectively.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.3.6.3.

#### 9.3.6.3	Event-triggered Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered measurement reports shall meet the requirements in sections 10.1.4.1, 10.1.5.1, 10.1.9.1, 10.1.10.1, 10.1.14.1 and 10.1.15.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. Both Tidentify_inter_without_index and Tidentify_inter_with_index are defined in clause 9.3.4. When L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_inter_without_index or Tidentify_inter_with_index defined in clause 9.3.4 and then triggers the measurement report as per TS 38.331 [2], the event triggered measurement reporting delay shall be less than T SSB_measurement_period_inter  defined in clause 9.3.5 provided the timing to that cell has not changed more than  3200 Tc while measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used an additional delay can be expected.

### 9.3.7	Void

Next Modified Subclause