3GPP TSG-RAN WG4 Meeting #89	R4-1816114

# Spokane, WA, US, November 12th–16th, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for SSB-based inter-frequency measurements (section 9.3.4 and 9.3.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Align the notation of carrier-specific scaling factor - Correct the required samples in FR2 - Inter-freq. can’t be configured with dual SMTC periodicities |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align the notation CSSFwithin_gap_i to CSSFwithin_gap,i - Add 1.5 relxation back when DRX cycle ≤ [320]ms - Delete the note “If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements on inter-frequency measurements requirements will be unclarified in TS 38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.3.4, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change ------------------------------------------------------------

9.3.4	Inter frequency cell identification

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within TSSB_time_index_inter.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1 and table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3 and table 9.3.4-4.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1 and table 9.3.5-2.

Mpss/sss_sync_inter : For a UE supporting power class 1, Mpss/sss_sync_inter=[64] samples. For a UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[40] samples. For a UE supporting power class 3 (handheld), Mpss/sss_sync_inter=[40] samples. For a UE supporting power class 4, Mpss/sss_sync=[TBD] samples.

MSSB_index_inter: For a UE supporting power class 1, MSSB_index_inter =[TBD] samples. For a vehicle mounted UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[TBD] samples.For a UE supporting power class 3 (handheld), MSSB_index_inter =[TBD] samples. For a UE supporting power class 4, Mmeas_period_inter =[TBD] samples.

Mmeas_period_inter: For a UE supporting power class 1, Mmeas_period_inter =[64] samples. For a vehicle mounted UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[40] samples.For a UE supporting power class 3 (handheld), Mmeas_period_inter =[40] samples. For a UE supporting power class 4, Mmeas_period_inter =[TBD] samples.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in section 9.1.5.2 for measurement conducted within measurement gaps.

# Table 9.3.4-1: Time period for PSS/SSS detection, (Frequency range FR1)

| Condition | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max[ 600ms, [8] x max(MGRP, SMTC period)x CSSFinter] |
| DRX cycle ≤ [320]ms | max[ 600ms, ceil(8x1.5) x max(MGRP, SMTC period, DRX cycle)x CSSFinter] |
| DRX cycle > [320]ms | [8] x DRX cycle x CSSFinter |

Table 9.3.4-2: Time period for PSS/SSS detection, (Frequency range FR2)

| Condition | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max[ 600ms, Mpss/sss_sync_inter x max(MGRP, SMTC period)x CSSFinter] |
| DRX cycle ≤ [320]ms | max[ 600ms, (1.5 x Mpss/sss_sync_inter) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] |
| DRX cycle > [320]ms | Mpss/sss_sync_inter x DRX cycle x CSSFinter |

Table 9.3.4-3: Time period for time index detection (Frequency range FR1)

| Condition | TSSB_time_index_inter |
| --- | --- |
| No DRX | max[ 120ms, [3] x max(MGRP, SMTC period)x CSSFinter ] |
| DRX cycle ≤ [320]ms | max[ 120ms, ceil(3 x 1.5) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] |
| DRX cycle > [320]ms | [3] x DRX cycle x CSSFinter |

Table 9.3.4-4: Time period for time index detection (Frequency range FR2)

| Condition | TSSB_time_index_inter |
| --- | --- |
| No DRX | max[ 200ms, MSSB_index_inter x max(MGRP, SMTC period)x CSSFinter] |
| DRX cycle ≤ [320]ms | max[ 200ms, (1.5 x MSSB_index_inter) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] |
| DRX cycle > [320]ms | MSSB_index_inter x DRX cycle x CSSFinter |

9.3.4.1	Void

9.3.4.2	Void

9.3.5	Inter frequency measurements

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses [TBD], [TBD], and [TBD], respectively,  as shown in table 9.3.5-1 and 9.3.5-2:

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max[ 200ms, [8] x max(MGRP, SMTC period)x CSSFinter] |
| DRX cycle ≤ [320]ms | max[ 200ms, ceil(8 x 1.5) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] |
| DRX cycle > [320]ms | [8] x DRX cycle x CSSFinter |
| … | … |

Table 9.3.5-2: Measurement period for inter-frequency measurements with gaps (Frequency FR2)

| Condition | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max[ 400ms, Mmeas_period_inter x max(MGRP, SMTC period) x CSSFinter] |
| DRX cycle ≤ [320]ms | max[ 400ms, (1.5 x Mmeas_period_inter) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] |
| DRX cycle > [320]ms | Mmeas_period_inter x DRX cycle x CSSFinter |
| … | … |

----------------------------------------------------------- End of Change ----------------------------------------------------------------