3GPP TSG-RAN WG4 Meeting #89 	R4-1816138

# Spokane, Washington, USA, 12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding references to SUL-related side conditions (Sections 3.3, 4.2.1, 9.1.1, 10.1.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing references to SUL-related side conditions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding references to SUL-related side conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing references to SUL-related side conditions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.2.1, 9.1.1, 10.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## --- start of change 1 ---

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

BWP	Bandwidth Part

CA	Carrier Aggregation

CC	Component Carrier

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

DC	Dual Connectivity

DL	Downlink

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FR	Frequency Range

HARQ	Hybrid Automatic Repeat Request

HO	Handover

MAC	Medium Access Control

MCG	Master Cell Group

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

NR	New Radio

NSA	Non-Standalone operation mode

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

PBCH	Physical Broadcast Channel

PCell	Primary Cell

PLMN	Public Land Mobile Network

PRACH	Physical RACH

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RRC	Radio Resource Control

RRM	Radio Resource Management

RSSI	Received Signal Strength Indicator

SA	Standalone operation mode

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing.

SFN	System Frame Number

SI	System Information

SIB	System Information Block

SMTC	SSB-based Measurement Timing configuration

SRS	Sounding Reference Signal

SS	Synchronization Signal

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TDD	Time Division Duplex

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

## --- end of change 1 ---

## --- start of change 2 ---

## 4.2	Cell Re-selection

### 4.2.1	Introduction

The cell reselection procedure allows the UE to select a more suitable cell and camp on it.

When the UE is in either Camped Normally state or Camped on Any Cell state on a cell, the UE shall attempt to detect, synchronise, and monitor intra-frequency, inter-frequency and inter-RAT cells indicated by the serving cell. For intra-frequency and inter-frequency cells the serving cell may not provide explicit neighbour list but carrier frequency information and bandwidth information only. UE measurement activity is also controlled by measurement rules defined in TS 38.304, allowing the UE to limit its measurement activity.

In the requirements of Section 4.2, the exceptions for side conditions apply as follows:

- for the UE capable of CA, the applicable exceptions for side conditions are specified in Annex B, Section B.3.2.1, B.3.2.3, or B.3.2.5 for UE supporting CA in FR1, CA in FR2 and CA between FR1 and FR2, respectively;

- for the UE capable of SUL, the applicable exceptions for side conditions are specified in Annex B, Section B.3.4.1 for UE supporting SUL in FR1.

--- end of change 2 ---

--- start of change 3 ---

### 9.1.1	Introduction

This clause contains general requirements on the UE regarding measurement reporting in RRC_CONNECTED state. The requirements are split in intra-frequency, inter-frequency, inter-RAT E-UTRAN FDD, and inter-RAT E-UTRAN TDD requirements. These measurements may be used by the NG-RAN. The measurement quantities are defined in TS38.215[4], the measurement model is defined in TS38.300[10], TS37.340[17] and measurement accuracies are specified in clause 10. Control of measurement reporting is specified in [16].

In the requirements of Section 9, the exceptions for side conditions apply as follows:

- for the UE capable of CA but not configured with any SCell, the applicable exceptions for side conditions are specified in Annex B, Section B.3.2.1 for UE supporting CA in FR1, and Section B.3.2.3 for UE supporting CA in FR2, respectively;

- for the UE capable of CA and configured with at least one SCell, the applicable exceptions for side conditions are specified in Annex B, Section B.3.2.2 for UE configured with CA in FR1, and Section B.3.2.4 for UE configured with CA in FR2, respectively;

- for the UE capable of SUL but not configured with SUL, the applicable exceptions for side conditions are specified in Annex B, Section B.3.4.1 for UE supporting SUL in FR1;

- for the UE capable of SUL and configured with at least one SUL, the applicable exceptions for side conditions are specified in Annex B, Section B.3.4.2 for UE configured with SUL in FR1.

--- end of change 3 ---

--- start of change 4 ---

### 10.1.1	Introduction

Editor’s note: new measurement metrics may be added according to the RAN4 discussion. Absolute/relative accuracy requirement, mapping table of RSRP/RSRQ may be specified in this section. The numerology and BW combinations might be reflected in the accuracy requirement table.

In the requirements of Section 10.1, the exceptions for side conditions apply as follows:

- for the UE capable of CA but not configured with any SCell, the applicable exceptions for side conditions are specified in Annex B, Section B.3.2.1 for UE supporting CA in FR1, and Section B.3.2.3 for UE supporting CA in FR2, respectively;

- for the UE capable of CA and configured with at least one SCell, the applicable exceptions for side conditions are specified in Annex B, Section B.3.2.2 for UE configured with CA in FR1, and Section B.3.2.4 for UE supporting CA in FR2, respectively;

- for the UE capable of SUL but not configured with SUL, the applicable exceptions for side conditions are specified in Annex B, Section B.3.4.1 for UE supporting SUL in FR1;

- for the UE capable of SUL and configured with at least one SUL, the applicable exceptions for side conditions are specified in Annex B, Section B.3.4.2 for UE configured with SUL in FR1.

## --- end of change 4 ---