3GPP TSG-RAN WG4 Meeting #89	R4-1816169

Spokane, USA, 12 – 16 November 2018

Source:	Ericsson, Telstra

Title:	TP to TR 37.716-21-11: Addition of EN-DC configuration DC_3C-28A_n78A_BCS0

Agenda item:	9.4.2

Document for:	Approval

# Background

This contribution provides text proposal to TR 37.716-21-11 for addition of EN-DC configuration DC_3C-28A_n78A_BCS0 as defined in [1].

# Text Proposal

##### ---Start of changes---

## 5.1.x	DC_3C-28A_n78A_BCS0

#### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-28_n782 | CA_3-28 | n78 | DC_3_n78 |
| NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3C-28A_n78A | DC_3A_n78A DC_28A_n78A | CA_3C-28A | n78A |

#### 5.1.x.3	∆TIB and ∆RIB values

For DC_3-28_n78 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n78 | 3 | 0.2 |
|  | n78 | 0.5 |

### 5.1.x.4	REFSENS requirements

The impacts are covered in the fallback combination DC_3A-28A_n78A and will also apply for DC_3C-28A_n78A and need to be included in Table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | FDD | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A |  | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | TDD | N/A |

##### ---End of changes---

# Reference

[1]	RP-181855, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei