3GPP TSG-RAN WG4 Meeting #89	R4-1816171

Spokane, USA, 12 – 16 November 2018

Source:	Ericsson

Title:	TP to TR 36.716-02-02: Addition of CA configuration for 2DL_48C_2UL_48C_BCS0, 3DL_48A-48C_2UL_48C_BCS0 and 4DL_48C-48C_2UL_48C_BCS0

Agenda item:	8.1.2

Document for:	Approval

# Background

This contribution provides text proposal on the CA configuration 2DL_48C_2UL_48C_BCS0, 3DL_48A-48C_2UL_48C_BCS0 and 4DL_48C-48C_2UL_48C_BCS0 as defined in [1].

# Text Proposal

##### ---Start of changes---

## 5.X	2DL_48C_2UL_48C_BCS0
3DL_48A-48C_2UL_48C_BCS0
4DL_48C-48C_2UL_48C_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Bandwidth combinations for Intra band contiguous CA configurations

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | CBW {MHz] | CBW {MHz] | CBW {MHz] | CBW {MHz] | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_48C | CA_48C | 48 | 5, 10, 15, 20 | 20 |  |  | 40 | 0 |
|  |  |  | 20 | 5, 10, 15 |  |  |  |  |

Table 5.X.1-1: Bandwidth combinations for Intra band non-contiguous CA configurations

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | CBW {MHz] | CBW {MHz] | CBW {MHz] | CBW {MHz] | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_48A-48C | CA_48C | 48 | 5, 10, 15, 20 | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  | 60 | 0 |
|  |  |  | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  | 5, 10, 15, 20 |  |  |  |
| CA_48C-48C | CA_48C | 48 | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  | 80 | 0 |

### 5.X.2	Coexistence studies

No co-existence studies needed for these combinations.

### 5.X.3	REFSENS requirements

No REFSENS requirements need to be defined for these combinations.

### 5.x.4	Vendor A, A-MPR for uplink CA bandwidth class C

#### 5.x.4.1 Simulation Assumptions

A new spectrum emission mask for band 48 for running carrier aggregation was proposed and Figure 5.x.4.1-1 shows the new accepted mask. It has more relaxed requirements within the band compare to the out-of-band emissions.

Figure 5.x.4.1-1: FCC agreed on a new mask for CA_48C [2]

Figure 5.x.4.1-1 shows that the hardest requirements are when transmitting on the edge of the band. All simulations are allocated at the lower end of the band shown in Figure 5.x.4.1-2 with the implemented mask.

Adjacent Channel Leakage Ratio (ACLR) simulation assumptions were as follows:

- UTRAACLR1 = 33dB

- UTRAACLR2 = 36dB

- CA E-UTRAACRL = 30dB

- CA NRACRL = 30 dB

Spurious emissions limit was -30dBm with 1MHz measurement bandwidth

PA operating point: UTRAACLR1 = 33 dBc with Pout = 22 dBm.

- All further back off (to fulfil the A-MPR requirements) is calculated from the operating point of 22 dBm.

Figure 5.x.4.1-2. Visualizing the mask when the CA is located on the edge of band 48.

The spectrum emission mask may be formed as one within the band and with an additional requirement out-of-band. It may give us Table 5.x.4.1-3 and Table 5.x.4.1-4, respectively where B is either 3530 MHz or 3720 MHz depending on if it is the above or the below the band.

Table 5.x.4.1-3: Additional requirement for CA_48C within the band

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25+100RB (25 MHz) | 50+100RB (30 MHz) | 75+100RB (35MHz) | 100+100RB (40 MHz) | Measurement bandwidth |
| 0-25 | -13 | -13 | -13 | -13 | 1 MHz |
| 25-30 | -25 | -13 | -13 | -13 | 1 MHz |
| 30-35 | -25 | -25 | -13 | -13 | 1 MHz |
| 35-40 | -25 | -25 | -25 | -13 | 1 MHz |
| 40-B | -25 | -25 | -25 | -25 | 1 MHz |

.

Table 5.x.4.1-4: Additional requirement for CA_48C out-of-band emissions

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25+100RB (25 MHz) | 50+100RB (30 MHz) | 75+100RB (35MHz) | 100+100RB (40 MHz) | Measurement bandwidth |
| 0-10 | -13 | -13 | -13 | -13 | 1 MHz |
| 10-20 | -25 | -25 | -25 | -25 | 1 MHz |
| 20- | -40 | -40 | -40 | -40 | 1 MHz |

#### 5.x.4.2 Contiguous RB Transmission

Results from these simulations are shown in the figures below.

Figure 5.x.4.2-1. Required A-MPR for 25 + 100 RB, QPSK

Figure 5.x.4.2-2. Required A-MPR for 100 + 25 RB, QPSK

Figure 5.x.4.2-3. Required A-MPR for 50 + 100 RB, QPSK

Figure 5.x.4.2-4. Required A-MPR for 100 + 50 RB, QPSK

Figure 5.x.4.2-5. Required A-MPR for 75 + 100 RB, QPSK

Figure 5.x.4.2-6. Required A-MPR for 100 + 75 RB, QPSK

Figure 5.x.4.2-7. Required A-MPR for 100 + 100 RB, QPSK

Based on the simulation results presented from Figure 5.x.4.2-1 to Figure 5.x.4.2-7, the A-MPR requirements for contiguously allocated CA_48C is given in Table 5.x.4.2-1.

Table 5.x.4.2-1: A-MPR requirements for contiguously allocated CA_48C configuration

| CA_48C: CA_NS_xx | RBStart | L_CRB [RBs] | A-MPR for QPSK [dB] |
| --- | --- | --- | --- |
| 25 RB / 100 RB and 100 RB / 25 RB | 0 – 2 and 121 - 124 | >0 | ≤11dB |
|  | 3 – 27 | <85 | ≤3dB |
|  |  | >85 | ≤5dB |
|  | 27 – 120 | >0 | ≤2dB |
| 50 RB / 100 RB and 100 RB / 50 RB | 0 – 10 and 139 – 149 | >0 | ≤13dB |
|  | 11 – 28 | <85 | ≤3dB |
|  |  | >85 | ≤5dB |
|  | 29 – 138 | >0 | ≤4dB |
| 75 RB / 100 RB and 100 RB / 75 RB | 0 – 18 and 156 – 174 | >0 | ≤13dB |
|  | 19 – 42 | <95 | ≤3dB |
|  |  | >95 | ≤6dB |
|  | 43 – 155 | >0 | ≤4dB |
| 100 RB / 100 RB | 0 – 26 and 173 – 199 | >0 | ≤14dB |
|  |  | 27 – 45 |  | ≤100 |  | ≤3dB |  |
|  |  |  |  | >100 |  | ≤6dB |  |
|  |  | 46 – 172 |  | ≤100 |  | ≤2dB |  |
|  |  |  |  | >100 |  | ≤5dB |  |

#### 5.x.4.3	2-cluster RB transmission

The simulations for 2-cluster RB transmission shall have the same simulation assumptions and requirement as Contiguous RB transmission.

Because of the large number of scenarios (almost 5 million combinations for 20 + 20 MHz) a set of around 20 000 randomly created allocation scenarios were simulated per bandwidth combination and appropriate back off value was searched.

Results from these simulations with proposed A-MPR mask are shown in the figures below

Figure 5.x.4.3-1: 2 clusters in 2CCs vs. allocation ratio, 25 RB + 100 RB

Figure 5.x.4.3-2: 2 clusters in 2CCs vs. allocation ratio, 50 RB + 100 RB

Figure 5.x.4.3-3: 2 clusters in 2CCs vs. allocation ratio, 75 RB + 100 RB

Figure 5.x.4.3-4: 2 clusters in 2CCs vs. allocation ratio, 100 RB + 100 RB

The mask shown in from figures 5.x.4.3-1 to figures 5.x.4.3-4 can be formally defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =       17; 					0 ≤ A < 0.025

-40A + 18;			0.025 ≤ A < 0.1

-3.75A + 14.375;		0.1 ≤ A < 0.5

-7A + 16; 				0.5 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

### 5.x.5	Vendor B, A-MPR for uplink CA bandwidth class C

Assumption, simulation data, possible conclusion to be further investigated.

##### ---End of changes---

# Reference

[1]	181548, “Revised WID on Rel-16 LTE intra-band Carrier Aggregation for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y)”, Ericsson

[2] DOC-354370A1, “Promoting Investment in the 3550-3700 MHz Band”, FCC