3GPP TSG-RAN WG4 Meeting #89                                                   R4-1816215

Spokane, US, 12th – 16th Nov, 2018

Agenda Item:	9.2.2

Source: 	MediaTek Inc., CHTTL

Title: 	TP for TR 38.716-02-00: MSD for CA_n1A-n77A due to the 2nd harmonic

Document for:	Approval

- 1. Introduction

The NR CA_n1A-n77A was approved in RAN4 meetings. In the contribution, we propose the MSD requirement for CA_n1A-n77A due to the 2nd harmonic.

- 2. Discussion

CA_n1A-n77A is subjected to the 2nd order harmonic issue. The issue can be comprehended in Table 2-1. The supported bandwidths in CA_n1A-n77A is shown in Table 2-2.

|  | n1 UL | n1 UL * 2 | n77 DL |
| --- | --- | --- | --- |
| Range (MHz) | 1920 - 1980 | 3840 - 3960 | 3300 - 4200 |

Table 2-1 NR UL Harmonic relation for CA_n1A-n77A

| CA operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| CA_n1A-n77A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 120 |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

Table 2-2: Supported NR bandwidths per CA configuration for inter-band CA

In this contribution, based on measurement, the integrated harmonic power of 80MHz is 0.5 dB higher than the one of 40MHz. Since the 0.5dB power difference does not cause significant issue, the difference is neglected for MSD analysis.

In this contribution, the n77 MSD result for CA_n1A-n77A due to the 2nd harmonic is shown insTable 2-3.

|  |  | MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n1 | n77 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |

Table 2-3: n77 MSD due to the CA_n1A-n77A 2nd harmonic issue

- Text Proposal for TR 38.716-02-00

- -----Start of TP-----

- <<Unchanged sections omitted>>

6.5.1.5		REFSENS requirements

For single uplink operation of this combination, only harmonic issue need to be considered. MSD values for Band n77 due to 2nd harmonic of Band n1 in CA_n1A-n77A are captured in Table 6.5.1.5-1.

Table 6.5.1.5-1: MSD due to harmonic issue for CA_n1-n77

|  |  |  | MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  |  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n1 |  | n771,2 |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  |  | n773 |  |  | 1.1 | 0.8 | 0.3 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
|  | NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5.1.5-2 Uplink configuration for the low band

|  |  | NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n77 |  |  | 25 | 36 | 50 |  |  |  | 100 | 100 | 100 | 100 | 100 | 100 |

- <<Unchanged sections omitted>>

- References