3GPP RAN WG4 Meeting #89		R4-1816216
Spokane, USA, November 12 – 16, 2018

Agenda item:	7.6.13.2.1

Source:	Apple Inc.

Title:	FR2 multi-band support adhoc minutes

Document for:	Approval

# Available data in RAN4

Based on the submitted data in [1] through [6] during the RAN4 #88 meeting, the submitted data in [9] through [18] during the RAN4 #88bis meeting, the submitted data in [19] through [27], and also based on offline discussion, Tables 1 and 2 below collect the data sets on peak and spherical coverage relaxations.

Table 1: Available multi-band relaxation data for peak EIRP

| Case | Supported bands | Band | [1] | [2,22] | [13] | [4,10] | [5] | [6, 27] | [9] | [14] | [18] | [23,26] | [24]a | [24]b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | n257, n258 | n257 |  | 1.0 |  |  | 2.0 | 1.5 | 1.0 |  |  |  |  |  |
|  | n257, n258 | n258 |  | 1.0 |  |  | 2.0 | 1.5 | 1.0 |  |  |  |  |  |
| 7 | n258, n260 | n258 |  | 1.0 |  |  |  |  | 1.0 |  |  |  |  |  |
|  | n258, n260 | n260 |  | 1.0 |  |  |  |  | 1.0 |  |  |  |  |  |
| 6 | n258, n261 | n258 |  | 1.0 |  |  | 2.0 |  | 1.5 |  |  |  |  |  |
|  | n258, n261 | n261 |  | 0.5 |  |  | 2.0 |  | 1.0 |  |  |  |  |  |
| 5 | n260, n261 | n260 | 0.0 | 0.0 | 0.0 | 0.0 |  |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | n260, n261 | n261 | 0.0 | 0.0 | 0.0 | 1.0 |  |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.0 |
| 4 | n257, n258, n261 | n257 |  | 1.0 |  |  | 2.0 | 1.5 | 1.5 |  |  |  |  |  |
|  | n257, n258, n261 | n258 |  | 1.0 |  |  | 2.0 | 1.5 | 1.5 |  |  |  |  |  |
|  | n257, n258, n261 | n261 |  | 0.5 |  |  | 2.0 | 1.5 | 1.0 |  |  |  |  |  |
| 3 | n257, n260, n261 | n257 |  | 0.0 |  |  |  |  | 1.5 |  |  |  |  |  |
|  | n257, n260, n261 | n260 |  | 0.0 |  |  |  |  | 1.5 |  |  |  |  |  |
|  | n257, n260, n261 | n261 |  | 0.0 |  |  |  |  | 1.5 |  |  |  |  |  |
| 2 | n258, n260, n261 | n258 |  | 1.0 |  |  |  |  | 2.0 |  |  |  |  |  |
|  | n258, n260, n261 | n260 |  | 1.0 |  |  |  |  | 2.0 |  |  |  |  |  |
|  | n258, n260, n261 | n261 |  | 0.0 |  |  |  |  | 1.5 |  |  |  |  |  |
| 1 | n257, n258, n260, n261 | n257 | 2.1 |  |  | 1.8 |  | 2.0 | 2.0 |  | 0.0 |  |  |  |
|  | n257, n258, n260, n261 | n258 | 2.1 |  |  | 1.8 |  | 2.0 | 2.0 |  | 0.0 |  |  |  |
|  | n257, n258, n260, n261 | n260 | 1.5 |  |  | 0.0 |  | 0.0 | 2.0 |  | 0.0 |  |  |  |
|  | n257, n258, n260, n261 | n261 | 2.0 |  |  | 1.8 |  | 2.0 | 2.0 |  | 0.0 |  |  |  |

Table 2: Available multi-band relaxation data for 50%-tile CDF EIRP

| Case | Supported bands | Band | [1] | [2,22] | [13] | [4,10] | [5] | [6, 27] | [9] | [14] | [18] | [23,26] | [24]a | [24]b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | n257, n258 | n257 |  | 0.0 |  |  | 2.0 | 1.5 | 1.0 |  |  |  |  |  |
|  | n257, n258 | n258 |  | 0.0 |  |  | 2.0 | 1.5 | 1.0 |  |  |  |  |  |
| 7 | n258, n260 | n258 |  | 0.0 |  |  |  |  | 1.0 |  |  |  |  |  |
|  | n258, n260 | n260 |  | 0.0 |  |  |  |  | 1.0 |  |  |  |  |  |
| 6 | n258, n261 | n258 |  | 0.0 |  |  | 2.0 |  | 1.5 |  |  |  |  |  |
|  | n258, n261 | n261 |  | 0.0 |  |  | 2.0 |  | 1.0 |  |  |  |  |  |
| 5 | n260, n261 | n260 | 1.6 | 0.0 | 0.6 | 0.0 |  |  | 1.0 | 0.0 | 0.0 | 0.4 | 0.1 | 1.3 |
|  | n260, n261 | n261 | 0.0 | 0.0 | 0.0 | 1.6 |  |  | 0.0 | 0.0 | 0.0 | 1.4 | 2.4 | 0.2 |
| 4 | n257, n258, n261 | n257 |  | 0.0 |  |  | 2.0 | 1.5 | 1.5 |  |  |  |  |  |
|  | n257, n258, n261 | n258 |  | 0.0 |  |  | 2.0 | 1.5 | 1.5 |  |  |  |  |  |
|  | n257, n258, n261 | n261 |  | 0.0 |  |  | 2.0 | 1.5 | 1.0 |  |  |  |  |  |
| 3 | n257, n260, n261 | n257 |  | 0.0 |  |  |  |  | 1.5 |  |  |  |  |  |
|  | n257, n260, n261 | n260 |  | 0.0 |  |  |  |  | 1.5 |  |  |  |  |  |
|  | n257, n260, n261 | n261 |  | 0.0 |  |  |  |  | 1.5 |  |  |  |  |  |
| 2 | n258, n260, n261 | n258 |  | 0.0 |  |  |  |  | 2.0 |  |  |  |  |  |
|  | n258, n260, n261 | n260 |  | 0.0 |  |  |  |  | 2.0 |  |  |  |  |  |
|  | n258, n260, n261 | n261 |  | 0.0 |  |  |  |  | 1.5 |  |  |  |  |  |
| 1 (NOTE 1) | n257, n258, n260, n261 | n257 | 1.9 | 0.0 |  | 2.4 |  | 2.0 | 2.0 |  | 0.0 |  |  |  |
|  | n257, n258, n260, n261 | n258 | 2.0 | 0.0 |  | 2.4 |  | 2.0 | 2.0 |  | 0.0 |  |  |  |
|  | n257, n258, n260, n261 | n260 | 1.8 | 0.0 |  | 1.3 |  | 1.0 | 2.0 |  | 0.0 |  |  |  |
|  | n257, n258, n260, n261 | n261 | 1.2 | 0.0 |  | 2.4 |  | 2.0 | 2.0 |  | 0.0 |  |  |  |

NOTE 1: Multi-band relaxation data for 50%-tile CDF from [2, 22] for Case 1 has been provided based on network performance considerations alone.

Discussion:

Chair requests to clarify:

To proponents of [22], is it possible to share the technical analysis related to the changes to values for Case 4 and Case 3?  Also, is the understanding correct that peak EIRP relaxations for Case 3 are no longer proposed by the proponents?  If so, what is the meaning of the proposed spherical coverage relaxation values for Case 3?

Qualcomm: we had intended to carry forward numbers from our previous submission, but we had a copy and paste error; we can revert to our previously submitted values

Chair note: values in Table 1, Table 2, and Table 3 are updated based on the clarification from Qualcomm

# Deriving the multi-band relaxation values

Based on the available data and the prioritization of cases, Table 3 below summarizes the data in terms of minimum, maximum, and average of the proposed values.

Effort to develop the proposals is prioritized to target Case 5 and then other cases.  Columns to handle potential alternate proposals are provided to help the discussion.

Table 3: Summary of multi-band relaxation data and proposals

| Case | Supported bands | Band | Peak Relaxation |  |  |  |  | Spherical relaxation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Avg | Alt.1 | Alt.2 | Min | Max | Avg | Alt.1 | Alt.2 |
| 8 | n257, n258 | n257 | 1.0 | 2.0 | 1.4 |  |  | 0.0 | 2.0 | 1.1 |  |  |
|  | n257, n258 | n258 | 1.0 | 2.0 | 1.4 |  |  | 0.0 | 2.0 | 1.1 |  |  |
| 7 | n258, n260 | n258 | 1.0 | 1.0 | 1.0 |  |  | 0.0 | 1.0 | 0.5 |  |  |
|  | n258, n260 | n260 | 1.0 | 1.0 | 1.0 |  |  | 0.0 | 1.0 | 0.5 |  |  |
| 6 | n258, n261 | n258 | 1.0 | 2.0 | 1.5 |  |  | 0.0 | 2.0 | 1.2 |  |  |
|  | n258, n261 | n261 | 0.5 | 2.0 | 1.2 |  |  | 0.0 | 2.0 | 1.0 |  |  |
| 5 | n260, n261 | n260 | 0.0 | 0.0 | 0.0 |  |  | 0.0 | 1.6 | 0.5 |  |  |
|  | n260, n261 | n261 | 0.0 | 1.0 | 0.2 |  |  | 0.0 | 2.4 | 0.6 |  |  |
| 4 | n257, n258, n261 | n257 | 1.0 | 2.0 | 1.5 |  |  | 0.0 | 2.0 | 1.3 |  |  |
|  | n257, n258, n261 | n258 | 1.0 | 2.0 | 1.5 |  |  | 0.0 | 2.0 | 1.3 |  |  |
|  | n257, n258, n261 | n261 | 0.5 | 2.0 | 1.3 |  |  | 0.0 | 2.0 | 1.1 |  |  |
| 3 | n257, n260, n261 | n257 |  |  |  |  |  |  |  |  |  |  |
|  | n257, n260, n261 | n260 |  |  |  |  |  |  |  |  |  |  |
|  | n257, n260, n261 | n261 |  |  |  |  |  |  |  |  |  |  |
| 2 | n258, n260, n261 | n258 | 1.0 | 2.0 | 1.5 |  |  | 0.0 | 2.0 | 1.0 |  |  |
|  | n258, n260, n261 | n260 | 1.0 | 2.0 | 1.5 |  |  | 0.0 | 2.0 | 1.0 |  |  |
|  | n258, n260, n261 | n261 | 0.0 | 1.5 | 0.8 |  |  | 0.0 | 1.5 | 0.8 |  |  |
| 1 | n257, n258, n260, n261 | n257 | 0.0 | 2.1 | 1.6 |  |  | 0.0 | 2.4 | 1.4 |  |  |
|  | n257, n258, n260, n261 | n258 | 0.0 | 2.1 | 1.6 |  |  | 0.0 | 2.4 | 1.4 |  |  |
|  | n257, n258, n260, n261 | n260 | 0.0 | 2.0 | 0.7 |  |  | 0.0 | 2.0 | 1.0 |  |  |
|  | n257, n258, n260, n261 | n261 | 0.0 | 2.0 | 1.6 |  |  | 0.0 | 2.4 | 1.3 |  |  |

Discussion:

Qualcomm: for Case 3 we do not have peak submissions there

LGE: we provided values for Case 3 only; we can prioritize Case 5 and Case 1

Intel: for Case 5 it seems that the average is essentially 0 for the peak for both bands and 0.5 and 0.6 for the spherical coverage; can we discuss whether the averages make sense?

Sony: we have seen simulation so far; based on this we can see we can optimize for different bands; we can’t get 0 for both bands at the same time; there is some margin in the specification also; we assume that it is possible to optimize one band over the other: can we ask operators which band to optimize in Case 5?

Intel: we could prioritize bands, but the question is whether everyone will be able to agree to this; perhaps splitting the pain between the two bands makes more sense; we think shared pain approach is more fair

Chair: does shared pain mean 0.5 dB each band?

Intel: yes

AT&T: it would be hard to prioritize one band over the other; we don’t have an issue with agreeing with the shared pain approach

LGE: shared pain is confusing; in LTE CA case we averaged all interested companies’ additional IL and then we divided half and half between vendor and operator; this would mean we adopt 0.25 dB; in our understanding, Intel’s proposal is only average manner

Intel: we meant sharing the pain between two bands

Qualcomm: we thought that n261 spherical coverage impact was already agreed; we thought just the n260 spherical was under discussion

Verizon: we agree with Qualcomm

Chair: in Gothenburg meeting n261 value was 0 based on the data available then; since then more data was submitted

Apple: perhaps we can start capturing alternatives and see where we can converge; what is the range of the values we are talking about for Case 5

Alt. 1: Case 5 peak EIRP relaxation = 0 for n261 and n260; spherical EIRP relaxation = 0.5 for n261 and n260

Alt. 2: Case 5 peak EIRP relaxation = 0 for n261 and n260; spherical EIRP relaxation = 1.0 dB total available to be allocated to n261 and n260 according to UE implementation

Alt. 3: Agree band-specific relaxation values X dB (for peak) and Y dB (for spherical) to be allocated to each appropriate supported band according to UE implementation

Vodafone: we wonder if it makes sense to define total pain without explicitly stating which band is applies to; would this work?

Apple: from the earlier comments we understood that some operators said they need to have a clear number for planning; is it correct understanding that we need to have a clear number rather than a range?

Qualcomm: we think that a band combination agnostic setup could be pursued; this way network operators could know what kind of degradation to expect

Intel: if we understood Qualcomm correctly, should we agree that whatever value we agree should apply to all of the cases 1 through 8?

Qualcomm: yes

Apple: going to the data, the larger numbers are for Case 1; does this mean we derive the value from Case 1 so that all the other cases can be supported?

Qualcomm: suggest clarifying Alt. 3 to include peak and spherical impact (X and Y)

Verizon: can Qualcomm clarify?

Qualcomm: we want to make this forward compatible to future bands; the X value (peak impact) for n260 and n261 for Case 5 are already stable

# How to implement the framework in the specification

Proposal in [20]:

--- begin text proposal ---

#### 6.2.1.3	UE maximum output power for power class 3

…

Table 6.2.1.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |

…

Peak EIRP and spherical coverage performance for UEs which support multiple bands is expected to be optimized over the entire supported range of frequencies and is reflected in the requirement as follows:

-	Single-band peak EIRP limits, TSBP, in Table 6.2.1.3-1 are modified by the factor ΔTMBP from Table 6.2.1.3-4, such that the minimum peak EIRP limit for UEs which support the bands indicated in Table 6.2.1.3-4 is TMBP = TSBP - ΔTMBP.

-	Single-band EIRP spherical coverage limits, TSBS, in Table 6.2.1.3-3 are modified by the factor ΔTMBS from Table 6.2.1.3-4, such that the EIRP spherical coverage limit for UEs which support the bands indicated in Table 6.2.1.3-4 is TMBS = TSBS - ΔTMBS.

Table 6.2.1.3-4: UE multi-band factors for power class 3

| Supported bands | Applicable to band | ΔTMBP (dB) | ΔTMBS (dB) |
| --- | --- | --- | --- |
| n260, n261 | n260 | 0 | 1.6 |
| n260, n261 | n261 | 0 | 0 |
| n257, n258, n260, n261 | n257 | 2.1 | 1.9 |
| n257, n258, n260, n261 | n258 | 2.1 | 2.0 |
| n257, n258, n260, n261 | n260 | 1.5 | 1.8 |
| n257, n258, n260, n261 | n261 | 2.0 | 1.2 |
| NOTE 1: The requirements in this table are applicable to UEs which support only the indicated bands |  |  |  |

--- end text proposal ---

Proposal in [21]:

--- begin text proposal ---

### 6.2.4	Configured transmitted power

The UE can configure its maximum output power. The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement in each receiver branch as specified in 38.215.

The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

PPowerclass – MAX(MPRf,c+ ΔMBP, P-MPRf,c) – MAX{T(MPRf,c), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ TRPmax

with PPowerclass the UE power class as specified in sub-clause 6.2.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1, MPRf,c as specified in sub-clause 6.2.2, P-MPRf,c the power management term for the UE, ΔMBP the peak EIRP relaxation as specified in section 6.2.1 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1. The tolerance T(∆P) for applicable values of ∆P (values in dB) is specified in Table 6.2.4-1.

…

For the UE that supports operation in multiple FR2 bands, peak EIRP relaxation parameter MBP and EIRP spherical coverage relaxation parameter MBS are as specified in table 6.2.1.3-4

Table 6.2.1.3-4: UE EIRP multi-band relaxation parameters for power class 3

| Band Combination | Operating Band | MBP [dB] | MBS [dB] |
| --- | --- | --- | --- |
| n257+n258 | n257 | 1.0 | 0.0 |
|  | n258 | 1.0 | 0.0 |
| n258+n260 | n258 | 1.0 | 0.0 |
|  | n260 | 1.0 | 0.0 |
| n258+n261 | n258 | 1.0 | 0.0 |
|  | n261 | 0.0 | 0.0 |
| n260+n261 | n260 | 0.0 | 0.0 |
|  | n261 | 0.0 | 0.0 |
| n257+n258+n261 | n257 | 0.0 | 0.0 |
|  | n258 | 0.0 | 0.0 |
|  | n261 | 0.0 | 0.0 |
| n257+n260+n261 | n257 |  | 0.0 |
|  | n260 |  | 0.0 |
|  | n261 |  | 0.0 |
| n258+n260+n261 | n258 | 1.0 | 0.0 |
|  | n260 | 1.0 | 0.0 |
|  | n261 | 0.0 | 0.0 |
| n257+n258+n260+n261 | n257 |  | 0.0 |
|  | n258 |  | 0.0 |
|  | n260 |  | 0.0 |
|  | n261 |  | 0.0 |

…

### 7.3.3	Multi-band relaxation

For the UE that supports operation in multiple FR2 bands, reference sensitivity relaxation parameter MBP and EIS spherical coverage relaxation parameter MBS are as specified in section 6.2.1

--- end text proposal ---

Discussion:

Qualcomm: in the interest of progress, we can remove the values if we can find agreement on the wording and sections; can we agree that like all other relaxations the configured power will reflect it?

Components to merge:

Pcmax equation

Apple: if we have one single number, can we merge this with this Pcmax equation?

Qualcomm: the equation can remain the way it is; the value we converge on can feed into the equation

Agreement: the Pcmax equation includes multi-band relaxation associated with peak EIRP only, and the change associated with Clauses 3.2 and 6.2.4 in [21] can be endorsed

Multi-band relaxation values and description

Proposal: add “NOTE 2: The requirements in this table are only applicable for UE which support single band in FR2” to the peak EIRP table

NTT DOCOMO: our preference is to capture this note together with agreed relaxation values

Verizon: for now this is OK; we are on the way to build up a framework; how can we add the note and later on we have a framework

EIRP relaxation descriptions

Alt. 1: For the UE that supports operation in multiple FR2 bands, peak EIRP relaxation parameter MBP and EIRP spherical coverage relaxation parameter MBS are as specified in table 6.2.1.3-4

Alt. 2: Peak EIRP and spherical coverage performance for UEs which support multiple bands is expected to be optimized over the entire supported range of frequencies and is reflected in the requirement as follows:

-	Single-band peak EIRP limits, TSBP, in Table 6.2.1.3-1 are modified by the factor ΔTMBP from Table 6.2.1.3-4, such that the minimum peak EIRP limit for UEs which support the bands indicated in Table 6.2.1.3-4 is TMBP = TSBP - ΔTMBP.

-	Single-band EIRP spherical coverage limits, TSBS, in Table 6.2.1.3-3 are modified by the factor ΔTMBS from Table 6.2.1.3-4, such that the EIRP spherical coverage limit for UEs which support the bands indicated in Table 6.2.1.3-4 is TMBS = TSBS - ΔTMBS.

Apple: the delta in Alt. 2 is clear that it is subracted from the Tx requirements, and for EIS it is added; Alt. 1 may make it confusing how to apply the delta to the requirement

LGE: the configured Tx power equation already captures the subtraction; Alt. 1 is more clear

Apple: we are OK with Alt 1 for EIRP, but we need to do something for EIS

Agreement: adopt Alt. 1 for the EIRP relaxation description

EIS multi-band relaxation description

Alt. 1: For the UE that supports operation in multiple FR2 bands, reference sensitivity relaxation parameter MBP and EIS spherical coverage relaxation parameter MBS are as specified in section 6.2.1

Apple: as long as we clarify that the delta is added for EIS, we would be OK; maybe for EIRP we can say it is negative or positive, and we say for EIS it is negative

Qualcomm: perhaps we can take a look at the LTE notation and suggest a way forward after further offline discussion

# Others

Discussion:

# References

RAN4 #88, Gothenburg

- R4-1809786, “Extending FR2 power class requirements to multi-band UEs,” Apple Inc.

- R4-1811021, “FR2 Multi-band Framework for PC3 UEs,” Qualcomm

- R4-1810038, “FR2 MOP Multiband Relaxation Discussion,” MediaTek

- R4-1810611, “UE Spherical EIRP for the 28GHz and 39GHz bands,” Sony

- R4-1811205, “CR for 38.101-2: UE maximum output power for power class 3,” Huawei, HiSilicon

- R4-18xxxxx, OPPO

- R4-1809787, “Draft CR to TS 38.101-2: Introducing multi-band applicability for PC3,” Apple Inc.

- R4-1811022, “Draft CR to 38.101-2: FR2 EIRP Multiband Framework,” Qualcomm

RAN4 #88bis, Chengdu

- R4-1812247, “Multi-band relaxation for global NR UE with common platform at FR2,” LG Electronics

- R4-1812337, “UE Spherical EIRP for the 28GHz and 39GHz bands,” Sony

- R4-1812390, “Further views on extending FR2 power class requirements to multi-band UEs,” Apple Inc.

- R4-1812391, “Draft CR to TS 38.101-2: Introducing multi-band applicability for PC3,” Apple Inc.

- R4-1812452, “FR2 MOP Multiband Relaxation Discussion 2,” MediaTek

- R4-1812459, “EIRP requirements for UE supporting n260 and n261,” Samsung

- R4-1812705, “Multi-band cover effect for EIRP,” NTT DOCOMO

- R4-1813361, “Draft CR to 38.101-2: FR2 EIRP Multiband Relaxation in PCMAX,” Qualcomm

- R4-1813828, “Further views on extending FR2 power class requirements to multi-band UEs,” Apple Inc.

- R4-1813830, “Multi-band cover effect for EIRP,” NTT DOCOMO

RAN4 #89, Spokane

- R4-1813828, "Further views on extending FR2 power class requirements to multi-band UEs," Apple Inc.

- R4-1814703, “Draft CR to TS 38.101-2: Introducing multi-band applicability for PC3,” Apple Inc.

- R4-1815621, “Draft CR to 38.101-2: FR2 Multi-band Framework,” Qualcomm

- R4-1815627, “On FR2 PC3 UE Multi-band Relaxations,” Qualcomm

- R4-1815786, “On multi-band support for PC3,” Intel Corporation

- R4-1815845, “UE Spherical EIRP for the 28GHz and 39GHz bands,” Sony

- R4-1816034, “Impact of multi-band spherical relaxation on network performance,” Qualcomm

- R4-18xxxxx, “On multi-band support for PC3,” Intel Corporation

- R4-18xxxxx, “Discussion on EIRP multi-band relaxation,” OPPO

Other references

- RP-182083, “Rel-15 Work Item Exception for New Radio Access Technology – Core,” NTT DOCOMO, 3GPP RAN #81, September 2018