3GPP TSG-RAN WG4 Meeting #89 	R4-1816257

# Spokane, USA, November 12 – 16, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-2 to correct UL CA scope for FR2 in Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc., Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the RAN4 #88bis meeting, the following agreement was captured in the RAN4 Chairman’s minutes: Requirements which are not in the exception sheet will be out of scope of Rel-15 RAN4 specifications. • Uplink non-continous CA requirement is out of scope of Rel-15 RAN4 specification according to the RAN plenary decision in RP-182083. If uplink non-continous CA is introduced in future release, FFS on which release, uplink non-continous CA shall be release independent from. The Rel-15 NR Work Item exception sheet, as approved during RAN #81 in RP-182083, lists only uplink CA with aggregate BW up to 800 MHz in the scope of the TS38.101-2 remaining issues for Rel-15. Thus, there is no scope for UL NC CA and UL inter-band CA (FR2 + FR2) in the Rel-15 version of the TS38.101-2 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removes aspects associated with uplink non-contiguous carrier aggregation and uplink inter-band (FR2 + FR2) carrier aggregation from the requirements in the Rel-15 UE RF specification. Change 1: clarified scope of intra-band non-contiguous CA configurations to apply to DL only (Clause 5.5A.2) Change 2: Removes UL NC CA and inter-band CA aspects from the CA frequency error requirement (Clause 6.4A.1) Change 3: Clarifies applicability of Clause 6.4A.2.0 to intra-band contiguous CA Change 4: Removes UL NC CA aspects from the CA EVM requirement (Clause 6.4A.2.1) Change 5: Removes UL NC CA aspects from the CA carrier leakage requirement (Clause 6.4A.2.2) Change 6: Removes UL NC CA aspects from the CA SEM requirement (Clause 6.5A.2.1). Also removes inter-band CA aspects from this requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 requirements for FR2 UL CA are not correctly specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.2, 6.4A.1, 6.4A.2.0, 6.4A.2.1, 6.4A.2.2, 6.5A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-2 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes from R4-1814697 and R4-1815622 have been merged together |  |  |  |  |  |  |  |  |

<< start of change 1 >>

### 5.5A.2	Configurations for intra-band non-contiguous CA

### Configurations listed in this clause apply to downlink carrier aggregation only.

<< end of change 1 >>

<< start of change 2 >>

### 6.4A.1	Frequency error

The requirements in this subsection apply to UEs of all power classes.

For intra-band contiguous carrier aggregation, the UE modulated carrier frequencies per band shall be accurate to within ±0.1 PPM observed over a period of 1ms compared to the carrier frequency of primary component carrier received from the gNB.

The frequency error is defined as a directional requirement. The requirement is verified in beam locked mode on beam peak direction.

<< end of change 2 >>

<< start of change 3 >>

#### 6.4A.2.0	General

For intra-band contiguous carrier aggregation, the requirements in subclauses 6.4A.2.1, 6.4A.2.2, and 6.4A.2.3 apply.

All the parameters defined in subclause 6.4A.2 are defined using the measurement methodology specified in Annex F.

All the requirements in 6.4A.2 are defined as directional requirement. The requirements are verified in beam locked mode on beam peak direction, with both UL polarizations active.

<< end of change 3 >>

<< start of change 4 >>

#### 6.4A.2.1	Error Vector magnitude

The requirements in this subsection apply to UEs of all power classes. For intra-band contiguous carrier aggregation, the Error Vector Magnitude requirement of section 6.4.2.2 is defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers. Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform.

<< end of change 4 >>

<< start of change 5 >>

#### 6.4A.2.2	Carrier leakage

##### 6.4A.2.2.1	General

Carrier leakage is an additive sinusoid waveform. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

Note:	When UE has DL configured for non-contiguous CA, carrier leakage may land outside the spectrum occupied by all configured UL and DL CC.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform.. The requirement is verified with the test metric of Carrier Leakage (Link=TX beam peak direction, Meas=Link angle).

<< end of change 5 >>

<< start of change 6 >>

#### 6.5A.2.1	Spectrum emission mask for CA

The requirement specified in this section shall apply if the UE has at least one of UL or DL configured for CA or if the UE is configured for single CC operation with different channel bandwidths in UL and DL carriers.

For intra-band contiguous carrier aggregation, the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the ± edge of the aggregated channel bandwidth (Table 5.3A.5-1). For any bandwidth class defined in Table 5.3A.5-1, the UE emission shall not exceed the levels specified in Table 6.5A.2.1-1. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction).

Table 6.5A.2.1-1: General NR spectrum emission mask for intra-band contiguous CA in frequency range 2

| ΔfOOB (MHz) | Any carrier aggregation bandwidth class | Measurement bandwidth |
| --- | --- | --- |
| 0-0.1*BWChannel_CA | -5 | 1 MHz |
| 0.1*BWChannel_CA -2*BWChannel_CA | -13 | 1 MHz |
| NOTE 1: If carrier leakage or I/Q image lands inside the spectrum occupied by the configured UL and DL CCs, exception to the general spectrum emission mask limit applies. For carrier leakage the requirements specified in section 6.4A.2.2 shall apply. For I/Q image the requirements specified in section 6.4A.2.3 shall apply. |  |  |

<< end of change 6 >>