3GPP TSG-RAN4 Meeting #89 	R4-1816264

# Spokane, US, 12-16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0025 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

raft

| Title: | CR to TR 38.817-02:Improvements of applicability table (5.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The applicability table shows the corresponding subclauses for each requirement. For TR 38.817-02 does not have subclauses for demodulation requirements and subclauses referred in the applicability table are not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the wrong subclause numbers in the applicability table with Note which how performance requirements were developed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability table is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

5.6	Applicability of requirements

The mapping between requirement set and individual requirement is captured in Table 5.6-1.

Table 5.6-1: Requirement set applicability

| Requirement | Requirement set |  |  |  | Spatial applicability of the requirements |
| --- | --- | --- | --- | --- | --- |
|  | 1-C | 1-H | 1-O | 2-O |  |
| Base station output power | 6.2 | 6.2 | NA | NA | Conducted requirement |
| Output power dynamics | 6.3 | 6.3 | NA | NA |  |
| Transmit ON/OFF power | 6.4 | 6.4 | NA | NA |  |
| Transmitted signal quality | 6.5 | 6.5 | NA | NA |  |
| Occupied bandwidth | 6.6.2 | 6.6.2 | NA | NA |  |
| ACLR | 6.6.3 | 6.6.3 | NA | NA |  |
| Operating band unwanted Emissions | 6.6.4.x | 6.6.4.x | NA | NA |  |
| Transmitter spurious emissions | 6.6.5.x | 6.6.5.x | NA | NA |  |
| Transmitter intermodulation | 6.7.2 | 6.7.2 and 6.7.3 | NA | NA |  |
| Reference sensitivity level | 7.2 | 7.2 | NA | NA |  |
| Dynamic range | 7.3 | 7.3 | NA | NA |  |
| In-band selectivity and blocking | 7.4 | 7.4 | NA | NA |  |
| Out-of-band blocking | 7.5 | 7.5 | NA | NA |  |
| Receiver spurious emissions | 7.6 | 7.6 | NA | NA |  |
| Receiver intermodulation | 7.7 | 7.7 | NA | NA |  |
| In-channel selectivity | 7.8 | 7.8 | NA | NA |  |
| Performance requirements | [Note] | [Note] | NA | NA |  |
| Radiated transmit power | NA | 9.2 | 9.2 | 9.2 | Radiated directional requirement |
| OTA Base station output power | NA | NA | 9.3 | 9.3 | Radiated TRP requirement |
| OTA Output power dynamics | NA | NA | 9.4 | 9.4 | Radiated directional requirement |
| OTA Transmit ON/OFF power | NA | NA | 9.5 | 9.5 | Co-location requirement for 1-O Radiated TRP requirement for 2-O |
| OTA Transmitted signal quality | NA | NA | 9.6 | 9.6 | Radiated directional requirement |
| OTA Occupied bandwidth | NA | NA | 9.7.2 | 9.7.2 | Radiated directional requirement |
| OTA ACLR | NA | NA | 9.7.3 | 9.7.3 | Radiated TRP requirement |
| OTA Operating band unwanted emission | NA | NA | 9.7.4 | 9.7.4 | Radiated TRP requirement |
| OTA Transmitter spurious emission | NA | NA | 9.7.5 | 9.7.5 | Radiated TRP requirement except for co-location requirements applicable for 1-O |
| OTA Transmitter intermodulation | NA | NA | 9.8 | NA | Co-location requirement |
| OTA sensitivity | NA | 10.2 | 10.2 | NA | Radiated directional requirement |
| OTA Reference sensitivity level | NA | NA | 10.3 | 10.3 | Radiated directional requirement |
| OTA Dynamic range | NA | NA | 10.4 | NA | Radiated directional requirement |
| OTA In-band selectivity and blocking | NA | NA | 10.5 | 10.5 | Radiated directional requirement |
| OTA Out-of-band blocking | NA | NA | 10.6 | 10.6 | Radiated directional requirement except for co-location requirements applicable for 1-O |
| OTA Receiver spurious emission | NA | NA | 10.7 | 10.7 | Radiated TRP requirement |
| OTA Receiver intermodulation | NA | NA | 10.8 | 10.8 | Radiated directional requirement |
| OTA In-channel selectivity | NA | NA | 10.9 | 10.9 | Radiated directional requirement |
| Radiated Performance requirements | NA | NA | [Note] | [Note] | Radiated directional requirement |
| Note: Performance requirements / radiated performance requiremenst were developed based on performance requirements for eAAS BS [9]. |  |  |  |  |  |

--------------End of change-------------