3GPP TSG-RAN4 Meeting #89	R4-1816269

# Spokane, US, 12th - 16th November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0018 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Adding technical background FBW declaration of EIRP in sub-clause 9.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The reason for change is to add missing technical background information related to manufacturer declarations part of radiated transmit power requirement in TS 38.104, sub-clause 9.2. This is a revision of previous version (R4-1812900) submitted to RAN4#88bis. This is a revised version of R4-1815319. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In R4-1806907 a concept to allow for separate declaration for EIRP for wide supported NR bands or sub-bands was introduced in TS 38.104, sub-clause 9.2. Due to the physical size of the antenna aperture the directivty will change over large frequency bands, therefore a concept for declaring EIRP for lowest supported frequency and highest supported freqeuncy was introduced in NR. Currently, the technical background and justifiaction is not documented in TR 38.817-02. This CR adds relevant background information. In this revision: - Table with frequency bands is reduced to only hold wide NR bands. - Some text is removed to foucs on the core of directivity impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, TR 38.817-02 will not capture all technical background information used to develop RF core requirements specified in TS 38.104. Information related to manufacturer declarations relevant for EIRP and radiated transmit power for wide NR operating bands could be missused and/or missunderstood. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 9.2	Radiated transmit power

### 9.2.1	General

The minimum requirements for radiated transmit power, are placed on one or more manufacturer declared beam(s) over a declared OTA peak direction set. OTA requirements for NR BS output power are defined for directional EIRP requirements as radiated transmit power requirements (sub-clauses 9.2.2 and 9.2.3) and for TRP requirements as OTA base station output power (sub-clauses 9.3.2 and 9.3.3).

Some NR bands have a significantly larger fractional band width than existing bands as shown in Table 9.2.1-1.

Table 9.2.1-1: NR bands

| Band | Band Definition fl to fh (MHz) | Absolute Bandwidth fh – fl (MHz) | Relative Bandwidth fh/fl (dB) | Fractional Bandwidth 100.(fh-fl)/fc (%) |
| --- | --- | --- | --- | --- |
| n41 | 2496 to 2690 | 194 | 0.3 | 7.5 |
| n77 | 3300 to 4200 | 900 | 1.0 | 24.0 |
| n78 | 3300 to 3800 | 500 | 0.6 | 14.1 |
| n79 | 4400 to 5000 | 600 | 0.6 | 12.8 |
| n257 | 26500 to 29500 | 3000 | 0.5 | 10.7 |
| n258 | 24250 to 27500 | 3250 | 0.5 | 12.6 |
| n260 | 37000 to 40000 | 2500 | 0.3 | 6.5 |
| n261 | 27500 to 28350 | 850 | 0.13 | 3.0 |

The fractional bandwidth FBW is given in percent as:

FBW=100fh-flfc=200fh-flfh+fl   (%)

The radiated transmit power in a specific direction in terms of EIRP can be expressed as: EIRP = TRP + D in dBm, where TRP is the total radiated power in dBm and D is the directivity in dBi. For an array antenna the composite directivity is determined by the element directivity and the array factor directivity. From TR 38.803, Annex C [2], the element directivity can be expressed as:

DE≈10log4πAeffλ2

, where Aeff is the antenna aperture in m2 and  is the wave length in m. For a uniform rectangular array antenna, where elements as separated d m along both y-axis and the z-axis, the maximum area for the antenna aperture is limited to an area of (d)2 m2. Typically, the d is in the range of 0.5 to 0.7 and  is derived from the highest supported frequency. Since the directivity is depending on frequency, it is interesting to analyze the directivity characteristics as function of very wide operation bands.

In Table 9.2.1-2, some wide NR bands have been analyzed with respect to directivity variations (due to fixed antenna aperture.

Table 9.2.1-2: Directivity variation over supported frequency range

| Band |  (dB) |
| --- | --- |
| n41 | -0.6 |
| n77 | -2.1 |
| n78 | -1.2 |
| n79 | -1.1 |
| n257 | -0.9 |
| n258 | -1.1 |
| n260 | -0.7 |

It’s clear that the directivity is lower at lowest supported frequency compared with the highest supported frequency as expected. For a NR base station supporting wide bands the directivity variation is manganged by declaring EIRP at lowest supported frequency and highest supported frequency within a specific band.

### 9.2.2	Minimum requirement for BS type 1-O

For requirement set 1-H and 1-O, the radiated transmit power requirements will be the same as those for E-UTRA, background information for the minimum requirement is captured in 3GPP TR 37.842 [8], sub-clause 7.2.4. The radiated transmit power requirements for FR1 are defined as the directional requirements on the output power accuracy for EIRP.

As the radiated transmit power for FR1 is based on the manufacturer’s declarations, the basic limit concept does not apply to this requirement.