3GPP TSG-RAN WG4 Meeting #89	R4-1816295

Spokane, USA, 12th - 16th Nov, 2018

Agenda item:	7.9.4.2

Source: 	ZTE Corporation .

Title: 	TP to TS 38.141-2: OTA transmitter OFF power (Section 6.5.1)

Document for:	Approval

Introduction

This contribution is a text proposal to draft TS 38.141-2 [1] to complete OTA transmitter OFF power conformance testing based on the contribution [2].

- Reference

- Draft TS 38.141-2 v1.1.0_afterChengdu, Huawei.

- R4-1814516, Timing alignment for intra-band contiguous CA, ZTE

Text Proposal

--------------Start of text proposal-------------

6.5	OTA transmit ON/OFF power

6.5.1	OTA transmitter OFF power

6.5.1.1	Definition and applicability

OTA transmitter OFF power requirements apply only to TDD operation of NR BS.

OTA transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For BS supporting intra-band contiguous CA, the transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the Aggregated BS Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2 during the transmitter OFF period.

For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location test antenna conducted output(s). For BS type 2-O the transmitter OFF power is defined as TRP.

For multi-band co-location test antenna conducted output(s), the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band co-location test antenna conducted output(s) supporting multiple operating bands, the requirement is applicable per supported operating band.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band RIBs supporting transmission in multiple operating bands, the requirement is applicable per supported operating band.

6.5.1.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.2.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.2.3.

6.5.1.3	Test purpose

The purpose of this test is to verify the OTA transmitter OFF power is within the limits of the minimum requirements.

6.5.1.4	Method of test

6.5.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for mult-carrier and/or CA:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

Directions to be tested:

- The requirement for FR1 is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

- The requirement for FR2 is specified as TRP requirement. The beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP output power level is achieved.

6.5.1.4.2	Procedure

6.5.1.4.2.1	General procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing the specified measurements.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

6.5.1.4.2.2	BS type 1-O

3)	Place the co-location test antenna as specified in subclause 4.12.

4)	Set the BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test model NR-FR1-TM1.1 and set of physical channels in subclause 4.9.2.

5)	Measure the mean power spectrum density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

6)	For an NR BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is the smallest supported Sub Carrier Spacing in kHz in the Aggregated BS Channel Bandwidth.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.1.4.2.3	BS type 2-O

[3) Configure the BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

4)	Set the NR BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test models or set of physical channels in subclause 4.9.

In addition, for an NR BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.7.3.

5)	Set the NR BS in the reference direction of the appropriated TRP measurement grid (see annex I).

6)	Measure EIRP by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3μs to 35/N μs before start of next transmitter ON period - 3μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	For an NR BS supporting contiguous CA, measure EIRP by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	Repeat step 6-8 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

9)	Calculate TRP using the EIRP measurements.

For multi-band capable NR BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.]

6.5.1.5	Test requirements

6.5.1.5.1	BS type 1-O

The mean power spectral density measured according to subclause 6.5.1.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.1.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.1.5.2	BS type 2-O

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency f ≤ XX GHz.

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

--------------End of text proposal-------------