3GPP TSG-RAN WG4 Meeting #89	R4-1816300

Spokane, WA, USA, 12 – 16 November 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.141-2 – adding further details on reference steps to the annex

Agenda item:	7.9.4.2

Document for:	Approval

# 1	Introduction

This is a revision of R4-1815975 which proposes to add additional definitions for the reference angular step to Annex I in TS 38.141-2 [1] based on the outcome of the discussion in [2].

Constructive comments are encouraged.

# 4	References

- TS 38.141-2 (v1.1.0), Huawei, HiSilicon

- R4-1815969, On reference angular steps of spherical grids for TRP measurements, Nokia, Nokia Shanghai Bell

# 5	Text proposal

TS 38.141-2 v1.1.0

--------------Start of text proposal-------------

## I.2.2	Reference angular step criteria

The reference angular steps Δϕ and Δθ in are radians are defined as

Δϕref=min⁡(λDcyl,[π12])

Δθref=⁡(λD,[π12])

Dcyl and D are calculated as

Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in figure I2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure I2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

In the case of Uniform Linear Array (ULA) and the EUT is mounted along the yz plane as shown in figure I2.2-2, the  reference angular step can be determined by

∆θref=180°πarcsin⁡(λ/Dz)

Δϕref=180°πarcsin⁡(λDy)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis and  is wavelength for the measured frequency.

Figure I2.2-2. Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

Where and  are the actual angular steps used in the measurement.

For each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

--------------end of text proposal-------------