3GPP TSG-RAN WG4 Meeting #89 	R4-1816335

# Spokane, USA, 12 – 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6124 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of references in CONNECTED mode requirements for category M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are many incorrect references in curren CONNECTED mode requirements for category M1 UEs which are corrected in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: correction of references for intra-frequency CEModeA Change #2: correction of references for intra-frequency CEModeB Change #2: correction of references for inter-frequency CEModeB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect references will exist in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.13.2.1.1, 8.13.2.1.2, 8.13.2.1.3, 8.13.3.1.1, 8.13.3.1.2, 8.13.3.1.3, 8.13.3.5.1, 8.13.3.5.2, 8.13.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- Start of Change 1 ---

#### 8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A

The UE shall be able to identify new intra-frequency cells and perform RSRP and RSRQ measurements of identified intra-frequency cells without an explicit intra-frequency neighbour cell list containing physical layer cell identities. During the RRC_CONNECTED state the UE shall continuously measure identified intra frequency cells and additionally search for and identify new intra frequency cells.

##### 8.13.2.1.1	E-UTRAN FDD intra frequency measurements

8.13.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used

When no DRX is in use the UE shall be able to identify and measure a new detectable FDD intra frequency cell according to requirements in Table 8.13.2.1.1.1-1 when SCH Ês/Iot >= -6 dB, provided

-	G=1, or

-	rmax*G < 80ms, or

-	UE is receiving PDSCH.

Otherwise, requirements in Table 8.13.2.1.1.1-3 apply, where rmax and G are given by higher layer parameter mPDCCH-NumRepetition and mPDCCH-startSF-UESS respectively as defined in TS 36.213 [3].

Table 8.13.2.1.1.1-1: Requirement on cell identification delay and measurement delay for FDD intrafrequency cell

| Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- |
| 0 | 1.44 * Kintra_M1_NC * KRSTD_M1_NC seconds | 480 * Kintra_M1_NC * KRSTD_M1_NC ms |
| 1 | 2.88 * Kintra_M1_NC * KRSTD_M1_NC seconds | 960 * Kintra_M1_NC * KRSTD_M1_NC ms |

Kintra_M1_NC = 100 / X where X is signalled by the RRC parameter measGapSharingScheme [2] and is defined as in Table 8.13.2.1.1.1-2 when highSpeedMeasGapCE-ModeA [2] is not configured, and in Table 8.13.2.1.1.1-2A when highSpeedMeasGapCE-ModeA [2] is configured.  is total number of inter-frequency layers to be monitored as defined in 8.1.2.1.1. When inter frequency measurement is not configured, Kintra_M1_NC=1 regardless whether or how parameter measGapSharingScheme [2] is configured.

Table 8.13.2.1.1.1-2: Value of parameter X for CEModeA

| measGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ |  |
| ‘01’ | 40 |
| ‘10’ | 50 |
| ‘11’ | 60 |

Table 8.13.2.1.1.1-2A: Value of parameter X for CEModeA for UE configured with highSpeedMeasGapCE-ModeA

| measGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ | [] |
| ‘01’ | [50] |
| ‘10’ | [80] |
| ‘11’ | [90] |

Table 8.13.2.1.1.1-3: Requirement on cell identification delay and measurement delay for FDD intrafrequency cell with MPDCCH scaling

| Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- |
| 0 | Max(20 * rmax*G / 1000, 1.44) * Kintra_M1_NC * KRSTD_M1_NC seconds | Max(5 * rmax*G, 480) * Kintra_M1_NC * KRSTD_M1_NC ms |
| 1 | Max(20 * rmax*G / 1000, 2.88) * Kintra_M1_NC * KRSTD_M1_NC seconds | Max(5 * rmax*G, 960) * Kintra_M1_NC * KRSTD_M1_NC ms |

KRSTD_M1_NC is applicable provided following conditions are met:

-	 > 40 ms

-	 >

- PRS bandwidth is less than the bandwidth of the cell used for RSTD measurement in which case gaps are required

where

-	is the cell-specific positioning subframe configuration period as defined in TS 36.211 [16],

-	 is the number of consecutive downlink positioning subframes in a positioning occation defined in TS 36.211

Otherwise KRSTD_M1_NC = 1.

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.1 and 9.1.21.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.6 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-1 for a corresponding Band.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_UE cat M1. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra frequency measurements is according to Table 8.13.2.1.1.1-1. When measurement gaps are activated the UE shall be capable of performing measurements for at least 6 cells. If the UE has identified more than 6 cells, the UE shall perform measurements but the reporting rate of RSRP and RSRQ measurement of cells from UE physical layer to higher layers may be decreased.

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.1 and 9.1.21.2.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.6.

8.13.2.1.1.1.1	Measurement Reporting Requirements

8.13.2.1.1.1.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

8.13.2.1.1.1.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.2.1.1.1.1.3.

8.13.2.1.1.1.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2  and 9.1.21.6.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resoureces for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra_UE cat M1_NC defined in Clause 8.13.2.1.1.1. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_NC defined in clause 8.13.2.1.1.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than TMeasurement_Period_UE cat M1, Intra provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

8.13.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used

When DRX is in use the UE shall be able to identify a new detectable FDD intra frequency cell within Tidentify_intra_UE cat M1_NC as shown in table 8.13.2.1.1.2-1.

When eDRX_CONN is in use the UE shall be able to identify a new detectable FDD intra frequency cell within Tidentify_intra_UE cat M1_NC as shown in table 8.13.2.1.1.2-1A.

Table 8.13.2.1.1.2-1: Requirement to identify a newly detectable FDD intrafrequency cell

| Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1_NC (s) (DRX cycles) |
| --- | --- | --- |
| 0 | ≤0.04 | 1.44 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.04<DRX-cycle≤0.08 | Note 2 (40 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128 | 3.2 * Kintra_M1_NC * KRSTD_M1_NC (25 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128<DRX-cycle≤2.56 | Note 2(20 * Kintra_M1_NC * KRSTD_M1_NC) |
| 1 | <0.128 | 2.88 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.128 | 3.2 * Kintra_M1_NC * KRSTD_M1_NC (25 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128<DRX-cycle≤2.56 | Note 2(20 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |

Table 8.13.2.1.1.2-1A: Requirement to identify a newly detectable FDD intrafrequency cell when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tidentify_intra_UE cat M1_NC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (20 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.1 and 9.1.21.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.6 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex B.2.14-1 for a corresponding Band

In the RRC_CONNECTED state the measurement period for intra frequency measurements is Tmeasure_intra_UE cat M1. When DRX is used, Tmeasure_intra_UE cat M1_NC is as specified in table 8.13.2.1.1.2-2. When eDRX_CONN is used, Tmeasure_intra_UE cat M1_NC is as specified in table 8.13.2.1.1.2-3. The UE shall be capable of performing RSRP and RSRQ measurements for 6 identified-intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_UE cat M1.

Table 8.13.2.1.1.2-2: Requirement to measure FDD intrafrequency cells

| Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1_NC (s) (DRX cycles) |
| --- | --- | --- |
| 0 | <0.128 | 0.48 * Kintra_M1_NC * KRSTD_M1_NC (Note1) |
|  | 0.128≤DRX-cycle≤2.56 | Note 2 (5 * Kintra_M1_NC * KRSTD_M1_NC) |
| 1 | <0.256 | 0.960 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.256≤DRX-cycle≤2.56 | Note 2 (*Kintra_M1_NC * KRSTD_M1_NC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |

Table 8.13.2.1.1.2-3: Requirement to measure FDD intrafrequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_intra_UE cat M1_NC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.1 and 9.1.21.2.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.6.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.2.1.1.2.1	Measurement Reporting Requirements

8.13.2.1.1.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

8.13.2.1.1.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.2.1.1.2.1.3.

8.13.2.1.1.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra, UE cat M1  defined in Clause 8.13.2.1.1.2 When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_NC  defined in clause 8.13.2.1.1.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_intra_UE cat M1_NC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

##### 8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD

8.13.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used

The requirements in this section are applicable for the UE which supports half duplex operation on one or more supported frequency bands [2].

The requirements defined in clause 8.13.2.1.1.1 also apply for this section provided the following conditions are met:

-	at least downlink subframe # 0 or downlink subframe # 5 per radio frame of an intra-frequency cell to be identified by the UE is available at the UE over Tidentify_intra_UE cat M1;

-	at least one downlink subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell over Tmeasure_intra_UE cat M1.

-	RSRP related side conditions given in Sections 9.1.2.1 and 9.1.2.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.6 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-2 for a corresponding Band

8.13.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used

The requirements in this section are applicable for the UE which supports half duplex operation on one or more supported frequency bands [2].

When DRX is in use the UE shall be able to identify a new detectable HD-FDD intra frequency cell within Tidentify_intra_UE cat M1_NC  as shown in table 8.13.2.1.2.2-1.

When eDRX_CONN is in use, the UE shall be able to identify a new detectable FDD intra frequency cell within Tidentify_intra_UE cat M1_NC as shown in table 8.13.2.1.2.2-1A.

Table 8.13.2.1.2.2-1: Requirement to identify a newly detectable HD-FDD intrafrequency cell

| Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1_NC (s) (DRX cycles) |
| --- | --- | --- |
| 0 | ≤0.04 | 1.44 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.04<DRX-cycle≤0.08 | Note 2 (40 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128 | 3.2 * Kintra_M1_NC * KRSTD_M1_NC (32 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128<DRX-cycle≤2.56 | Note 2 (25 * Kintra_M1_NC * KRSTD_M1_NC) |
| 1 | ≤0.08 | 2.88 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.128 | 3.2 * Kintra_M1_NC * KRSTD_M1_NC (32 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128<DRX-cycle≤2.56 | Note 2 (25 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |

Table 8.13.2.1.2.2-1A: Requirement to identify a newly detectable HD-FDD intrafrequency cell when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tidentify_intra_UE cat M1_NC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (25 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.1 and 9.1.21.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.6 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-2 for a corresponding Band

In the RRC_CONNECTED state the measurement period for intra frequency measurements is Tmeasure_intra_UE cat M1. When DRX is used, Tmeasure_intra_UE cat M1_NC is as specified in table 8.13.2.1.2.2-2. When eDRX_CONN is used, Tmeasure_intra_UE cat M1_NC is as specified in table 8.13.2.1.2.2-3. The UE shall be capable of performing RSRP and RSRQ measurements for 6 identified-intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_UE cat M1.

Table 8.13.2.1.2.2-2: Requirement to measure HD-FDD intrafrequency cells

| Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1_NC (s) (DRX cycles) |
| --- | --- | --- |
| 0 | <0.08 | 0.48 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.08≤DRX-cycle≤0.16 | Note 2 (7 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.16<DRX-cycle≤2.56 | Note 2 (5 * Kintra_M1_NC * KRSTD_M1_NC) |
| 1 | <0.16 | 0.96 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | DRX-cycle=0.16 | 1.12 * Kintra_M1_NC * KRSTD_M1_NC (7 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.16<DRX-cycle≤2.56 | Note 2 (5 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |

Table 8.13.2.1.2.2-3: Requirement to measure HD-FDD intrafrequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_intra_UE cat M1_NC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.1 and 9.1.21.2.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.6.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.2.1.2.2.1	Measurement Reporting Requirements

8.13.2.1.2.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

8.13.2.1.2.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2, and 9.1.21.6.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.2.1.2.2.1.3.

8.13.2.1.2.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra_UE cat M1_NC defined in Clause 8.13.2.1.2.2 When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_NC defined in clause 8.13.2.1.2.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_intra_UE cat M1_NC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

##### 8.13.2.1.3	E-UTRAN TDD intra frequency measurements

8.13.2.1.3.1	E-UTRAN intra frequency measurements when no DRX is used

When no DRX is in use, the UE shall be able to identify and measure a new detectable TDD intra frequency cell according to requirements in Table 8.13.2.1.3.1-1 when SCH Ês/Iot >= -6 dB, provided

-	G=1, or

-	rmax*G < 80ms, or

-	UE is receiving PDSCH.

Otherwise, requirements in Table 8.13.2.1.3.1-3 apply, where rmax and G are given by higher layer parameter mPDCCH-NumRepetition and mPDCCH-startSF-UESS respectively as defined in TS 36.213 [3].

Table 8.13.2.1.3.1-1: Requirement on cell identification delay and measurement delay for TDD intrafrequency cell

| Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- |
| 0 | 1.44 * Kintra_M1_NC * KRSTD_M1_NC seconds | 480 * Kintra_M1_NC * KRSTD_M1_NC ms |
| 1 | 2.88 * Kintra_M1_NC * KRSTD_M1_NC seconds | 960 * Kintra_M1_NC * KRSTD_M1_NC ms |

Kintra_M1_NC = 100 / X  where X is signalled by the RRC parameter measGapSharingScheme [2] and is defined as in Table 8.13.2.1.3.1-2 when highSpeedMeasGapCE-ModeA [2] is not configured, and in Table 8.13.2.1.3.1-2A when highSpeedMeasGapCE-ModeA [2] is configured.  is total number of inter-frequency layers to be monitored as defined in 8.1.2.1.1. When inter frequency measurement is not configured, Kintra_M1_NC=1 regardless whether or how parameter measGapSharingScheme [2] is configured.

Table 8.13.2.1.3.1-2: Value of parameter X for CEModeA

| measGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ |  |
| ‘01’ | 40 |
| ‘10’ | 50 |
| ‘11’ | 60 |

Table 8.13.2.1.3.1-2A: Value of parameter X for CEModeA for UE configured with highSpeedMeasGapCE-ModeA

| measGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ | [] |
| ‘01’ | [50] |
| ‘10’ | [80] |
| ‘11’ | [90] |

Table 8.13.2.1.3.1-3: Requirement on cell identification delay and measurement delay for TDD intrafrequency cell with MPDCCH scaling

| Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- |
| 0 | Max(20 * rmax*G / 1000, 1.44) * Kintra_M1_NC * KRSTD_M1_NC seconds | Max(5 * rmax*G, 480) * Kintra_M1_NC * KRSTD_M1_NC ms |
| 1 | Max(20 * rmax*G / 1000, 2.88) * Kintra_M1_NC * KRSTD_M1_NC seconds | Max(5 * rmax*G, 960) * Kintra_M1_NC * KRSTD_M1_NC ms |

KRSTD_M1_NC is applicable provided following conditions are met:

-	 > 40 ms

-	 >

- PRS bandwidth is less than the bandwidth of the cell used for RSTD measurement in which case gaps are required

where

-	is the cell-specific positioning subframe configuration period as defined in TS 36.211 [16],

-	 is the number of consecutive downlink positioning subframes in a positioning occation defined in TS 36.211

Otherwise KRSTD_M1_NC = 1.

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.1 and 9.1.21.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.6 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-1 for a corresponding Band.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_UE cat M1. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra frequency measurements is according to Table 8.13.2.1.3.1-1. When measurement gaps are activated the UE shall be capable of performing measurements for at least 6 cells. If the UE has identified more than 6 cells, the UE shall perform measurements but the reporting rate of RSRP and RSRQ measurements of cells from UE physical layer to higher layers may be decreased.

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.1 and 9.1.21.2.

The RSRQmeasurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.6.

8.13.2.1.3.1.1	Measurement Reporting Requirements

8.13.2.1.3.1.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

8.13.2.1.3.1.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.2.1.3.1.1.3.

8.13.2.1.3.1.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra_UE cat M1_NC defined in Clause 8.13.2.1.3.1. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_NC defined in clause 8.13.2.1.3.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than TMeasurement_Period Intra_UE cat M1_NC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

8.13.2.1.3.2	E-UTRAN intra frequency measurements when DRX is used

When DRX is in use the UE shall be able to identify a new detectable TDD intra frequency cell within Tidentify_intra_ UE catM1 as shown in table 8.13.2.1.3.2-1.

When eDRX_CONN is in use the UE shall be able to identify a new detectable TDD intra frequency cell within Tidentify_intra_UE cat M1_NC as shown in table 8.13.2.1.3.2-1A.

Table 8.13.2.1.3.2-1: Requirement to identify a newly detectable TDD intrafrequency cell

| Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1_NC (s) (DRX cycles) |
| --- | --- | --- |
| 0 | ≤0.04 | 1.44 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.04<DRX-cycle≤0.08 | Note 2 (40 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128 | 3.2 * Kintra_M1_NC * KRSTD_M1_NC (25 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128<DRX-cycle≤2.56 | Note 2 (20 * Kintra_M1_NC * KRSTD_M1_NC) |
| 1 | <0.128 | 2.88 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.128 | 3.2 * Kintra_M1_NC * KRSTD_M1_NC (25 * Kintra_M1_NC * KRSTD_M1_NC) |
|  | 0.128<DRX-cycle≤2.56 | Note 2 (20 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |

Table 8.13.2.1.3.2-1A: Requirement to identify a newly detectable TDD intrafrequency cell when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tidentify_intra_UE cat M1_NC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (20 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Clause 9.1.21.1 and 9.1.21.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.6 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-1 for a corresponding Band

In the RRC_CONNECTED state the measurement period for intra frequency measurements is Tmeasure_intra_UE cat M1. When DRX is used, Tmeasure_intra_UE cat M1_NC is as specified in table 8.13.2.1.3.2-2. When eDRX_CONN is used, Tmeasure_intra_UE cat M1_NC is as specified in table 8.13.2.1.3.2-3. The UE shall be capable of performing RSRP and RSRQ measurements for 6 identified intra-frequency cells and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_UE cat M1.

Table 8.13.2.1.3.2-2: Requirement to measure TDD intra frequency cells

| Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1_NC (s) (DRX cycles) |
| --- | --- | --- |
| 0 | <0.128 | 0.48 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.128≤DRX-cycle≤2.56 | Note 2 (5 * Kintra_M1_NC * KRSTD_M1_NC) |
| 1 | <0.256 | 0.96 * Kintra_M1_NC * KRSTD_M1_NC (Note 1) |
|  | 0.256≤DRX-cycle≤2.56 | Note 2 (5 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |

Table 8.13.2.1.3.2-3: Requirement to measure TDD intra frequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_intra_UE cat M1_NC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5 * Kintra_M1_NC * KRSTD_M1_NC) |
| Note: Time depends upon the eDRX_CONN cycle in use. |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.1 and 9.1.21.2.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.6.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.2.1.3.2.1	Measurement Reporting Requirements

8.13.2.1.3.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

8.13.2.1.3.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.2.1.3.2.1.3.

8.13.2.1.3.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.1, 9.1.21.2 and 9.1.21.6.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra_UE cat M1_NC defined in Clause 8.13.2.1.3.2. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_NC defined in clause 8.13.2.1.3.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_intra_UE cat M1_NC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

--- End of Change 1 ---

--- Start of Change 2 ---

##### 8.13.3.1.1	E-UTRAN FDD intra frequency measurements

8.13.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used

When no DRX is in use the UE shall be able to identify and measure a new detectable FDD intra frequency cell according to requirements in Table 8.13.3.1.1.1-1  provided that additional conditions table 8.13.3.1.1.1-1 is met, and

-	G=1, or

-	rmax*G < 800ms, or

-	UE is receiving PDSCH.

Otherwise, requirements in Table 8.13.3.1.1.1-4 apply, where rmax and G are given by higher layer parameter mPDCCH-NumRepetition and mPDCCH-startSF-UESS respectively as defined in TS 36.213 [3].

Table 8.13.3.1.1.1-1: Requirement on cell identification delay and measurement delay for FDD intrafrequency cell

| Neighouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | 320.8 * Kintra_M1_EC * KRSTD_M1_EC s | 800 * Kintra_M1_EC * KRSTD_M1_EC ms |
|  | 1 | 321.6 * Kintra_M1_EC * KRSTD_M1_EC s | 1600 * Kintra_M1_EC * KRSTD_M1_EC ms |
| Q2-6 | 0 | 21.8* Kintra_M1_EC * KRSTD_M1_EC s | 800 * Kintra_M1_EC * KRSTD_M1_EC ms |
|  | 1 | 22.6* Kintra_M1_EC* KRSTD_M1_EC s | 1600 * Kintra_M1_EC * KRSTD_M1_EC ms |

Table 8.13.3.1.1.1-2: Void

Kintra_M1_EC = 100 / X where X is signalled by the RRC parameter measGapSharingScheme [2] and is defined as in Table 8.13.3.1.1.1-3.  is total number of inter-frequency layers to be monitored as defined in 8.1.2.1.1. When inter frequency measurement is not configured, Kintra_M1_EC=1 regardless whether or how parameter measGapSharingScheme [2] is configured.

Table 8.13.3.1.1.1-3: Value of parameter X for CEModeB

| measGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ |  |
| ‘01’ | 50 |
| ‘10’ | 75 |
| ‘11’ | 87.5 |

Table 8.13.3.1.1.1-4: Requirement on cell identification delay and measurement delay for FDD intrafrequency cell

| Neighouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | Max(400 * rmax* G / 1000, 320.8) * Kintra_M1_EC * KRSTD_M1_EC s | Max(5 * rmax* G, 800) * Kintra_M1_EC * KRSTD_M1_EC ms |
|  | 1 | Max(400 * rmax* G / 1000, 321.6) * Kintra_M1_EC * KRSTD_M1_EC s | Max(5 * rmax* G, 1600) * Kintra_M1_EC * KRSTD_M1_EC ms |
| Q2-6 | 0 | Max(20 * rmax* G / 1000, 21.8)* Kintra_M1_EC * KRSTD_M1_EC s | Max(5 * rmax* G, 800) * Kintra_M1_EC * KRSTD_M1_EC ms |
|  | 1 | Max(20 * rmax* G / 1000, 22.6)* Kintra_M1_EC * KRSTD_M1_EC s | Max(5 * rmax* G, 1600) * Kintra_M1_EC * KRSTD_M1_EC ms |

KRSTD_M1_NC is applicable provided following conditions are met:

-	 > 40 ms

-	 >

- PRS bandwidth is less than the bandwidth of the cell used for RSTD measurement in which case gaps are required

where

-	is the cell-specific positioning subframe configuration period as defined in TS 36.211 [16],

-	 is the number of consecutive downlink positioning subframes in a positioning occation defined in TS 36.211

Otherwise KRSTD_M1_EC = 1.

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.3 and 9.1.21.4 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.7 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-3 for a corresponding Band.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_UE cat M1_EC. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra frequency measurements is according to Table 8.13.3.1.1.1-1. When measurement gaps are activated the UE shall be capable of performing measurements for at least 6 cells. If the UE has identified more than 6 cells, the UE shall perform measurements but the reporting rate of RSRP and RSRQ measurements of cells from UE physical layer to higher layers may be decreased.

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.7.

8.13.3.1.1.1.1	Measurement Reporting Requirements

8.13.3.1.1.1.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1. 21.3, 9.1.21.4 and  9.1.21.7.

8.13.3.1.1.1.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.3,9.1.21.4 and 9.1.21.7.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.1.1.1.1.3.

8.13.3.1.1.1.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resoureces for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra_UE cat M1_EC defined in Clause 8.13.3.1.1.1. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_EC defined in clause 8.13.3.1.1.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than TMeasurement_Period_UE cat M1_EC, Intra provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

8.13.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used

When DRX is in use the UE shall be able to identify a new detectable FDD intra frequency cell within Tidentify_intra_UE cat M1_EC as shown in table 8.13.3.1.1.2-1 provided that additional conditions Table 8.13.3.1.1.2-1 is met.

When eDRX_CONN is in use the UE shall be able to identify a new detectable FDD intra frequency cell within Tidentify_intra_UE cat M1_EC as shown in table 8.13.3.1.1.2-1B.

Table 8.13.3.1.1.2-1: Requirement to identify a newly detectable FDD intrafrequency cell

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | ≤0.64 | 320.8 * Kintra_M1_EC* KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2 (400 * Kintra_M1_EC* KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 321.6 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kintra_M1_EC* KRSTD_M1_EC) |
| Q2-6 | 0 | ≤0.64 | 21.8 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kintra_M1_EC* KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 22.6 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kintra_M1_EC* KRSTD_M1_EC) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.1.1.2-1A: Void

Table 8.13.3.1.1.2-1B: Requirement to identify a newly detectable FDD intrafrequency cell when eDRX_CONN is used

| eDRX_CONN cycle length (s) | Tidentify_intra_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (400 * Kintra_M1_EC* KRSTD_M1_EC) |
| NOTE: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.3 and 9.1.21.4 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.7 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-3 for a corresponding Band

In the RRC_CONNECTED state the measurement period for intra frequency measurements is Tmeasure_intra_UE cat M1_EC. When DRX is used, Tmeasure_intra_UE cat M1_EC is as specified in table 8.13.3.1.1.2-2 provided that additional conditions table 8.13.3.1.1.2-2 is met. When eDRX_CONN is used, Tmeasure_intra_UE cat M1_EC is as specified in table 8.13.3.1.1.2-4. The UE shall be capable of performing RSRP and RSRQ measurements for 6 identified-intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_UE cat M1_EC.

Table 8.13.3.1.1.2-2: Requirement to measure FDD intrafrequency cells

| Target cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| Q2-15 | 0 | ≤0.16 | 0.8 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.16<DRX-cycle≤2.56 | Note2(5 * Kintra_M1_EC* KRSTD_M1_EC) |
|  | 1 | ≤0.32 | 1.6 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.32<DRX-cycle≤2.56 | Note2(5 * Kintra_M1_EC* KRSTD_M1_EC) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.1.1.2-3: Void

Table 8.13.3.1.1.2-4: Requirement to measure FDD intrafrequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_intra_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5 * Kintra_M1_EC* KRSTD_M1_EC) |
| NOTE: Time depends upon the eDRX_CONN cycle in use |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.3 and 9.1.21.4.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.7.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.3.1.1.2.1	Measurement Reporting Requirements

8.13.3.1.1.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

8.13.3.1.1.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.1.1.2.1.3.

8.13.3.1.1.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra, UE cat M1_EC  defined in Clause 8.13.3.1.1.2 When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_EC  defined in clause 8.13.3.1.1.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_intra_UE cat M1_EC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

##### 8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD

8.13.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used

The requirements in this section are applicable for the UE which supports half duplex operation on one or more supported frequency bands [2].

The requirements defined in clause 8.13.3.1.1.1 also apply for this section provided the following conditions are met:

-	at least downlink subframe # 0 and downlink subframe # 5 per radio frame of an intra-frequency cell to be identified by the UE is available at the UE over Tidentify_intra_UE cat M1_EC;

-	at least two consecutive downlink subframe per radio frame of measured cell is available at the UE for RSRP measurements  assuming measured cell is identified cell over Tmeasure_intra_UE cat M1_EC.

-	RSRP related side conditions given in Sections 9.1.21.3 and 9.1.21.4 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.7 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-4

8.13.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used

The requirements in this section are applicable for the UE which supports half duplex operation on one or more supported frequency bands [2].

When DRX is in use the UE shall be able to identify a new detectable HD-FDD intra frequency cell within Tidentify_intra_UE cat M1_EC  as shown in table 8.13.3.1.2.2-1 provided that additional conditions table 8.13.3.1.2.2-1 is met.

When eDRX_CONN is in use, the UE shall be able to identify a new detectable FDD intra frequency cell within Tidentify_intra_UE cat M1_EC  as shown in table 8.13.3.1.2.2-1B.

Table 8.13.3.1.2.2-1: Requirement to identify a newly detectable HD-FDD intrafrequency cell

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | ≤0.64 | 320.8 * Kintra_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2 (400 * Kintra_M1 * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 321.6 * Kintra_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kintra_M1 * KRSTD_M1_EC) |
| Q2-6 | 0 | ≤0.64 | 21.8 * Kintra_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2 (24 * Kintra_M1 * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 22.6 * Kintra_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kintra_M1 * KRSTD_M1_EC) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.1.2.2-1A: Void

Table 8.13.3.1.2.2-1B: Requirement to identify a newly detectable HD-FDD intrafrequency cell when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tidentify_intra_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (400 * Kintra_M1_EC * KRSTD_M1_EC) |
| NOTE: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.3 and 9.1.21.4 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.7 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-4 for a corresponding Band

In the RRC_CONNECTED state the measurement period for intra frequency measurements is Tmeasure_intra_UE cat M1_EC. When DRX is used, Tmeasure_intra_UE cat M1_EC is as specified in table 8.13.3.1.2.2-2 provided that additional conditions Table 8.13.3.1.2.2-2 is met. When eDRX_CONN cycle is used, Tmeasure_intra_UE cat M1_EC is as specified in table 8.13.3.1.2.2-4. The UE shall be capable of performing RSRP and RSRQ measurements for 6 identified-intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_UE cat M1_EC.

Table 8.13.3.1.2.2-2: Requirement to measure HD-FDD intrafrequency cells

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| Q2-15 | 0 | <0.128 | 0.8 * Kintra _EC * KRSTD_M1_EC (Note1) |
|  |  | 0.128≤DRX-cycle≤0.16 | Note2 (7 * Kintra_EC * KRSTD_M1_EC) |
|  |  | 0.16<DRX-cycle≤2.56 | Note2(5 * Kintra_EC * KRSTD_M1_EC) |
|  | 1 | ≤0.32 | 1.6 * Kintra_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.32<DRX-cycle≤2.56 | Note2(5 * Kintra_EC * KRSTD_M1_EC) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.1.2.2-3: Void

Table 8.13.3.1.2.2-4: Requirement to measure HD-FDD intrafrequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_intra_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | NOTE (5 * Kintra_M1_EC * KRSTD_M1_EC) |
| NOTE: Time depends upon the eDRX_CONN cycle in use |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.3 and 9.1.21.4.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.7.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.3.1.2.2.1	Measurement Reporting Requirements

8.13.3.1.2.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

8.13.3.1.2.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.1.2.2.1.3.

8.13.3.1.2.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurements contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra_UE cat M1_EC defined in Clause 8.13.3.1.2.2 When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_EC defined in clause 8.13.3.1.2.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_intra_UE cat M1_EC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

##### 8.13.3.1.3	E-UTRAN TDD intra frequency measurements

8.13.3.1.3.1	E-UTRAN intra frequency measurements when no DRX is used

When no DRX is in use the UE shall be able to identify and measure a new detectable TDD intra frequency cell according to requirements in Table 8.13.3.1.3.1-1provided that additional conditions Table 8.13.3.1.3.1-2 is met, and

-	G=1, or

-	rmax*G < 800ms, or

-	UE is receiving PDSCH.

Otherwise, requirements in Table 8.13.3.1.3.1-4 apply, where rmax and G are given by higher layer parameter mPDCCH-NumRepetition and mPDCCH-startSF-UESS respectively as defined in TS 36.213 [3].

Table 8.13.3.1.3.1-1: Requirement on cell identification delay and measurement delay for TDD intrafrequency cell

| Neighouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) for neighbouring cell SCH Ês/Iot (Q): -15≤ Q2 < -6 | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | 320.8 * Kintra_M1_EC * KRSTD_M1_EC s | 800 * Kintra_M1_EC * KRSTD_M1_EC Note1 [1600 * Kintra_M1_EC * KRSTD_M1_EC] Note2 |
|  | 1 | 321.6 * Kintra_M1_EC * KRSTD_M1_EC s | 1600 * Kintra_M1_EC * KRSTD_M1_EC ms Note1 [3200 * Kintra_M1_EC ms] Note2 |
| Q2-6 | 0 | 21.8 * Kintra_M1_EC * KRSTD_M1_EC S | 800 * Kintra_M1_EC * KRSTD_M1_EC ms Note1 [1600 * Kintra_M1_EC * KRSTD_M1_EC ms] Note2 |
|  | 1 | 22.6 * Kintra_M1_EC * KRSTD_M1_EC S | 1600 * Kintra_M1_EC * KRSTD_M1_EC ms Note1 [3200 * Kintra_M1_EC * KRSTD_M1_EC ms] Note2 |
| Note1: Under TDD UL/DL configuration other than 0. Note2: Under TDD UL/DL configuration 0. |  |  |  |

Kintra_M1_EC = 100 / X where X is signalled by the RRC parameter measGapSharingScheme [2] and is defined as in Table 8.13.3.1.3.1-3.  is total number of inter-frequency layers to be monitored as defined in 8.1.2.1.1. When inter frequency measurement is not configured, Kintra_M1_EC=1 regardless whether or how parameter measGapSharingScheme [2] is configured.

Table 8.13.3.1.3.1-2: Void

Table 8.13.3.1.3.1-3: Value of parameter X for CEModeB

| measGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ |  |
| ‘01’ | 50 |
| ‘10’ | 75 |
| ‘11’ | 87.5 |

Table 8.13.3.1.3.1-4: Requirement on cell identification delay and measurement delay for TDD intrafrequency cell

| Neighouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | Max(400 * rmax* G / 1000, 320.8) * Kintra_M1_EC * KRSTD_M1_EC s | Max(5 * rmax* G, 800) * Kintra_M1_EC * KRSTD_M1_EC ms Note1 [Max(5 * rmax* G, 1600) * Kintra_M1_EC * KRSTD_M1_EC ms] Note2 |
|  | 1 | Max(400 * rmax* G / 1000, 321.6) * Kintra_M1_EC * KRSTD_M1_EC s | Max(5 * rmax* G, 1600) * Kintra_M1_EC * KRSTD_M1_EC ms Note1 [Max(5 * rmax* G, 3200) * Kintra_M1_EC * KRSTD_M1_EC ms] Note2 |
| Q2-6 | 0 | Max(20 * rmax* G / 1000, 21.8)* Kintra_M1_EC * KRSTD_M1_EC s | Max(5 * rmax* G, 800) * Kintra_M1_EC * KRSTD_M1_EC ms Note1 [Max(5 * rmax* G, 1600) * Kintra_M1_EC * KRSTD_M1_EC ms] Note2 |
|  | 1 | Max(20 * rmax* G / 1000, 22.6)* Kintra_M1_EC * KRSTD_M1_EC s | Max(5 * rmax* G, 1600) * Kintra_M1_EC * KRSTD_M1_EC ms Note1 [Max(5 * rmax* G, 3200) * Kintra_M1_EC * KRSTD_M1_EC ms] Note2 |
| Note1: Under TDD UL/DL configuration other than 0. Note2: Under TDD UL/DL configuration 0. |  |  |  |

KRSTD_M1_NC is applicable provided following conditions are met:

-	 > 40 ms

-	 >

- PRS bandwidth is less than the bandwidth of the cell used for RSTD measurement in which case gaps are required

where

-	is the cell-specific positioning subframe configuration period as defined in TS 36.211 [16],

-	 is the number of consecutive downlink positioning subframes in a positioning occation defined in TS 36.211

Otherwise KRSTD_M1_EC = 1.

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.3 and 9.1.21.4 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.7 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-3 for a corresponding Band.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_UE cat M1_EC. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra frequency measurements is according to Table 8.13.3.1.3.1-1. When measurement gaps are activated the UE shall be capable of performing measurements for at least 6 cells. If the UE has identified more than 6 cells, the UE shall perform measurements but the reporting rate of RSRP and RSRQ measurements of cells from UE physical layer to higher layers may be decreased.

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.3 and 9.1.21.4.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.7.

8.13.3.1.3.1.1	Measurement Reporting Requirements

8.13.3.1.3.1.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

8.13.3.1.3.1.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.1.3.1.1.3.

8.13.3.1.3.1.1.3	Event Triggered Reporting

Reported RSRP measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra_UE cat M1_EC defined in Clause 8.13.3.1.3.1. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_EC defined in clause 8.13.3.1.3.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than TMeasurement_Period Intra_UE cat M1_EC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

8.13.3.1.3.2	E-UTRAN intra frequency measurements when DRX is used

When DRX is in use the UE shall be able to identify a new detectable TDD intra frequency cell within Tidentify_intra_ UE cat M1_EC as shown in table 8.13.3.1.3.2-1 provided that additional conditions table 8.13.3.1.3.2-1 is met.

When eDRX_CONN is in use the UE shall be able to identify a new detectable TDD intra frequency cell within Tidentify_intra_UE cat M1_EC as shown in table 8.13.3.1.3.2-1B.

Table 8.13.3.1.3.2-1: Requirement to identify a newly detectable TDD intrafrequency cell

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | ≤0.64 | 320.8 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2 (400 * Kintra_M1_EC * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 321.6 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kintra_M1_EC * KRSTD_M1_EC) |
| Q2-6 | 0 | ≤0.64 | 21.8 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kintra_M1_EC * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 22.6 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kintra_M1_EC* KRSTD_M1_EC) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.1.3.2-1A: Void

Table 8.13.3.1.3.2-1B: Requirement to identify a newly detectable TDD intrafrequency cell when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tidentify_intra_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (400 * Kintra_M1_EC * KRSTD_M1_EC) |
| NOTE: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Clause 9.1.21.3 and 9.1.21.4 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.7 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.14-3 for a corresponding Band

In the RRC_CONNECTED state the measurement period for intra frequency measurements is Tmeasure_intra_UE cat M1_EC. When DRX is used, Tmeasure_intra_UE cat M1_EC is as shown in table 8.13.3.1.3.2-2 provided that additional conditions Table 8.13.3.1.3.2-2 is met. When eDRX_CONN is used, Tmeasure_intra_UE cat M1_EC is as shown in table 8.13.3.1.3.2-4. The UE shall be capable of performing RSRP and RSRQ measurement for 6 identified intra-frequency cells and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_UE cat M1_EC.

Table 8.13.3.1.3.2-2: Requirement to measure TDD intra frequency cells

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | TDD Uplink-downlink configuration | Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- | --- |
| Q2-15 | Other than 0 | 0 | ≤0.16 | 0.8 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  |  | 0.16<DRX-cycle≤2.56 | Note2 (5 * Kintra_M1_EC * KRSTD_M1_EC) |
|  |  | 1 | ≤0.32 | 1.6 * Kintra_M1_EC * KRSTD_M1_EC (Note1) |
|  |  |  | 0.32<DRX-cycle≤2.56 | Note2(5 * Kintra_M1_EC * KRSTD_M1_EC) |
|  | 0 | 0 | ≤0.32 | [1.6 * Kintra_M1_EC * KRSTD_M1_EC] (Note1) |
|  |  |  | 0.32<DRX-cycle≤2.56 | Note2 (5 * Kintra_M1_EC * KRSTD_M1_EC) |
|  |  | 1 | ≤0.64 | [3.2 * Kintra_M1_EC * KRSTD_M1_EC] (Note1) |
|  |  |  | 0.64<DRX-cycle≤2.56 | Note2 (5 * Kintra_M1_EC * KRSTD_M1_EC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use. Note 2: Time depends upon the DRX cycle in use. |  |  |  |  |

Table 8.13.3.1.3.2-3: Void

Table 8.13.3.1.3.2-4: Requirement to measure TDD intra frequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_intra_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5 * Kintra_M1_EC * KRSTD_M1_EC) |
| NOTE: Time depends upon the eDRX_CONN cycle in use. |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.3 and 9.1.21.4.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.7.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.3.1.3.2.1	Measurement Reporting Requirements

8.13.3.1.3.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

8.13.3.1.3.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.1.3.2.1.3.

8.13.3.1.3.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.3, 9.1.21.4 and 9.1.21.7.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra_UE cat M1_EC defined in Clause 8.13.3.1.3.2. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1_EC defined in clause 8.13.3.1.3.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_intra_UE cat M1_EC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

--- End of Change 2 ---

--- Start of Change 3 ---

#### 8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B

The UE shall be able to identify new inter-frequency cells and perform RSRP and RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the PCell, even if no explicit neighbour list with physical layer cell identities is provided. During the RRC_CONNECTED state the UE shall continuously measure identified inter frequency cells and additionally search for and identify new inter frequency cells.

##### 8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements

8.13.3.5.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used

When no DRX is in use and when measurement gaps are scheduled, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify and measure a new detectable FDD inter-frequency cell according to requirements in Table 8.13.3.5.1.1-1 when additional condition in Table 8.13.3.5.1.1-1 is met, and

-	G=1, or

-	rmax*G < 800ms, or

-	UE is receiving PDSCH.

Otherwise, requirements in Table 8.13.3.5.1.1-3 apply, where rmax and G are given by higher layer parameter mPDCCH-NumRepetition and mPDCCH-startSF-UESS respectively as defined in TS 36.213 [3].

Table 8.13.3.5.1.1-1: Requirement on cell identification delay and measurement delay for FDD interfrequency cell

| Neighouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | 320.8 * Kinter_M1_EC * KRSTD_M1_EC s | 800 * Kinter_M1_EC * KRSTD_M1_EC ms |
|  | 1 | 321.6 * Kinter_M1 s | 1600 * Kinter_M1 ms |
| Q2-6 | 0 | 21.8 * Kinter_M1_EC * KRSTD_M1_EC s | 800 * Kinter_M1_EC * KRSTD_M1_EC ms |
|  | 1 | 22.6 * Kinter_M1_EC s | 1600 * Kinter_M1_EC ms |

where X is signalled by the RRC parameter measGapSharingScheme [2] and is defined as in Table 8.13.3.5.1.1-2.  is total number of inter-frequency layers to be monitored as defined in 8.1.2.1.1.

Table 8.13.3.5.1.1-2: Value of parameter X for CEModeB

| measGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ |  |
| ‘01’ | 50 |
| ‘10’ | 75 |
| ‘11’ | 87.5 |

Table 8.13.3.5.1.1-3: Requirement on cell identification delay and measurement delay for FDD interfrequency cell

| Neighouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | Cell identification delay (Tidentify_inter_UE cat M1) | Measurement delay (Tmeasure_inter_UE cat M1) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | Max(400 * rmax* G / 1000, 320.8) * Kinter_M1_EC* KRSTD_M1_EC s | Max(5 * rmax* G, 800) * Kinter_M1_EC* KRSTD_M1_EC ms |
|  | 1 | Max(400 * rmax* G / 1000, 321.6) * Kinter_M1_EC* KRSTD_M1_EC s | Max(5 * rmax* G, 1600) * Kinter_M1_EC* KRSTD_M1_EC ms |
| Q2-6 | 0 | Max(20 * rmax* G / 1000, 21.8)* Kinter_M1_EC* KRSTD_M1_EC s | Max(5 * rmax* G, 800) * Kinter_M1_EC* KRSTD_M1_EC ms |
|  | 1 | Max(20 * rmax* G / 1000, 22.6)* Kinter_M1_EC* KRSTD_M1_EC s | Max(5 * rmax* G, 1600) * Kinter_M1_EC* KRSTD_M1_EC ms |

KRSTD_M1_NC is applicable provided following conditions are met:

-	 > 40 ms

-	 >

where

-	 is the cell-specific positioning subframe configuration period as defined in TS 36.211 [16],

-	 is the number of consecutive downlink positioning subframes in a positioning occation defined in TS 36.211

Otherwise  KRSTD_M1_EC = 1.

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.11 and 9.1.21.12 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.15 and 9.1.21.16 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.18-1 for a corresponding Band.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_inter_UE cat M1_EC. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for inter frequency measurements is according to Table 8.13.3.5.1.1-1. When measurement gaps are scheduled for FDD inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting RSRP and RSRQ measurements to higher layers with measurement accuracy as specified in sub-clauses 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The UE shall be capable of performing RSRP and RSRQ measurements of at least 4 inter-frequency cells per FDD inter-frequency for up to 3 FDD inter-frequencies and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements to higher layers with the measurement period defined in Table 8.13.3.5.1.1-1.

8.13.3.5.1.1.1	Measurement Reporting Requirements

8.13.3.5.1.1.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

8.13.3.5.1.1.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ  measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.5.1.1.1.3.

8.13.3.5.1.1.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resoureces for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify inter_UE cat M1_EC defined in Clause 8.13.3.5.1.1. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_inter_UE cat M1_EC defined in clause 8.13.3.5.1.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than TMeasurement_Period_UE cat M1_EC, Inter provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

8.13.3.5.1.2	E-UTRAN inter frequency measurements when DRX is used

When DRX is in use and when measurement gaps are scheduled, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable FDD inter-frequency cell within Tidentify_inter_UE cat M1_EC as shown in table 8.13.3.5.1.2-1.

When eDRX_CONN is in use and when measurement gaps are scheduled, or the UE supports capability of conducting such measurements without gaps,  the UE shall be able to identify a new detectable FDD inter-frequency cell within Tidentify_inter_UE cat M1_EC as shown in table 8.13.3.5.1.2-1.

Table 8.13.3.5.1.2-1: Requirement to identify a newly detectable FDD interfrequency cell

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | ≤0.64 | 320.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kinter_M1 * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 321.6 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kinter_M1 * KRSTD_M1_EC) |
| Q2-6 | 0 | ≤0.64 | 21.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kinter_M1 * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 22.6 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kinter_M1 * KRSTD_M1_EC) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.5.1.2-1B: Requirement to identify a newly detectable FDD interfrequency cell when eDRX_CONN is used

| eDRX_CONN cycle length (s) | Tidentify_inter_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (400 * Kinter_M1 * KRSTD_M1_EC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.11 and 9.1.21.12 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.15 and 9.1.21.16 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex B.2.18-1 for a corresponding Band

When DRX or eDRX_CONN is in use, the UE shall be capable of performing RSRP and RSRQ measurements of at least 4 inter-frequency cells per FDD inter-frequency and the UE physical layer shall be capable of reporting RSRP and RSRQ to higher layers with the measurement period Tmeasure_inter_UE cat M1_EC, either measurement gaps are scheduled or the UE supports capability of conducting such measurements without gaps. When DRX is used, Tmeasure_inter_UE cat M1_EC is as defined in Table 8.13.3.5.1.2-2, and when eDRX_CONN is in use, Tmeasure_inter_UE cat M1_EC is as defined in Table 8.13.3.5.1.2-2.

Table 8.13.3.5.1.2-2: Requirement to measure FDD interfrequency cells

| Target cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| Q2-15 | 0 | ≤0.16 | 0.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.16<DRX-cycle≤2.56 | Note2(5 * Kinter_M1 * KRSTD_M1_EC) |
|  | 1 | ≤0.32 | 1.6 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.32<DRX-cycle≤2.56 | Note2(5 * Kinter_M1 * KRSTD_M1_EC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.5.1.2-3: Requirement to measure FDD interfrequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_inter_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5* Kinter_M1 * KRSTD_M1_EC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.11 and 9.1.21.12.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses  9.1.21.15 and 9.1.21.16.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.3.5.1.2.1	Measurement Reporting Requirements

8.13.3.5.1.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

8.13.3.5.1.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.5.1.2.1.3.

8.13.3.5.1.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_inter, UE cat M1_EC  defined in Clause 8.13.3.5.1.2 When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_inter_UE cat M1_EC  defined in clause 8.13.3.5.1.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_inter_UE cat M1_EC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

##### 8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD

8.13.3.5.2.1	E-UTRAN inter-frequency measurements when no DRX is used

The requirements in this section are applicable for the UE which supports half duplex operation on one or more supported frequency bands [2].

The requirements defined in clause 8.13.3.5.1.1 also apply for this section provided the following conditions are met:

-	RSRP related side conditions given in Sections 9.1.21.11 and 9.1.21.12 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.15 and 9.1.21.16 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.18-2 for a corresponding Band

8.13.3.5.2.2	E-UTRAN inter frequency measurements when DRX is used

The requirements in this section are applicable for the UE which supports half duplex operation on one or more supported frequency bands [2].

When DRX is in use and when measurement gaps are scheduled, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable FDD inter-frequency cell within Tidentify_inter_UE cat M1_EC as shown in table 8.13.3.5.2.2-1.

When eDRX_CONN is in use and when measurement gaps are scheduled, or the UE supports capability of conducting such measurements without gaps,  the UE shall be able to identify a new detectable FDD inter-frequency cell within Tidentify_inter_UE cat M1_EC as shown in table 8.13.3.5.2.2-1.

Table 8.13.3.5.2.2-1: Requirement to identify a newly detectable HD-FDD interfrequency cell

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | ≤0.64 | 320.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kinter_M1 * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 321.6 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kinter_M1 * KRSTD_M1_EC) |
| Q2-6 | 0 | ≤0.64 | 21.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kinter_M1 * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 22.6 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kinter_M1 * KRSTD_M1_EC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.5.2.2-1B: Requirement to identify a newly detectable HD-FDD interfrequency cell when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tidentify_inter_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (400 * Kinter_M1 * KRSTD_M1_EC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.11 and 9.1.21.12 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.15 and 9.1.21.16 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.18-2 for a corresponding Band

When DRX or eDRX_CONN is in use, the UE shall be capable of performing RSRP and RSRQ measurements of at least 4 inter-frequency cells per FDD inter-frequency and the UE physical layer shall be capable of reporting RSRP and RSRQ to higher layers with the measurement period Tmeasure_inter_UE cat M1_EC, either measurement gaps are scheduled or the UE supports capability of conducting such measurements without gaps. When DRX is used, Tmeasure_inter_UE cat M1_EC is as defined in Table 8.13.3.5.2.2-2, and when eDRX_CONN is in use, Tmeasure_inter_UE cat M1_EC is as defined in Table 8.13.3.5.2.2-2.

Table 8.13.3.5.2.2-2: Requirement to measure HD-FDD interfrequency cells

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| Q2-15 | 0 | <0.128 | 0.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.128≤DRX-cycle≤0.16 | Note2 (7 * Kinter_M1 * KRSTD_M1_EC) |
|  |  | 0.16<DRX-cycle≤2.56 | Note2(5 * Kinter_M1 * KRSTD_M1_EC) |
|  | 1 | ≤0.32 | 1.6 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.32<DRX-cycle≤2.56 | Note2(5 * Kinter_M1 * KRSTD_M1_EC) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.5.2.2-3: Requirement to measure HD-FDD interfrequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_inter_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5 * Kinter_M1 * KRSTD_M1_EC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.11 and 9.1.21.12.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses  9.1.21.15 and 9.1.21.16.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.3.5.2.2.1	Measurement Reporting Requirements

8.13.3.5.2.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

8.13.3.5.2.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.5.2.2.1.3.

8.13.3.5.2.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_inter_UE cat M1_EC defined in Clause 8.13.3.5.2.2 When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_inter_UE cat M1_EC defined in clause 8.13.3.5.2.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_inter_UE cat M1_EC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

##### 8.13.3.5.3	E-UTRAN TDD inter frequency measurements

8.13.3.5.3.1	E-UTRAN inter frequency measurements when no DRX is used

When no DRX is in use and when measurement gaps are scheduled, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify and measure a new detectable TDD inter frequency cell according to requirements in Table 8.13.3.5.3.1-1 when additional condition in Table 8.13.3.5.3.1-1 is met, and

-	G=1, or

-	rmax*G < 800ms, or

-	UE is receiving PDSCH.

Otherwise, requirements in Table 8.13.3.5.3.1-3 apply, where rmax and G are given by higher layer parameter mPDCCH-NumRepetition and mPDCCH-startSF-UESS respectively as defined in TS 36.213 [3].

Table 8.13.3.5.3.1-1: Requirement on cell identification delay and measurement delay for TDD interfrequency cell

| Neighouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | Cell identification delay (Tidentify_intra_UE cat M1) for neighbouring cell SCH Ês/Iot (Q): -15≤ Q2 < -6 [dB] | Measurement delay (Tmeasure_intra_UE cat M1) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | 320.8 * Kinter_M1_EC * KRSTD_M1_EC s | 800 * Kinter _M1_EC * KRSTD_M1_EC ms Note1 [1600 * Kinter _M1_EC * KRSTD_M1_EC ms] Note2 |
|  | 1 | 321.6 * Kinter_M1_EC s | 1600 * Kinter_M1_EC ms Note1 [3200 * Kinter_M1_EC ms] Note2 |
| Q2-6 | 0 | 21.8 * Kinter_M1_EC * KRSTD_M1_EC S | 800 * Kinter_M1_EC * KRSTD_M1_EC ms Note1 [1600 * Kinter_M1_EC * KRSTD_M1_EC ms] Note2 |
|  | 1 | 22.6 * Kinter_M1_EC S | 1600 * Kinter_M1_EC ms Note1 [3200 * Kinter_M1_EC ms] Note2 |
| Note 1: Under TDD UL/DL configuration other than 0. Note 2: Under TDD UL/DL configuration 0. |  |  |  |

where X is signalled by the RRC parameter measGapSharingScheme and is defined as in Table 8.13.3.5.3.1-2.  is total number of inter-frequency layers to be monitored as defined in 8.1.2.1.1.

Table 8.13.3.5.3.1-2: Value of parameter X for CEModeB

| measGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ |  |
| ‘01’ | 50 |
| ‘10’ | 75 |
| ‘11’ | 87.5 |

Table 8.13.3.5.3.1-3: Requirement on cell identification delay and measurement delay for TDD interfrequency cell

| Neighouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | Cell identification delay (Tidentify_inter_UE cat M1) | Measurement delay (Tmeasure_inter_UE cat M1) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | Max(400 * rmax* G / 1000, 320.8) * Kinter_M1_EC* KRSTD_M1_EC s | Max(5 * rmax* G, 800) * Kinter_M1_EC* KRSTD_M1_EC ms Note1 [Max(5 * rmax* G, 1600) * Kinter_M1_EC* KRSTD_M1_EC ms] Note2 |
|  | 1 | Max(400 * rmax* G / 1000, 321.6) * Kinter_M1_EC* KRSTD_M1_EC s | Max(5 * rmax* G, 1600) * Kinter_M1_EC* KRSTD_M1_EC ms Note1 [Max(5 * rmax* G, 3200) * Kinter_M1_EC* KRSTD_M1_EC ms] Note2 |
| Q2-6 | 0 | Max(20 * rmax* G / 1000, 21.8)* Kinter_M1_EC* KRSTD_M1_EC s | Max(5 * rmax* G, 800) * Kinter_M1_EC* KRSTD_M1_EC ms Note1 [Max(5 * rmax* G, 1600) * Kinter_M1_EC* KRSTD_M1_EC ms] Note2 |
|  | 1 | Max(20 * rmax* G / 1000, 22.6)* Kinter_M1_EC* KRSTD_M1_EC s | Max(5 * rmax* G, 1600) * Kinter_M1_EC* KRSTD_M1_EC ms Note1 [Max(5 * rmax* G, 3200) * Kinter_M1_EC* KRSTD_M1_EC ms] Note2 |
| Note1: Under TDD UL/DL configuration other than 0. Note2: Under TDD UL/DL configuration 0. |  |  |  |

KRSTD_M1_NC is applicable provided following conditions are met:

-	 > 40 ms

-	 >

where

-	 is the cell-specific positioning subframe configuration period as defined in TS 36.211 [16],

-	 is the number of consecutive downlink positioning subframes in a positioning occation defined in TS 36.211

Otherwise KRSTD_M1_EC = 1.

A cell shall be considered detectable when

-	RSRP related side conditions given in Sections 9.1.21.11 and 9.1.21.12 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.15 and 9.1.21.16 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.16-1 for a corresponding Band.

When measurement gaps are scheduled for TDD inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting RSRP and RSRQ to higher layers with measurement accuracy as specified in sub-clauses 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16 with measurement period (Tmeasure_inter_UE cat M1_EC) given by table 8.13.3.5.3.1-1:

The UE shall be capable of performing RSRP and RSRQ measurements of at least 4 inter-frequency cells per TDD inter-frequency and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements to higher layers with the measurement period Tmeasure_inter_UE cat M1_EC.

8.13.3.5.3.1.1	Measurement Reporting Requirements

8.13.3.5.3.1.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

8.13.3.5.3.1.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.5.3.1.1.3.

8.13.3.5.3.1.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify inter_UE cat M1_EC defined in Clause 8.13.3.5.3.1. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_inter_UE cat M1_EC defined in clause 8.13.3.5.3.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than TMeasurement_Period Inter_UE cat M1_EC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

8.13.3.5.3.2	E-UTRAN inter frequency measurements when DRX is used

When DRX is in use and when DRX or eDRX_CONN is in use, either measurement gaps are scheduled or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable TDD inter frequency cell within Tidentify_inter_ UE catM1 as shown in table 8.13.3.5.3.2-1.

When eDRX_CONN is in use the UE shall be able to identify a new detectable TDD inter frequency cell within Tidentify_inter_UE cat M1_EC as shown in table 8.13.3.5.3.2-1.

Table 8.13.3.5.3.2-1: Requirement to identify a newly detectable TDD interfrequency cell

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0 | ≤0.64 | 320.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kinter_M1 * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 321.6 * Kinter_M1 (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(400 * Kinter_M1) |
| Q2-6 | 0 | ≤0.64 | 21.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kinter_M1 * KRSTD_M1_EC) |
|  | 1 | DRX-cycle ≤ 0.640 | 22.6 * Kinter_M1 (Note1) |
|  |  | 0.64< DRX-cycle≤2.56 | Note2(24 * Kinter_M1) |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use Note 2: Time depends upon the DRX cycle in use |  |  |  |

Table 8.13.3.5.3.2-1A: Requirement to identify a newly detectable TDD interfrequency cell when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tidentify_inter_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (400 * Kinter_M1 * KRSTD_M1_EC) |
| Note: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Clause 9.1.21.11 and 9.1.21.12 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Clause 9.1.21.15 and 9.1.21.16 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex Table B.2.16-1 for a corresponding Band

When DRX or eDRX_CONN is in use, the UE shall be capable of performing RSRP and RSRQ measurements of at least 4 inter-frequency cells per TDD inter-frequency for up to 3 TDD inter-frequencies and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements to higher layers with the measurement period Tmeasure_inter_UE cat M1_EC, either measurement gaps are scheduled or the UE supports capability of conducting such measurements without gaps. When DRX is in use, Tmeasure_inter_UE cat M1_EC is as defined in Table 8.13.3.5.3.2-2, and when eDRX_CONN is in use, Tmeasure_inter_UE cat M1_EC  is as defined in Table 8.13.3.5.3.2-2.

Table 8.13.3.5.3.2-2: Requirement to measure TDD inter frequency cells

| Neighbouring cell SCH Ês/Iot: Q2 [dB] | TDD Uplink-downlink configuration | Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- | --- | --- |
| Q2-15 | Other than 0 | 0 | ≤0.16 | 0.8 * Kinter_M1 * KRSTD_M1_EC (Note1) |
|  |  |  | 0.16<DRX-cycle≤2.56 | Note2(5 * Kinter_M1 * KRSTD_M1_EC) |
|  |  | 1 | ≤0.32 | 1.6 * Kinter_M1 (Note1) |
|  |  |  | 0.32<DRX-cycle≤2.56 | Note2(5 * Kinter_M1) |
|  | 0 | 0 | ≤0.32 | [1.6 * Kinter_M1 * KRSTD_M1_EC] (Note1) |
|  |  |  | 0.32<DRX-cycle≤2.56 | Note2(5 * Kinter_M1 * KRSTD_M1_EC) |
|  |  | 1 | ≤0.64 | [3.2 * Kinter_M1] (Note1) |
|  |  |  | 0.64<DRX-cycle≤2.56 | Note2(5) * Kinter_M1 |
| Note 1: Number of DRX cycle depends upon the DRX cycle in use. Note 2: Time depends upon the DRX cycle in use. |  |  |  |  |

Table 8.13.3.5.3.2-3: Requirement to measure TDD inter frequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_inter_UE cat M1_EC (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5 * Kinter_M1 * KRSTD_M1_EC) |
| Note: Time depends upon the eDRX_CONN cycle in use. |  |

The RSRP measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.11 and 9.1.21.12.

The RSRQ measurement accuracy for all measured cells shall be as specified in the sub-clauses 9.1.21.15 and 9.1.21.16.

The requriements in this subcluse apply regardless of MPDCCH monitoring configuration.

8.13.3.5.3.2.1	Measurement Reporting Requirements

8.13.3.5.3.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurement contained in periodically triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

8.13.3.5.3.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurement contained in event triggered periodic measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.13.3.5.3.2.1.3.

8.13.3.5.3.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurement contained in event triggered measurement reports shall meet the requirements in sections 9.1.21.11, 9.1.21.12, 9.1.21.15 and 9.1.21.16.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: pusch-maxNumRepetitionCEmodeB x TTIDCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured for the UE in CE Mode B provided that pusch-maxNumRepetitionCEmodeB >1, othwerwise uncertainty is defined as 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_inter_UE cat M1_EC defined in Clause 8.13.3.5.3.2. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_inter_UE cat M1_EC defined in clause 8.13.3.5.3.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_inter_UE cat M1_EC provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

--- End of Change 3 ---