3GPP TSG-RAN WG4 Meeting #89                	R4-1816345

Spokane, Washington, USA, 12 - 16 November 2018

Source:	Huawei HiSilicon

Title:	Ad Hoc minutes for NR BS demodulation

Agenda Item:	7.13.2.1

Document for:	Discussion

Left open issues

Issue 1: Test applicability for PUSCH and PUCCH tests with different SCS and CBW combinations

Agreements:

  - Test all declared SCS, for each declared SCS, BS is required to test the highest CBW declared.

    - If the largest CBW declared is no in the subset with defined performance requirements, BS will be tested on the nearest lower BW (i.e. reference BW) in the subset, the reference BW will be placed in the middle of the channel BW during the test.

Issue 2: Test applicability for different PRACH formats with requirements defined

Agreements:

For those agreed PRACH formats, i.e. short sequence format A1, A2, A3, B4, C0 and C2 and long sequence format 0, BS is tested on all the supported preamble formats, and just choose one of the supported PRACH SCS per FR for PRACH formats with short sequence.

Issue 3: Test applicability for different configurations

Maybe we can discuss the following features’ test applicability case by case if company are not ok with the general rules

- PUSCH time domain resource allocation:

  - Option1: Type B for non-slot based.

  - Option2: Type B for slot based

  - Option3: Type B for both slot and non-slot based.

  - Option4: Type B not tested

- DMRS configurations with and without additional DMRS

  - PUSCH FR2

    - 1+0

    - 1+0 and 1+1

  - PUCCH format 3 & 4: FR2

    - Without additional DMRS

    - With and without additional DMRS

- PT-RS configuration

- Option 1:

  - QPSK: Do not configure PT-RS

  - 16QAM and 64QAM: Both PT-RS configured and not-configured

- Option 2: PT-RS configured

Ericsson: It is still needed to introduce new performance for Rel-15.

ZTE: BS declare configuration without requirements defined, then vendors should declare achievable performance for this configuration for test purpose, at same time, more efforts should be conducted to introduce the related requirements,

Ericsson: maybe it is feasible for PUCCH format 3 and 4 with and without additional DMRS configuration.

Nokia: FRC will be different for different configuration.

Agreement: No new test cases will be introduced for this meeting, and new requirements for other declared supported configurations shall be introduced after Nov..

Issue 4: Phase noise modelling

- Whether PN is modelled in FR2 test cases

  - Option 1: Yes

  - Option 2: No and add a certain amount of margin on top of the final averaged performance requirements for phase noise impact.

Nokia : postpone the discussion. Submitted ideal results do not included phase noise model and impairment results consider the phse noise impact. Nokia will bring results with explicit phase noise modelling.

SNR value should model phase noise someway.

Choose option 1 and do not preclude company introduce impaurment with phase noise impact

Ericsson : just model it and do not distinguish ideal and impairement results. Out ideal results do not include the phase noise, no impairent results submiteed.

Agree with Nokia. Would not take risk value in the end for those artifical value.

Fixed margin without PN, margin cannot cover the phase noise impact.

ZTE : Ideal results do not include the phase noise, did not submit impairment results, should provide informaton about how to consider phase noise impact if it is feasible or acceptable.

How to derive the margin value for different case ?

In order to derive the feasible margin value for each case evntually, only one option 1, because no uniform margin value for each cases.

Samsung : no uniform value and based on their implementation. Different configurations and implementation, why to exactly model the phase noise.

- Whether PN is modelled in FR2 test cases

  - Option 1: Yes (Ericsson, Nokia)

  - Option 2: No and add a certain amount of margin on top of the final averaged performance requirements for phase noise impact. (Samsung, Huawei)

Issue 5: CBW for DFT-s-OFDM

  - CBW for FR2 120kHz SCS

    - Change CBW from 100MHz to 50MHz for FR2 120kHz SCS

Nokia: we do not know what is the impact to the simulation if we change the channel bandwidth.

It is fine for us., agree with CATT

CATT: we just change the BW, the impact is small, we can leave the requirement with TBD and update it for next meeting.

Issue 6: TP drafting for TS 38.141-2

Open issues:

1: The necessity to add Annex X for “General rules for statistic testing” (R4-1815841 and R4-1815842 from Nokia)

=>As per offline discussion, this Annex is not needed?

Offline discussion, Huawei and Ericsson do not need this annex, it should be covered by notes

The TPs can be noted

2: “Method of test” for 1-O and 2-O should be put in one common sub-clause or separate sub-clauses

At least one sub-clause on “Method of test” for each waveform of PUSCH, each format of PUCCH and PRACH”

=> Common sub-clause structure for both BS type 1-O and BS type 2-O?

Use the common sub-clause structure for TS 38.141-2 test cases drafting.

3: “Number RX antennas" used in conducted performance requirements need to change to “Number of demodulation branches” (i.e. not physical antennas of the antenna array) for radiated performance requirements description?

Option a) add the following general description on demodulation branch as in this TP, i.e., Whenever the "RX antennas" term is used for the radiated performance requirements description, it shall refer to the demodulation branches (i.e. not physical antennas of the antenna array). ()

Option b) to replace "RX antennas" with "demodulation branches" in the requirement tables for each physical channel.

=>TS 38.104 OTA part and TS 38.141-2, "RX antennas" should be replaced with "demodulation branches".

CATT: we should align

Nokia: use Number of demodulation braches for everywhere

ZTE: a general description to specify

Issue 7: Rules and Procedure to derive the performance requirement

General rule:

Basically AVE impairment results should be used if ideal SPAN<=2dB;

Procedure to derive the performance requirements for RAN4#89 meeting:

1: Basically use AVERAGE impairment results for performance requirements;

2: If the ideal span <=2dB, the AVERAGE impairment results can be used for the performance requirement with [] in the TPs;

Else if the ideal span is larger than 2dB, the results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN (re-)calculation until the ideal span is <=2dB but still with at least 3 companies’ results available. The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the TPs. If the span of the impairment results after removal the outliers are larger than [3dB], then the procedure cannot be applied.

3: Otherwise put TBD for the related performance requirements.

4: The agreed SNR values during this meeting can be updated if more or updated results are collected from companies in future meeting or technical issues are identified.

5: Set deadline for the results submission for this meeting,

Company can double check the above procedure and the 3dB span for impairment results during this meeting.

Nokia: email is pretty aligned, extra margin issues raised by Samsung

Samsung: UE demodulation performance considers to add extra margin, whether we follow UE side or not.

Issue 8: Others

Issue 8.1:

CATT: whether company use the time estimation error value for PRACH discussed in the email? CATT consider it.

Last meeting, we agreed 1.04us for FR1 PRACH format 0. Other short preamble formats for both FR1 and FR2, the values discussed during the email should be used by companies in their simulations.

Samsung: FR1 consider it, FR2 did not consider it.

Ericsson use the time estimation error in their simulation

Nokia: will check whether apply that value or not.

Issue 8.2:

DCM: deadline for the requirements, we need to test requirements based on the current specification, we understand that many test cases, we can focus on those typical cases. Up to 4Rx for FR1, up to 2Rx for FR2; double check their preference for other configurations.

- Companies can focus on the following simulation configuration with higher priority for the next meeting to converge our simulation results and finalize the requirement as early as possible:

- For PUSCH CP-OFDM and PUCCH,:

  - FR1: 10MHz/15kHz; 40MHz/30kHz

  - FR2: 100MHz/60kHz; 100MHz/120kHz

Operators can provide their preference to further down select the higher priority cases.

DCM preference: Up to 4Rx for FR1, up to 2Rx for FR2;

Issue 8: Tdoc list for new WFs and TPs to be revised

| Tdoc# | Title | Agenda Item | Source |
| --- | --- | --- | --- |
| R4-181xxxx | Adhoc minutes for NR BS demodulation performance | 7.13.2.1 | Huawei, HiSilicon |
| R4-181xxxx | Way Forward for NR BS demodulation general part | 7.13.2.1 | Huawei, HiSilicon |
| R4-181xxxx | Way Forward for NR BS PUSCH demodulation performance | 7.13.2.1 | Nokia, Nokia Shanghai Bell |
| R4-181xxxx | Way Forward for NR BS PUCCH demodulation performance | 7.13.2.1 | Huawei, HiSilicon |
| R4-181xxxx | Way Forward for NR BS PRACH demodulation performance | 7.13.2.1 | Ericsson |
| R4-181xxxx | TP for introduing propagation conditions in TS 38.141-2 | 7.13.2.5 | Huawei, HiSilicon |
| R4-1814439 | Draft CR to TS 38.104: Performance requirements for DFT-s-OFDM based PUSCH | 7.13.2.2.2 | China Telecom |
| R4-1814440 | TP to TS 38.141-1: Update of AWGN power level and FRC index for DFT-s-OFDM based PUSCH demodulation requirements | 7.13.2.2.2 | China Telecom |
| R4-1814441 | TP to TS 38.141-2: Radiated test requirements for DFT-s-OFDM based PUSCH | 7.13.2.2.2 | China Telecom |
| R4-1814477 | Draft CR on PUCCH format2 performance requirements for TS 38.104 | 7.13.2.3.2 | Samsung |
| R4-1814478 | TP for TS 38.141-1 on NR PUCCH format2 conducted performance requirements | 7.13.2.3.2 | Samsung |
| R4-1814479 | TP for TS 38.141-2 on NR PUCCH format2 radiated performance requirements | 7.13.2.3.2 | Samsung |
| R4-1814632 | Draft CR for TS38.104: Performance requirements for PRACH | 7.13.2.4.2 | CATT |
| R4-1814633 | TP to TS38.141-1: Performance requirements for PRACH | 7.13.2.4.2 | CATT |
| R4-1814634 | TP to TS38.141-2: Performance requirements for PRACH | 7.13.2.4.2 | CATT |
| R4-1814817 | Draft CR on FR2 PUCCH format 1 performance requirement for TS 38.104 | 7.13.2.3.2 | ZTE Wistron Telecom AB |
| R4-1814818 | TP for TS38.141-2: PUCCH format 1 OTA conformance test | 7.13.2.3.2 | ZTE Wistron Telecom AB |
| R4-1815348 | TP to TS 38.141-2: Radiated test requirements for CP-OFDM based PUSCH in FR1 | 7.13.2.2.2 | Nokia, Nokia Shanghai Bell |
| R4-1815362 | Draft CR to TS 38.104: PUCCH format 0 requirement | 7.13.2.3.2 | Ericsson |
| R4-1815363 | TP to TS 38.141-1: PUCCH format 0 requirement testing | 7.13.2.3.2 | Ericsson |
| R4-1815364 | TP to TS 38.141-2: PUCCH format 0 requirement testing | 7.13.2.3.2 | Ericsson |
| R4-1815378 | DraftCR to TS 38.104: cleanup for the performance requirements sections (8, 11) | 7.13.2.1 | Huawei |
| R4-1815379 | DraftCR to TS 38.104: clarification on the use of "RX antennas" for the OTA demodulation (11.1.1) | 7.13.2.1 | Huawei |
| R4-1815380 | DraftCR to TS 38.104: OTA demodulation branches (11.1.2) | 7.13.2.1 | Huawei |
| R4-1815571 | Draft CR to 38.104 on Noise definition for BS demodulation | 7.13.2.1 | Ericsson |
| R4-1815708 | Draft CR to TS 38.104 – PUSCH requirements with CP-OFDM for FR2 | 7.13.2.2.2 | Ericsson |
| R4-1815709 | TP to 38.141-2 – PUSCH requirements with CP-OFDM for FR2 | 7.13.2.2.2 | Ericsson |
| R4-1815839 | CR for 38.104 on PUSCH requirements with CP-OFDM and FR1 | 7.13.2.2.2 | Nokia, Nokia Shanghai Bell |
| R4-1815840 | TP for 38.141-1 on PUSCH requirements with CP-OFDM and FR1 | 7.13.2.2.2 | Nokia, Nokia Shanghai Bell |
| R4-1816014 | TP for updating 38.104 clause 8 about PUCCH formats 3 and 4 performance requirements | 7.13.2.3.2 | Huawei, HiSilicon |
| R4-1816015 | TP for updating 38.141-1 clause 8 about PUCCH formats 3 and 4 conformance testing | 7.13.2.3.2 | Huawei, HiSilicon |
| R4-1816016 | TP for introducing PUCCH format 3 and 4 radiated conformance requirements for OTA test in 38.141-2 | 7.13.2.3.2 | Huawei, HiSilicon |
| R4-1816017 | TP for introducing propagation conditions in TS 38.104 | 7.13.2.5 | Huawei, HiSilicon |
| R4-1816018 | TP for introducing propagation conditions in TS 38.141-1 | 7.13.2.5 | Huawei, HiSilicon |
| R4-1814474 | Discussion and simulation results for NR PUSCH | 7.13.2.2.1 | Samsung |
| R4-1814475 | Discussion and simulation results for NR PUCCH | 7.13.2.3.1 | Samsung |
| R4-1814476 | Discussion and simulation results for NR PRACH | 7.13.2.4.1 | Samsung |
| R4-1814629 | Simulation results for NR PUSCH | 7.13.2.2.1 | CATT |

General Issues

Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1814434 | Further discussion on BS test applicability for different SCS and CHBW | China Telecom | Proposal 1: For PUSCH and PUCCH demodulation tests, BS is required to pass tests for all the declared SCS. Proposal 2: For PUSCH and PUCCH demodulation tests, for each declared SCS, BS is only required to pass tests for the highest channel BW from BS declared BWs. - If the BW for testing is not in the subset with defined performance requirements, this BW will be tested based on the requirements of the nearest lower BW (i.e. reference BW) in the subset. In the test, the reference BW will be placed in the middle of the channel BW. |
| R4-1815259 | NR OTA performance test AWGN level settings description | Keysight Technologies UK Ltd |  |
| R4-1815365 | BS demodulation - further considerations on applicability rules | Ericsson | Proposal 1: Mandate PRACH format requirements only if BS is supporting this format. Proposal 2: Mandate PUCCH format requirements only if BS is supporting this format. Proposal 3: If no applicability rule is valid or could be agreed, there shall be a demodulation requirement for each supported feature and associated configuration that is planned to be used. Proposal 4: Discuss demodulation specific declarations when demodulation conformance part will be settled. |
| R4-1815382 | OTA demodulation testing scope for multiple demodulation branches | Huawei | Proposal 1: do not multiple the amount of OTA testing in order to provide test coverage for all the demodulation branches. Additionally, in order to reassure fair and transparent performance evaluation, the following can be considered: Proposal 2: selection of the pair of demodulation branches shall not be based on manufacturer declarations. Proposal 3: in case of OTA demodulation testing, all the demodulation branches not selected for the testing shall be switched OFF, to reassure fair performance evaluation. |
| R4-1815710 | Open issues for FR2 BS Demodulation Simulations with Impairments | Ericsson | For FR2 BS demodulation performance with impairment simulations must consider phase noise modelling as input towards requirement levels. Simulation considerations of required SNR given the link budget considerations in FR2 for BS demodulation performance |
| R4-1815511 | On link budget for OTA demodulation testing in FR1 | Ericsson | In this paper, we discussed the link budget for demodulation test for FR1. We can see that the test assuming 20 dB added AWGN above the noise floor is feasible. |
| R4-1815512 | On link budget for OTA demodulation testing in FR2 | Ericsson |  |
| R4-1815269 | TP to TS 38.141-2 on OTA demodulation branches | Nokia, Nokia Shanghai Bell | a text proposal to align the wording in the OTA demodulation branches subclause in TS 38.141-2 with those in the agreed CR in R4-1814092 |

Discussions

Issue 1: Test applicability for PUSCH and PUCCH tests with different SCS and CBW combinations

Agreements in the previous meeting RAN4#88:

Approach 1: Agreements in the July AH meeting [3]

- BS is only required to pass tests for one BW selected from BS declared BWs

  - In principle, if the selected BW for testing is not in the subset with defined performance requirements, this BW will be tested based on the requirements of the nearest lower BW (i.e. reference BW) in the subset.

  - In the test, the reference BW will be placed in the middle of the BW for PUSCH and PUCCH

Approach 2: Agreements in the Aug meeting [4]

- PUSCH and PUCCH:

  - For selection of the SCS and BW combination(s), the following principle will be used:

    - If the SCS and BW combination(s) declared by BS are included in 38.104 with the corresponding performance requirements defined:

      - If only one declared SCS and BW combination is included, then this combination is selected for test.

      - If more than one declared SCS and BW combinations are included:

        - For cases with smaller than [X] dB gap on simulation results for different declared SCSs, select the smallest SCS

        - For cases with larger [X] dB gap on simulation results for different declared SCSs, tests all the declared SCSs

        - BS is only required to pass tests for the highest channel BW declared by BS for the selected SCS(s).

    - Else if the SCS and BW combination(s) declared by BS are not included in 38.104 with the corresponding performance requirements defined:

      - Choose the declared SCS(s) and its nearest lower BW (i.e. reference BW) as new combination(s)

        - If only one SCS and BW combination is declared, then its new combination is selected for test.

        - If more than one SCS and BW combinations are declared, for those corresponding new combinations:

          - For cases with smaller than [X] dB gap on simulation results for different declared SCSs, select the smallest SCS

          - For cases with larger [X] dB gap on simulation results for different SCSs, tests all the declared SCSs

          - BS is only required to pass tests for the selected highest nearest lower channel BW.

        - the reference BW will be placed in the middle of the BW.

Further rewording or other options not precluded.

Open issues:

- Different SCS

  - Option 1: BS is required to test all the declared SCS.

  - Option 2:

- Different CBW

  - Option 1: For each declared SCS, BS is required to test the highest CBW declared.

    - If the largest CBW declared is no in the subset with defined performance requirements, BS will be tested on the nearest lower BW (i.e. reference BW) in the subset, the reference BW will be placed in the middle of the channel BW during the test.

  - Option 2：Keep the agreement made in August meeting

Discussion:

Option 1: smaller SCS is more sensitivity to the frequency shift and show worse performance in propagation condition with big Doppler frequency; larger SCS is more sensitivity to the delay spread, show worse performance in propagation condition with larger delay spread.

ZTE: prefer option 1 but with narrowest declared CBW, because the CR will be higher

Nokia: prefer option 1 with largest CBW.

CTC: Prefer option1.

Agreements:

Tentative agreements : Agree to test all declared SCS, but for each declared SCS, the largest or the narrowest BW to be testd for further discussion

To be confirmed by Ericsson

Issue 2: Test applicability for different PRACH formats with requirements defined

Agreements in the last meeting:

- Set requirements for Preamble:

  - Short sequence

    - Format A1, A2, A3, B4, C0 and C2

  - Long sequence:

    - Format 0

    - SCS: cover 15kHz, 30kHz, 60kHz (FR2) and 120kHz for short sequence

Open issues:

- Option 1: BS is only tested on one of the supported {Preamble format, PRACH SCS} combination

- Option 2: BS is tested on the supported preamble format

Discussion:

Nokia: based

ZTE:

Ericsson: how to test the new formats declared by company but without requirements defined,

BS is only tested on all the supported preamble formats where a requirement is defined, and just choose one of the supported PRACH SCS for each supported preamble format.

If new formats introduced in the future, it need further discussion.

Agreements:

Tentative agreements: For those agreed formats, i.e. short sequence format A1, A2, A3, B4, C0 and C2 and long sequence format 0, BS is tested on all the supported preamble formats, and just choose one of the supported PRACH SCS for FR1 and FR2 for each supported preamble format.

To be confirmed by Ericsson

Issue 3: Test applicability for different PUCCH formats

Agreements in the previous meeting:

- Format:

  - Define requirements for PUCCH format 0,1,2,3,4

Open issues:

- BS is only required to test the PUCCH format(s) declared to support;

Discussion:

Possible Agreements:

BS is only required to test the PUCCH format(s) declared to support;

Issue 4: Test applicability for different configurations

Agreements in the previous meeting:

- FR1 (RAN4#88 R4-1811694)

  - Slot-based transmission with resource mapping type A

  - FFS:

  - Slot-based transmission with resource mapping type B

  - non-slot based transmission with resource mapping type B

- DMRS pattern for PUCCH format 3 and 4 (RAN4#88 R4-1811727):

  - FR1:

    - Without additional DMRS for all cases

    - With additional DMRS for cases with the number of OFDM symbols more than 9

  - FR2:

    - Without additional DMRS

    - FFS with additional DMRS

Open issues:

- Different configurations for one feature:

  - PUSCH time domain resource allocation:

    - Slot based and non-slot based transmission

    - PUSCH resource mapping type A and type B

  - DMRS configurations with and without additional DMRS:

    - PUSCH FR1 and FR2

    - PUCCH format 3 & 4: FR1 and FR2

Discussion:

Nokia: prefer to conclude the current agreements and discuss the other possible new cases after Nov.

Ericsson: Company just test one of the configuration they support. Late drop for new cases. Prefer to agree to introduce new case this meeting and further discussion the details after Nov, ok to finalize those agreed cases.

Possible Agreements:

Issue 5: Phase noise modelling

Agreements in the last meeting:

- Phase Noise (RAN4#88Bis R4-1813755)

  - FFS whether PN is modelled in FR2 simulation

    - Option 1: Yes, and select one of the two options from the TR.

    - Option 2: No, and add a certain amount of margin on top of the  impairment results provided by companies. The exact margin to be added is up to company’s implementation.

  - Further discuss whether to have carrier frequency specific requirements for FR2

Open issues:

- Whether PN is modelled in FR2 test cases

  - Option 1: Yes

  - Option 2: No and add a certain amount of margin on top of the final averaged performance requirements for phase noise impact.

- Phase noise impact to different modulation order

  - QPSK: less impact, from the investigation from UE demodulation performance, PT-RS is not necessary

  - 16QAM and 64QAM

Discussion:

Option 1 : Yes (Nokia, Ericsson, ZTE)

Option 2 Ericsson : For QPSK less impact from PN, it is ok to capture the results with impairments if any. For 16QAM for ideal results if we any results available with clarification of no PN modelled ; but 64QAM further study with PN modelled, no results should be included for FR2

ZTE : consider the whole for the impairment

Nokia : put the results for QPSK, 16QAM and 64QAM to TBD for this meeting.

ZTE : QPSK with impairments has higher priority.

Possible Agreements:

FR2: QPSK with impairments has higher priority, company are encouraged to provide results with impairments within this meeting and try to conclude it.

Issue 6: How to test BS type 1-O or BS type 2-O with 4 and 8 demodulation branches

Agreements in the last meeting:

Open issues:

- How to reassure that all the demodulation branches perform as intended.

Discussion:

R4-1815382:

Proposal 1: do not multiple the amount of OTA testing in order to provide test coverage for all the demodulation branches.

Additionally, in order to reassure fair and transparent performance evaluation, the following can be considered:

Proposal 2: selection of the pair of demodulation branches shall not be based on manufacturer declarations.

Proposal 3: in case of OTA demodulation testing, all the demodulation branches not selected for the testing shall be switched OFF, to reassure fair performance evaluation.

Ericsson: prefer to move to the RF session

Move to the RF session.

Agreements:

Issue 7: AWGN level setting for FR2 OTA testing

Agreements in the last meeting:

Open issues:

- AWGN level setting for FR2 OTA test

Discussion:

Move to the RF session.

Agreements:

PUSCH

Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1814436 | Remaining issues for NR PUSCH demodulation requirements | China Telecom | Proposal 1: For FR1, include non-slot based transmission with resource mapping type B, with a limited number of test cases. Proposal 2: For FR1 non-slot based transmission with resource mapping type B, configure the symbol length and start symbol index as 10 and 0 respectively. Proposal 3: Cover all the 4 SCS for DFT-s-OFDM requirements, and whether both SCS in one FR are to be tested depends on the test applicability. Proposal 4: For DFT-s-OFDM requirements, change the channel BW from 100MHz to 50MHz for 120kHz SCS. |
| R4-1814474 | Discussion and simulation results for NR PUSCH | Samsung | Proposal 1: For FR1, there is no performance requirement of PUSCH with type B. |
| R4-1815711 | Discussion and FR1 simulation results for NR PUSCH demodulation requirements | Ericsson | Observation 1: From the simulation results, it clearly shows that with only one DMRS, it is unreliable and not suitable for most of the test cases with the chosen channel conditions. Observation 2: Application rule holds for DFT-S-OFDM with (1+1) DMRS pattern and the specified bandwidth for FR1. However, the application rule fails for DFT-S-OFDM with front-loaded only DMRS due to the reason mentioned in observation 1. Proposal 1: Based on observation 1, suggest removing test cases with front-loaded only DMRS pattern for the current specified channel conditions in the specification for FR1. Proposal 2: Should define test cases that could test different decoders, and to further study (for the next meeting) to formulate appropriate test cases and test metrics. |
| R4-1815712 | FR2 ideal simulation results for NR PUSCH demodulation requirements | Ericsson |  |
| R4-1816012 | Discussion on NR PUSCH demodulation performance | Huawei, HiSilicon | Proposal 1: Agree to define performance requirements for FR1 slot-based transmission for both PUSCH mapping B: - The same performance requirements for cases with one additional DMRS configured as PUSCH mapping A - Separate performance requirements for cases with only front-load DMRS configured from PUSCH type A Proposal 2: Agree to define performance requirements with 1+1 DMRS configuration for FR2 non-slot based transmission with 10 and PUSCH mapping type B. Proposal 3: Define performance requirements with and without PT-RS configured for NR PUSCH FR2 and gNB can pass one set of requirements as per declaration. Proposal 4: Not configure PT-RS for NR PUSCH FR2 performance requirements with QPSK. Proposal 5: Applicability rules should be defined for different PUSCH resource mapping types, i.e. Type A and Type B, slot-based and non-slot based transmission, different number of additional DMRS configurations, i.e. with and without one additional DMRS and different PT-RS configurations, i.e. with and without PT-RS configured. |

Discussions

Issue 1: FR1: PUSCH resource mapping type B

Agreements in the previous meeting:

- For FR1,

  - Whether to test non-slot and/or slot based transmission with resource mapping type B

    - Option1: Type B for non-slot based.

    - Option2: Type B for slot based

    - Option3: Type B for both slot and non-slot based.

    - Option4: Type B not tested

Open issues:

- For FR1,

    - Option1: Type B for non-slot based.

    - Option2: Type B for slot based

    - Option3: Type B for both slot and non-slot based.

    - Option4: Type B not tested

As per simulation results (R4-1816012), for both slot-based transmission of PUSCH mapping type A and type B

- The same performance requirements for cases with one additional DMRS configured for both PUSCH mapping type A and type B

- Different performance requirements for cases with only front-load DMRS configured for PUSCH mapping type A and type B

Discussion:

- Option1: Type B for non-slot based.(CTC with limited number of test cases)

- Option2: Type B for slot based

- Option3: Type B for both slot and non-slot based.

- Option4: Type B not tested (Samsung)

This is related to the discussion of “test applicability for different configurations for one specific feature”

Agreement:

Issue 2: DMRS configuration

Agreements in RAN4 #AH-1807

- DMRS number

  - FR1: 1 (one front-loaded) and 1+1 (one front-loaded and one additional)

  - FR2: 1 (one front-loaded)

Open issues:

1: As per the simulation results and observation from in R4-1815711 and R4-1815349, the front-loaded only DMRS configuration is not applicable for some test cases for CP-OFDM with 2Tx, especially 2T2R and some 2T4R cases.

- How to handle those cases or the front-loaded only DMRS configuration?

- Option 1: Remove test cases with front-loaded only DMRS pattern for the current specified channel conditions in the specification (Ericsson)

- Other options.

DMRS for CP-OFDM

  - For FR1

    - Option 1: 1+0 and 1+1 (Original agreements)

    - Option 2: 1+1 (Ericsson, as per the simulation, some cases are not applicable for DMRS 1+0)

2: Whether to add DMRS 1+1 configuration for FR2 for both CP-OFDM and DFT-s-OFDM considering it is mandatory without UE capability signaling?

  - For FR2

    - Option 1: 1+0

    - Option 2: 1+0 and 1+1

Discussion:

Nokia: disagree to completely remove the front-loaded only pattern, just remove those specific test cases that cannot find the testable SNR as per the aligned idea simulation results for this meeting

ZTE: Keep the front-loaded only pattern and just remove some unfeasible test cases

Ericsson: Fine to just remove those unfeasible test cases.

Agreements:

Just remove some specific test cases with front-loaded DMRS configured that are not feasible, figure out the speficic test cases as per the aligned ideal results from companies during this meeting.

Nokia leads the discussion to figure out those specific test cases and include in the PUSCH WF.

Issue 3: MCS20 only for 1Tx

Agreements in the previous meeting (RAN4#1807 R4-1809336):

- Transmission scheme.

  - Define the requirements of PUSCH with 1Tx and 2Tx

  - Define the requirements for 2 layer transmission for 2Tx

- MCS to be used for test ()

  - CP-OFDM

    - QPSK: MCS2 (LDPC base graph #2)

    - 16QAM: MCS16 (LDPC base graph #1)

    - 64QAM: MCS20 (LDPC base graph #1) for 1Tx

Open issues:

During the email discussion before the meeting, nobody know what is reason behind to set MCS20 only for 1Tx

Discussion:

Nokia: 2T2R usually has too higher SNR, ok to only define for 1Tx

ERICSSON: OK to the current agreement. Try to avoid the higher SNR for 64QAM, 2 layers the corresponding SNR is about 23-26B depending on different conditions

ZTE: In general, if no strong, just keep the previous agreement.

Possible Agreements:

Keep the previous agreements of defining requirements for 64QAM with 1Tx only.

Issue 4: PTRS configuration

Agreements in last meeting:

- PTRS are configured in FR2 test cases, the configuration is:

  - For CP-OFDM

    - Frequency density KPTRS : 2 (PTRS every 2 RBs)

    - Time density LPTRS : 1 (all symbols with PTRS)

  - Default thresholds for PT-RS patterns for DFT-S-OFDM

    - NRB0=0, NRB1=8, NRB2=NRB3=32, and NRB4=108

Open issues:

  - QPSK: PT-RS has less impact, not configure PT-RS

  - 16QAM and 64QAM:

    - Both PT-RS configured and not-configured

Discussion:

Nokia: prefer to configure PT-RS for QPSK even less impact to the performance considering the same configuration for all cases’

Keep the previous agreements.

Samsung: No PT-RS figuration is the baseline and further study if PT-RS is needed for all modulation order.

Ericsson: No technical concern raised, we should keep the previous agreement

ZTE: Any harm to keep the PT-RS configuration for QPSK, keep the previous agreement

CATT: FRC, the PT-RS overhead is 0 during the email discussion.

Agreements:

Default Xoverhead value 0 is agreed to use in the TBS calculation for FR2 cases with PT-RS configured.

Issue 5: DFT-s-OFDM

Agreements in last meeting:

- Simulate the following by the next meeting

  - Based on results, it will be decided if one SCS/BW combination is sufficient per FR in the next meeting

| SCS | BW in PRB | BW in MHz |
| --- | --- | --- |
| 15 (FR1) | 25 | 5 |
| 30 (FR1) | 24 | 10 |
| 60 (FR2) | 30 in the middle of the channel BW | 50 |
| 120 (FR2) | 30 in the middle of the channel BW | 100 |

Open issues:

  - One SCS/CBW combination per FR

    - Option 1: Cover 4 SCS, whether both SCS in one FR to be tested depends on the test applicability

    - Other options

  - CBW for FR2 120kHz SCS

    - Change CBW from 100MHz to 50MHz for FR2 120kHz SCS

Discussion:

1: Whether cover 4 SCS? Whether 2 SCS for each FR need to tested or just select one SCS if the simulation results is quite similar.

CATT: As per the simulation results submitted, the results even for FR1 is quite different; for FR2, need to double check the results;

CTC: The number of test cases is small, prefer to test all SCS, the concern is the 50MHz CBW is the minimal for 120kHz SCS.

ZTE: FR1 results are quite different for 1+0, 2dB difference; but 1dB difference for 1+1; prefer to test all configurations considering the different SCS under fading channel. Need to rerun the simulation if we change the BW from 100MHz to 50MHz

Nokia: FR1 results are quite different for 1+0; but very similar for 1+1

2: Test applicability for each SCS per FR

Agreements:

Agree to test all 4 agreed CBW/SCS combinations,

PUCCH

Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1814475 | Discussion and simulation results for NR PUCCH | Samsung | Proposal 1: Both without additional and with additional DMRS should be considered for defining the performance requirement for PUCCH format 3 and format 4, considering the UCI payload, and high speed scenario, as well as the coding rate , in both FR1 and FR2. Observation 1: With additional DMRS can achieve better performance than that of without additional DMRS. Observation 2:NACK2ACK detection probability for Format 2 is not the bottleneck compared with ACK missed detection probability. Proposal 3: ACK missed detection probability and DTX to ACK can be regarded as the test metric for Format 2. |
| R4-1815359 | Remaining issues for PUCCH demodulation: metric for PUCCH format 2 | Ericsson | Proposal: PUCCH Format 2 shall not consider performance requirement based upon NACK2ACK test metric. |
| R4-1815360 | Simulation results on PUCCH demodulation | Ericsson |  |

Discussions

Issue 1: Test metric for PUCCH format 2

Agreements in the last meeting:

- Test metric

  - If number of bits <= 11

    - DTX to Ack probability <1% and Missed Ack probability < 1%

    - Define NACK to ACK requirement if it is the limiting factor compared with ACK miss requirement.

      - More simulation results are encouraged in the next meeting.

Open issues:

- NACK to ACK test metric for format 2

Simulation results provided by companies:

- Results shown in R4-1814820(ZTE), R4-1815350(Nokia) and R4-1815359(Ericsson) that NACK to ACK is not the limiting factor compared with 1% ACK missed

- Samsung R4-1814475 to be provided

Discussion:

Samsung: in last meeting, we provide our results to show NACK to ACK is not the limiting factor.

Agreements:

Not define NACK to ACK performance requirements for PUCCH format 2

Issue 2: Additional DMRS for PUCCH format 3 and 4 FR2

Agreements in the previous meeting:

- DMRS pattern for format 3 and 4:

  - FR1:

    - Without additional DMRS for all cases

    - With additional DMRS for cases with the number of OFDM symbols more than 9

  - FR2:

    - Without additional DMRS

    - FFS with additional DMRS

Open issues:

- Additional DMRS for format 3 and 4 in FR2 for cases with the number of OFDM symbols more than 9

  - Option 1: with and without DMRS (Samsung)

  - Option 2: without DMRS only

Discussion:

This is related to the discussion of “test applicability for different configurations for one specific feature”

Agreements:

Issue 3: Time mask for FR2 with frequency hopping for cases with 120kHz SCS

Agreements in the previous meeting:

Intra-slot frequency hopping is enabled

Open issues:

What is the impact to the FR2 PUCCH demodulation performance?

Discussion:

R4-1815049 feature item 4-1 Basic UL control channel:

“RAN4 needs to check feasibility of frequency hopping for PUCCH formats for FR2. “Feasibility” here means whether PUCCH demodulation performance can be kept or not when 120kHz SCS is assumed and RB hopping is applied in our understanding.”

“6.3.3	Transmit ON/OFF time mask

6.3.3.1	General

The transmit ON/OFF time mask defines the transient period(s) allowed

-	between transmit OFF power and transmit ON power symbols (transmit ON/OFF)

-	between continuous ON-power transmissions when power change or RB hopping is applied.

In case of RB hopping, transition period is shared symmetrically.

- A long subslot transmission is a Type B transmission with more than 2 symbols.

- A short subslot transmission is a Type B transmission with 1 or 2 symbols.

Agreements:

Further investigation is need to check the performance impact due to the time mask under the frequency hopping enabled.

Whether frequency hopping is needed considering the time mask impact to the PUCCH performances.

PRACH

Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1814445 | Remaining issues for NR PRACH demodulation requirements | China Telecom | Proposal 1: For preambles with short sequence, set the timing offset base value as 0, and this offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.8us. Then the loop is being reset and the timing offset is set again to 0us. Proposal 2: Use the time estimation error tolerance in the following table. PRACH Preamble; PRACH SCS; Proposed time estimation error tolerance for AWGN; Proposed time estimation error tolerance for TDL channel. Long sequence; 1.25kHz; 1.04 us; 1.04 us + 1.51 us = 2.55 us. Short sequence; 15kHz; 0.52 us; 0.52 us + 1.51 us = 2.03 us. 30kHz; 0.26 us; 0.26 us + 1.51 us = 1.77 us. 60kHz; 0.13 us; 0.13 us + 0.15 us = 0.28 us. 120kHz; 0.07 us; 0.07us + 0.15 us = 0.22us. |
| R4-1815366 | Further agreements on NR PRACH demodulation | Ericsson |  |
| R4-1816009 | Discussion on NR PRACH demodulation performance | Huawei, HiSilicon |  |

Discussions

Issue 1: Timing offset and time estimation error tolerance

Agreements in the last meeting:

- Reuse LTE timing offset sequence for format 0

  - Timing offset start value:

    - 50% of Ncs

  - Increased within the loop, adding 0.1us in each step (up to 0.9us)

  - Then loop is reset and restart again.

- Timing offset shall be scaled according to SCS and format

  - To be further investigated, Email discussion. Deadline: Oct 26th

- Tolerance on timing offset error should be further investigated for next meeting.

  - Option1: Scaling according to SCS

  - Other option is not precluded

  - Email discussion. Deadline: Oct 26th

Open issues:

Discussion:

Email discussion was initiated by Ericsson after RAN4#88bis meeting and summarized by China Telecom as below:

Candidate Agreements:

Timing offset: For preambles with short sequence, set the timing offset base value as 0, and this offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.8us. Then the loop is being reset and the timing offset is set again to 0us.

Time estimation error tolerance:

| PRACH Preamble | PRACH SCS | Proposed time estimation error tolerance for AWGN | Proposed time estimation error tolerance for TDL channel |
| --- | --- | --- | --- |
| Long sequence | 1.25kHz | 1.04 us | 1.04 us + 1.51 us = 2.55 us |
| Short sequence | 15kHz | 0.52 us | 0.52 us + 1.51 us = 2.03 us |
|  | 30kHz | 0.26 us | 0.26 us + 1.51 us = 1.77 us |
|  | 60kHz | 0.13 us | 0.13 us + 0.15 us = 0.28 us |
|  | 120kHz | 0.07 us | 0.07us + 0.15 us = 0.22us |

Draft CRs and TPs

Contributions list and summary of proposals

1) Draft CRs to TS 38.104

|  | Tdoc | Title | Source | Notes |
| --- | --- | --- | --- | --- |
| General and Annex | R4-1815378 | DraftCR to TS 38.104: cleanup for the performance requirements sections (8, 11) | Huawei |  |
|  | R4-1815379 | DraftCR to TS 38.104: clarification on the use of "RX antennas" for the OTA demodulation (11.1.1) | Huawei |  |
|  | R4-1815380 | DraftCR to TS 38.104: OTA demodulation branches (11.1.2) | Huawei |  |
|  | R4-1815570 | Draft CR to 38.104 on Combined CRs for BS Demodulation performance | Ericsson |  |
|  | R4-1815571 | Draft CR to 38.104 on Noise definition for BS demodulation | Ericsson |  |
|  | R4-1816017 | TP for introducing propagation conditions in TS 38.104 | Huawei, HiSilicon |  |
| PUSCH | R4-1814439 | Draft CR to TS 38.104: Performance requirements for DFT-s-OFDM based PUSCH | China Telecom |  |
|  | R4-1814442 | Draft CR to TS 38.104: FRC definitions for FR1 DFT-s-OFDM based PUSCH and FR2 PUSCH | China Telecom |  |
|  | R4-1815708 | Draft CR to TS 38.104 – PUSCH requirements with CP-OFDM for FR2 | Ericsson |  |
|  | R4-1815839 | CR for 38.104 on PUSCH requirements with CP-OFDM and FR1 | Nokia, Nokia Shanghai Bell |  |
| PUCCH | R4-1814477 | Draft CR on PUCCH format2 performance requirements for TS 38.104 | Samsung |  |
|  | R4-1814817 | Draft CR on FR2 PUCCH format 1 performance requirement for TS 38.104 | ZTE Wistron Telecom AB |  |
|  | R4-1815362 | Draft CR to TS 38.104: PUCCH format 0 requirement | Ericsson |  |
|  | R4-1816014 | TP for updating 38.104 clause 8 about PUCCH formats 3 and 4 performance requirements | Huawei, HiSilicon |  |
| PRACH | R4-1814632 | Draft CR for TS38.104: Performance requirements for PRACH | CATT |  |

2) TPs for TS 38.141-1

|  | Tdoc | Title | Source | Notes |
| --- | --- | --- | --- | --- |
| Annex | R4-1814435 | TP to TS 38.141-1: Measurement system set-up for BS type 1-H performance requirements | China Telecom |  |
|  | R4-1815841 | TP for 38.141-1 Annex for General rules for statistical testing | Nokia, Nokia Shanghai Bell | Related to the email discussion about the Annex of 38.141-1 |
|  | R4-1814443 | TP to TS 38.141-1: FRC definitions for FR1 DFT-s-OFDM based PUSCH | China Telecom |  |
|  | R4-1816018 | TP for introducing propagation conditions in TS 38.141-1 | Huawei, HiSilicon |  |
| PUSCH | R4-1814440 | TP to TS 38.141-1: Update of AWGN power level and FRC index for DFT-s-OFDM based PUSCH demodulation requirements | China Telecom |  |
|  | R4-1815840 | TP for 38.141-1 on PUSCH requirements with CP-OFDM and FR1 | Nokia, Nokia Shanghai Bell | FRC reference |
| PUCCH | R4-1814478 | TP for TS 38.141-1 on NR PUCCH format2 conducted performance requirements | Samsung | NACK to ACK no conclusion yet |
|  | R4-1815363 | TP to TS 38.141-1: PUCCH format 0 requirement testing | Ericsson |  |
|  | R4-1816015 | TP for updating 38.141-1 clause 8 about PUCCH formats 3 and 4 conformance testing | Huawei, HiSilicon |  |
| PRACH | R4-1814633 | TP to TS38.141-1: Performance requirements for PRACH | CATT |  |

3) TPs for TS 38.141-2

|  | Tdoc | Title | Source | Notes |
| --- | --- | --- | --- | --- |
| General and Annex | R4-1815381 | TP to 38.141-2: OTA demodulation alignment with TS38.104 (8.1) | Huawei |  |
|  | R4-1815842 | TP for 38.141-2 Annex for General rules for statistical testing | Nokia, Nokia Shanghai Bell | Related to the email discussion about the Annex of 38.141-1 |
|  | R4-1814444 | TP to TS 38.141-2: FRC definitions for PUSCH and test parameters for PRACH | China Telecom |  |
| PUSCH | R4-1814441 | TP to TS 38.141-2: Radiated test requirements for DFT-s-OFDM based PUSCH | China Telecom |  |
|  | R4-1815348 | TP to TS 38.141-2: Radiated test requirements for CP-OFDM based PUSCH in FR1 | Nokia, Nokia Shanghai Bell | Shall base on the merged TS 38.141-2 shared on RAN4 email reflector after RAN4#88Bis |
|  | R4-1815709 | TP to 38.141-2 – PUSCH requirements with CP-OFDM for FR2 | Ericsson |  |
| PUCCH | R4-1814479 | TP for TS 38.141-2 on NR PUCCH format2 radiated performance requirements | Samsung | No conclusion yet for NACK to ACK requirement, but missed DTX to ACK probability 1% |
|  | R4-1814818 | TP for TS38.141-2: PUCCH format 1 OTA conformance test | ZTE Wistron Telecom AB | Only type 2-O |
|  | R4-1815364 | TP to TS 38.141-2: PUCCH format 0 requirement testing | Ericsson | Common section for both type 1-O and 2-O |
|  | R4-1816016 | TP for introducing PUCCH format 3 and 4 radiated conformance requirements for OTA test in 38.141-2 | Huawei, HiSilicon | Separate section for BS type 1-O and 2-O |
| PRACH | R4-1814634 | TP to TS38.141-2: Performance requirements for PRACH | CATT | Common section for both type 1-O and 2-O |

Discussions

Issue 1: TP drafting in TS 38.141-2

Open issues:

1: The necessity to add Annex X for “General rules for statistic testing” (R4-1815841 and R4-1815842 from Nokia)

2: “Method of test” for 1-O and 2-O should be put in one common sub-clause or separate sub-clauses

- At least one sub-clause on “Method of test” for each waveform of PUSCH, each format of PUCCH and PRACH”

3: “Number RX antennas" used in conducted performance requirements need to change to “Number of demodulation branches” (i.e. not physical antennas of the antenna array) for radiated performance requirements description?

Option a) add the following general description on demodulation branch as in this TP, i.e., Whenever the "RX antennas" term is used for the radiated performance requirements description, it shall refer to the demodulation branches (i.e. not physical antennas of the antenna array).

Option b) to replace "RX antennas" with "demodulation branches" in the requirement tables for each physical channel.

And this needs to be aligned with between 38.104 and 38.141-2. (CTC)

Discussion:

1: During the email discussion initiated by Huawei about TS 38.141-1/2 drafting, some principles for consideration:

- As conformance testing specification, 38.141-1/2 needs to align with core specification 38.104

- 38.141-1/2 is for NR single RAT. As such, it can use 36.141 as a good reference

- Since 38.141-2 deals with radiated conformance testing, it may refer to eAAS conformance testing

- 38.141-1/2 should be aligned with each other as much as possible

- As is always commented, it is better for 38.141-1/2 to be self-contained to a large extent for the sake of convenience for testing people

2: Email discussion, Ericsson give the following proposals:

1-     The “Method of test”, including Initial conditions and Procedure will be common to all requirements test, only parameters and expected results would change.

To avoid repeating this for every test, we would propose to put this sub-clause in a common sub-section, and then, in each test requirement, we only mention the parameters + expected values.

2-     AWGN level: levels should be common for 1-O tests, and also for 2-O, right?

Again, to avoid useless repetition, we would propose to put those tables in some common sub-section, one for 1-O and one for 2-O.

As per the submitted TP, except Huawei and ZTE, all other companies are using the common section structure

3: Except R4-1814441(CTC) and R4-1814634(CATT), all other TPs are still using “Number of Rx antennas”

Agreements:

Summary of simulation results

PUSCH simulation results

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1814437 | Ideal simulation results for NR PUSCH | China Telecom |  |
| R4-1814438 | Impairment simulation results for NR PUSCH | China Telecom |  |
| R4-1814474 | Discussion and simulation results for NR PUSCH | Samsung |  |
| R4-1814629 | Simulation results for NR PUSCH | CATT |  |
| R4-1814746 | Updated simulation and impairment results for NR PUSCH | CMCC |  |
| R4-1814819 | Simulation results on NR PUSCH | ZTE Wistron Telecom AB |  |
| R4-1815349 | NR PUSCH simulation results | Nokia, Nokia Shanghai Bell |  |
| R4-1815711 | Discussion and FR1 simulation results for NR PUSCH demodulation requirements | Ericsson |  |
| R4-1815712 | FR2 ideal simulation results for NR PUSCH demodulation requirements | Ericsson |  |
| R4-1816013 | Simulation results for NR PUSCH | Huawei, HiSilicon |  |

PUCCH simulation results

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1814475 | Discussion and simulation results for NR PUCCH | Samsung |  |
| R4-1814630 | Simulation results for NR PUCCH | CATT |  |
| R4-1814747 | Updated simulation and impairment results for NR PUCCH | CMCC |  |
| R4-1814820 | Simulation results on NR PUCCH | ZTE Wistron Telecom AB |  |
| R4-1815350 | NR PUCCH simulation results | Nokia, Nokia Shanghai Bell |  |
| R4-1815360 | Simulation results on PUCCH demodulation | Ericsson |  |
| R4-1816011 | Simulation results for NR PUCCH | Huawei, HiSilicon |  |

PRACH simulation results

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1814476 | Discussion and simulation results for NR PRACH | Samsung |  |
| R4-1814631 | Simulation results for NR PRACH | CATT |  |
| R4-1814748 | Updated simulation and impairment results for NR PRACH | CMCC |  |
| R4-1814821 | Simulation results on NR PRACH | ZTE Wistron Telecom AB |  |
| R4-1815351 | NR PRACH simulation results | Nokia, Nokia Shanghai Bell |  |
| R4-1815361 | Simulation results on PRACH demodulation | Ericsson |  |
| R4-1816010 | Simulation results for NR PRACH | Huawei, HiSilicon |  |

Summary of simulation results

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1814432 | Summary of ideal results for NR BS demodulation requirements | China Telecom |  |
| R4-1814433 | Summary of impairment results for NR BS demodulation requirements | China Telecom |  |

Discussion: Companies are welcome to fill your simulation results in the summary of simulation results collected by China Telecomm on RAN4 email reflector, further discussion maybe is needed whether some requirements can be decided during RAN4#89 meeting as per the submitted simulation results.

Issue 1: Rules and Procedure to derive the performance requirement

General rule:

1: Basically AVE impairment results should be used if ideal SPAN<=2dB;

2: AVE impairment results by removing the highest and lowest values if ideal SPAN >2dB, but with editor notes that further revisit is needed;

Procedure to derive the performance requirements for RAN4#89 meeting: