3GPP TSG-RAN WG4 Meeting #89 	R4-186370

Spokane, USA, 12 - 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104 – PUSCH requirements with CP-OFDM for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update for PUSCH FR2 with CP-OFDM tests based on agreed draft CR R4-1813739. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update for PUSCH FR2 with CP-OFDM tests based on agreed draft CR R4-1813739 with new FRC indexes from Annex and the new terminology of demodulation branch instead of Rx antennas and remove MCS16 for 2T2R for both 60kHz 100MHz and 120kHz 100MHz according to the agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New requirement for PUSCH can’t be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of the change>

#### 11.2.2.1	Requirements for PUSCH with transform precoding disabled

##### 11.2.2.1.1 	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ retransmissions.

Table 11.2.2.1.1-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DMRS | DMRS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |
|  | Number of additional DMRS symbols | 0 |
|  | Number of DMRS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DMRS | -3 dB |
|  | DMRS port | {0}, {0, 1} |
|  | DMRS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PTRS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |

##### 11.2.2.1.2 	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in the tables 11.2.2.1.2-1 to 11.2.2.1.2-5 at the given SNR for 1Tx and for 2Tx two-layer spatial multiplexing transmission.

Table 11.2.2.1.2-1 Minimum requirements for PUSCH, 50 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal |  | TDLA30-300 |  | 70 % |  | G-FR2-A3-1 |  | 1+0 |  | [TBD] |  |
|  |  | Normal |  | TDLA30-300 |  | 70 % |  | G-FR2-A4-1 |  | 1+0 |  | [TBD] |  |
|  |  | Normal |  | TDLA30-75 |  | 70 % |  | G-FR2-A5-1 |  | 1+0 |  | [TBD] |  |
| 2 |  | Normal | TDLA30-300 | 70 % | G-FR2-A3-6 | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | G-FR2-A4-6 | 1+0 | [TBD] |

Table 11.2.2.1.2-2 Minimum requirements for PUSCH, 100 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal |  | TDLA30-300 |  | 70 % |  | G-FR2-A3-2 |  | 1+0 |  | [TBD] |  |
|  |  | Normal |  | TDLA30-300 |  | 70 % |  | G-FR2-A4-2 |  | 1+0 |  | [TBD] |  |
|  |  | Normal |  | TDLA30-75 |  | 70 % |  | G-FR2-A5-2 |  | 1+0 |  | [TBD] |  |
| 2 |  | Normal | TDLA30-300 | 70 % | G-FR2-A3-7 | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | G-FR2-A4-7 | 1+0 | [TBD] |

Table 11.2.2.1.2-3 Minimum requirements for PUSCH, 50 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal |  | TDLA30-300 |  | 70 % |  | G-FR2-A3-3 |  | 1+0 |  | [TBD] |  |
|  |  | Normal |  | TDLA30-300 |  | 70 % |  | G-FR2-A4-3 |  | 1+0 |  | [TBD] |  |
|  |  | Normal |  | TDLA30-75 |  | 70 % |  | G-FR2-A5-3 |  | 1+0 |  | [TBD] |  |
| 2 |  | Normal | TDLA30-300 | 70 % | G-FR2-A3-8 | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | G-FR2-A4-8 | 1+0 | [TBD] |

Table 11.2.2.1.2-4 Minimum requirements for PUSCH, 100 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal |  | TDLA30-300 |  | 70 % |  | G-FR2-A3-4 |  | 1+0 |  | [TBD] |  |
|  |  | Normal |  | TDLA30-300 |  | 70 % |  | G-FR2-A4-4 |  | 1+0 |  | [TBD] |  |
|  |  | Normal |  | TDLA30-75 |  | 70 % |  | G-FR2-A5-4 |  | 1+0 |  | [TBD] |  |
| 2 |  | Normal | TDLA30-300 | 70 % | G-FR2-A3-9 | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | G-FR2-A4-9 | 1+0 | [TBD] |

Table 11.2.2.1.2-5 Minimum requirements for PUSCH, 200 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 70 % | G-FR2-A3-5 | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | G-FR2-A4-5 | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 | 70 % | G-FR2-A5-5 | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 | 70 % | G-FR2-A3-10 | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | G-FR2-A4-10 | 1+0 | [TBD] |

<end of the change>